# Table of Contents

Introduction

Section 1 - Overview of MC-GPS

1.1  Equipment

1.2  The Keyboard

1.2.1  [ON]

1.2.2  [ENTER]

1.2.3  Shift Keys [SH1] and [SH2]

1.2.4  Function Keys [F1] - [F5]

1.2.5  Direction Keys [Ù], [Ú], [×], [Ø]

1.2.6  [ESC]

1.2.7  RESET Button

1.3  General Operations

1.3.1  Selecting Menu Options

1.3.2  Using Softkey Commands

1.3.3  Getting On-Screen HELP

1.3.4  Aborting an Operation

1.4  Entering/Editing Data

1.4.1  Automatic Data Entry

1.4.2  Error Checking

1.5  Status Indicators and Screens

1.5.1  Tracking Status Indicator

1.5.2  STATUS Screen

1.5.3  Satellite Status

1.5.4  MC-GPS System Status

Section 2 - Tutorial

2.1  Lesson 1 - Getting Started

2.1.1  Turn on MC-GPS (MAIN MENU)

2.1.2  Turn the GPS Receiver ON/OFF

2.1.3  Check the Tracking Status Indicator

2.1.4  Set up the GPS Receiver

2.2  Lesson 2 - Field Data Collection

2.2.1  Start a New Job

2.2.2  Select and Describe a Feature

2.2.3  Store a Line Feature

2.2.4  Store a Nested Point

2.2.5  Continue with the Line Feature

2.2.6  Turn OFF GPS Receiver

2.3  Lesson 3 - Transfer a Job to PC-GPS

2.4  Lesson 4 - Using PC-GPS

2.4.1  View Field Data

2.4.2  Perform Differential Correction

2.4.3  View and Edit a Job

2.4.4  Cut Uncorrected Points

2.4.5  Combine Several Job Files into One

2.4.6  Measure Distance

2.4.7  View the Details of a Feature

2.4.8  Creating and Editing a Feature List

2.4.9  Transfer a Feature List from PC-GPS to MC-GPS

Section 3 - Setup

3.1  GPS Receiver Setup

3.2  Data Logging Setup

3.3  Setup Datum/Coord. System

3.4  Time/Date Setup

3.5  Input/Output Setup

3.5.1  Bar Code Reader Setup

3.5.2  NMEA 0183 Setup

3.5.3  RTCM_104 Setup

Section 4 - Database Management (LISTS)

4.1  Job File Maintenance

4.1.1  Defining a Job and Editing Job Definition

4.1.2  Viewing Job Summary

4.2  Feature Lists, Attributes and Values

4.2.1  Maintaining Feature Lists

4.2.2  Defining Attributes

4.2.3  Defining Values

4.3  Waypoints and Routes

4.3.1  Editing a Route

4.3.2  Adding or Editing a Waypoint

4.3.3  Distance between Waypoints

4.3.4  Plotting a Route

Section 5 - GPS Data Collection

5.1  Identifying a Job

5.2  Collecting Feature Data (Featuring)

5.2.1  Selecting a Feature

5.2.2  Assigning Attributes and Values

5.2.3  Data Logging

5.2.4  Data Logging in OFFSET Mode

5.2.5  Data Logging in Traverse Mode (Only available in CMT Field Ver. 3.4)

5.2.5.1 Traversing using the Point-to-Point Method

5.2.5.2 Traversing using the From One Point Method

5.2.6  Editing Attributes and Values

5.2.7  Displaying Position Data

5.2.8  Carrier Phase GPS Data Logging

5.3  Appending Coordinate Points

5.4  Viewing and Editing Collected Feature Data

5.4.1  Viewing and Editing Feature/Attribute/Value

5.4.2  Viewing Location Data for a Feature

5.4.3  Viewing and Editing Offset Parameters

5.4.4  Viewing a Plot of the Collected Data

5.5  Logging Base Station Data

5.5.1  BASE Mode

5.5.2  BASEPC Mode

5.6  Viewing a Summary of the Collected Data

Section 6 - Navigation

6.1  Navigating along a Route

6.2  Moving from one Waypoint to another

6.3  Navigation Plot

Section 7 - Data File Transfer

7.1  Sending Data from MC-GPS to PC-GPS

7.2  Receiving Files from the PC

7.3  Sending Data from MC-GPS to another MC-GPS

Appendix A - User License Agreement

Appendix B - Specifications and Maintenance

Appendix C - Peripherals

Appendix D - Antennas

Appendix E - System Operations for Advanced Users

# Introduction

MC-GPS is an all-purpose Global Positioning System (GPS) Data Recorder and navigator.  MC-GPS consists of an MC5 Field Data Recorder, a GPS antenna and a GPS receiver integrated in one convenient hand-held package.  Your MC-GPS is configured to automatically run the GPS field data collection and navigation programs.  This is just one of the many applications that may be performed by using the MC5 computer.  This manual provides a brief discussion of the general characteristics of the MC-GPS Field Data Recorder and a detailed description of the GPS field data collection and navigation programs.

Using your MC-GPS Field Data Recorder, you can easily collect accurate position data.  With differential correction, the MC-GPS will provide 1-5 meter accuracy.  CMT's PC-GPS Windows-based software is required for differential correction and other pre-processing and post-processing tasks.  You may purchase this optional software package from CMT or its dealers.

MC-GPS can collect data for three different types of locations:

Point	One location.  Examples - corner post, tree, plot center, light post.

Line	A series of points connected together.  Examples - road, pipe line, waterway.

Area	A line that ends the same place it began.  Examples - lake, sale unit, park boundaries, wetlands.

Descriptions may be recorded and attached to the GPS coordinates.  To accomplish this, MC-GPS uses a process called Featuring.  Featuring is the process of tagging specific Features, Attributes and Values to one or more GPS coordinate positions.  A hierarchical database of information may be associated with each GPS position.

Feature		An identification.  The object being mapped.  Each Feature is designated as a Point, Line, or Area.

Attribute	A question about the Feature.

Value		An answer to the question asked by the Attribute.

An example of a position record using the above system might be:

FEATURE NAME	TYPE		ATTRIBUTE		VALUE
POWER POLE		POINT		CONDITION?		POOR
					AGE?			20-25 years
					TRANSFORMER?	NO

All of the data about this power pole is tied to one GPS coordinate location inside the MC-GPS.

To make the collection of this information easier, Features, Attributes, and Values may be defined ahead of time and stored in files called Feature Lists (look-up tables) on the MC-GPS.  The system of Features, Attributes, and Values is a powerful and flexible system, limited only by your imagination.  Once you have set up a Feature List, you may use it for different GPS Jobs.

MC-GPS also offers outstanding navigating capabilities.  You may pre-define a set of Navigation Routes.  A Route is simply a series of  Waypoints (known locations) set up in a pre-defined order.  To follow a Route is to begin at Waypoint #1 and proceed sequentially through the Waypoints to the last one.

In addtion, MC-GPS can be used as a GPS Base Station.  You can setup your MC-GPS over a known point in the field, such as a survey marker, and collect Base Station data.  You can then download the Base Station file to your PC and use it to perform differential corrections on your field data.

Finally, if you have the CMT-Field 3.4 option, you will also have the use of integrated Traverse data collection functions.  Once you have established at least one GPS position, you will be able to use standard traversing methods to enter distance, slope and bearing data to new positions.  These positions will be treated just as if they were GPS points.  You can do a standard traverse where you move from position to position, or you can do a radial traverse where you take multiple new points from a single reference point.

All the functions provided by the MC-GPS field data collection and navigation program are explained in detail in the following sections.  You are encouraged to study each section so that you may be able to fully utilize the capabilities of your MC-GPS.

This manual is organized as follows:

Section 1 	Section 1 provides an overview of the MC-GPS hardware and operations.

Section 2 	Section 2 provides a tutorial to quickly familiarize you with the MC-GPS and PC-GPS software.

Section 3 	Section 3 provides instructions on setting up the MC-GPS to optimize it for your requirements.

Section 4 	Section 4 discusses how to create and maintain a database of Jobs, Feature Lists and Routes on the MC-GPS.  Note that these lists can also be set up in PC-GPS and then downloaded to the MC-GPS.

Section 5 	Section 5 shows you how to use MC-GPS to collect field data and how to view the collected data. Section 5 also discusses how to use the Traverse functions built into CMT-Field 3.4, and how to use MC-GPS as a Base Station.

Section 6 	Section 6 shows you how to use the navigation functions of the MC-GPS.

Section 7 	Section 7 contains instructions for transferring data files between MC-GPS and another computer.

Appendices A through F provide additional information on battery maintenance, general care, accessories and peripherals as well as other options available for MC-GPS.

# Section 1 - Overview of MC-GPS

## 1.1  Equipment

MC-GPS is based on the rugged, waterproof MC5 hand-held Field Data Recorder.  Each unit comes with two rechargers (CMT Part# ACC-RE3B and ACC-RE6) and a communications cable for connecting to an IBM compatible PC.  The MC5 Field Data Recorder has a back-lit liquid crystal display (LCD), a keyboard for entering data and commands, a built-in antenna and an integrated GPS receiver for receiving satellite signals, as well as two RS-232 interface ports (COM ports) for communication with an external device such as a PC or a bar code reader.  The MC5 is powered by a rechargeable battery pack.  The GPS receiver is powered by a rechargeable battery pack that is easily replaced in the field.

Built into each MC-GPS is a set of powerful GPS field data collection programs as well as ample memory for data storage.  The MC-GPS system is configured with a 3 MB RAM Disk drive, with 5 MB or 8MB options available.

Battery Power

There are two batteries used to operate the MC-GPS.  The first battery is the internal system battery.  This NiCad battery pack will allow you to operate the MC5 and to maintain your files for about 20 hours on a full charge.  The system battery is recharged using the ACC-RE3B (110V) or ACC-RE4B (220V).  Charge the system battery for 16 hours.  It is best if you let the system battery discharge as much as possible before you recharge it.  Do not overcharge the batteries by leaving the charger connected for longer than 16 hours.  If overcharged, the batteries may be damaged and their life and performance may be reduced.

The second battery supplies power to the GPS receiver.  This battery is attached to the back of the MC-GPS by four screws, and is field replaceable.  You may charge the battery pack while it is installed in the MC-GPS or removed from the MC-GPS as the charging jack is located on the battery pack.  When the battery level goes down to about 20% of full capacity, MC-GPS will beep and display a “LOW” warning message and then it will automatically shut off the GPS receiver.  At this time, the MC5 is still operable, but it is a good idea to recharge the GPS receiver battery pack or to replace it with a recharged battery pack.  Extra battery packs (GPS-ENG-BAT) are available from CMT and its dealers.

To charge the battery pack, use the adapter/charger marked ACC-RE6 (110V) or ACC-RE11 (220V).  Recharge 16 hours.  You can plug it in before you leave the office and unplug it when you return the next morning.  Do not overcharge the batteries by leaving the charger connected for longer than 16 hours.  If overcharged, the batteries may be damaged and their life and performance may be reduced.

Once you have replaced your GPS receiver battery pack, you will want to adjust the charge level of the receiver battery which is displayed in the STATUS screen.  To adjust this level, press [SH1] and then [ENTER] to move to the System Status screen and then press [F2](SECND) to move to the Second Battery screen.  In the Second Battery screen, you can use the [+] and [-] keys to adjust the charge level to reflect the current status of the receiver battery.  Press [ESC] twice to return to the MC-GPS Field software.

To save power when you are done with a data-logging session, you may turn off the GPS receiver from within the GPS application.  To turn off the GPS receiver, call up the STATUS screen from anywhere in the program by pressing the [ON] key, then press [F1](OFF).

Note:	If the GPS receiver is turned off, MC-GPS will automatically shut itself off after about 6 minutes of inactivity.  When you turn the unit on again, you will find yourself where you were before the unit turned itself off.

If you will not be using your MC-GPS for an extended period of time, make sure you fully charge the system battery before putting the unit into storage.  You should then charge the system battery once a month while the unit is in storage.

## 1.2  The Keyboard

MC-GPS has a 55-key keyboard with dedicated alpha and numeric keys, 5 function keys, 4 direction keys and 2 shift keys.  The standard operation or character of the key will be printed in black, while the shifted operation(s) or character of the key will be printed in yellow or white.  To activate a yellow key operation or character, first press the yellow [SH2] key, release it, then press the desired key.  To activate a white key operation or to change the case of an alpha character, first press the white [SH1] key, release it, then press the desired key.

### 1.2.1  [ON]

The [ON] key performs several functions:

- To turn on the MC-GPS, press the [ON] key.  The MC-GPS Field Data Recorder is configured to automatically run the MC-GPS data logging programs.  If you see a “WAIT” annunciator in the upper-right-hand corner of the display, it means the built-in GPS receiver is also turned on and is trying to establish communication with the MC-GPS operating system.  This usually takes a few seconds.

Following are two ways of turning off the MC-GPS from within the GPS application:

- Press the [SH2] key and then press the [ON] key to execute the OFF function labeled in yellow.  You may do this at any time. When you turn on the MC-GPS again, the program will pick up from where it was turned off.

- Press the [F1](OFF) key from the MAIN MENU.

Note:	If the GPS receiver is turned off, MC-GPS will automatically shut itself off after about 6 minutes of inactivity.  When you turn the unit on again, you will find yourself where you were before the unit turned itself off.

- Normally, pressing the [ON] key from within the MC-GPS application displays the STATUS screen (Section 1.5.2).

### 1.2.2  [ENTER]

The [ENTER] key is used for confirming the current menu selection, data entry or command execution.  When you are at a data entry field, pressing [ENTER] will take you to the next data entry field.

The special [SH1][ENTER] key sequence is used for accessing the SYSTEM STATUS screen of the MC-GPS unit (Section 1.5.4).

### 1.2.3  Shift Keys [SH1] and [SH2]

The keys labeled “SH1” and “SH2” are always used in combination with a second key.  When you press [SH1] followed by another key, you will activate the function corresponding to the white label printed on that key, or you will change the case of an alpha key.  When you press [SH2] followed by another key, you will activate the function corresponding to the yellow label printed on that key.  Following are a few examples:

[SH2][ON](OFF)	Turn off MC-GPS as well as the GPS receiver.

[SH1][ç](INS)	Switch to INSERT mode for editing.

[SH2][F1](F6)	Perform the [F6] function (calling the Help screen).

### 1.2.4  Function Keys [F1] - [F5]

The Function keys [F1],[F2],[F3],[F4],[F5] located in the top row of the MC-GPS keyboard perform different operations depending on the currently active software routine.  Their current function assignments are shown at the bottom line of the LCD display and are referred to as softkey commands.  Some Function keys also perform functions which are accessible from any screen in the MC-GPS software.  These functions are activated by first pressing the [SH2] key, releasing it, and then pressing the desired Function key.  The available functions are described as follows:

[SH2] + [F1]	Displays a general Help screen which describes the functions of the MC-GPS keyboard.

[SH2] + [F2]	Displays the SV STATUS screen (see Section 1.5.3).

[SH2] + [F3]	Displays the MC-GPS COMPASS screen (Section 6.1).

### 1.2.5  Direction Keys [Ù], [Ú], [×], [Ø]

The four keys labeled with arrowheads are called the direction keys.  They are used for moving the cursor to the desired location on the screen or for pointing to the desired option among the available selections.

- [Ù] and [Ú] move the cursor between menu selections or data entry fields.

- [×] and [Ø] move the cursor left and right, respectively, in a data entry field.  If the data entry field is one that is preceded by a small triangular pointer (    ), it contains a pre-defined list of values and will not accept data input.  When such a field is highlighted, you may make a selection by using [Ø] and [×] to toggle among the available options.  The [×] and [Ø] keys are also used for pointing to a selection on an optionÙÚ×Ø menu.

### 1.2.6  [ESC]

The [ESC] key is used for “escaping” from the current operation.  Press [ESC] to move from the current sub-menu one level closer to the MAIN MENU.  You will use [ESC] often.

If you press [ESC] when the MAIN MENU is displayed, MC-GPS will ask you if you really wish to exit the MC-GPS application.  Select [F1](NO) to stay within the MC-GPS application.  If you pressed [F5](YES) by mistake and find yourself in the MC-GPS operating system, press [ENTER] until you see [F5](OFF).  Then press [F5](OFF) to turn off the unit.  Now, when you press the [ON] key, the unit will restart the MC-GPS field application programs.

### 1.2.7  RESET Button

Under normal conditions, you do not need to reset the MC-GPS computer.  However, you should know how to reset the machine in case there is a need to do so.

There are two ways to reset and reboot the MC-GPS:

- Hold down the [F3] key and then press the [ON] key.  This is the equivalent of a [CTRL][ALT][DEL] sequence on a PC.  MC-GPS will reboot itself.

- Use the tip of a ball-point pen to press the RESET button located between the COM ports of the MC-GPS.  MC-GPS will turn itself off.  Press [ON] to restart the MC-GPS program.  After some screen activities, you will see the MAIN MENU displayed.

Warning:	Do not reset the MC-GPS while you are collecting data in the MC-GPS program.  Doing so may irreversibly corrupt the associated data files.  In case you get stuck in a program or in the MC-GPS operating system, try holding the [F2] key down and pressing [ON].  Normally, this will terminate any running program and turn off the MC-GPS.  When you press the [ON] key again, the program will be restarted without any adverse effect on the data files.

Note:	When MC-GPS is turned on after a reset, it will automatically turn on the built-in GPS receiver.  If you will not be collecting data soon, you may turn off the GPS receiver to save battery power.

## 1.3  General Operations

### 1.3.1  Selecting Menu Options

MC-GPS is a menu-driven software package designed around a MAIN MENU which branches into sub-menus and function screens.  The menu system is designed to allow you to move quickly and easily between functions as well as to easily respond to screen prompts.  All sub-menus and function screens are accessed from, and return to, the MAIN MENU.

One item on each menu is always highlighted by inverse video; it is the current selection.  You may use the direction keys to change the current selection.  If the menu is larger than one page, the contents of the screen will scroll, as you move up or down, to keep the current selection in view.  When the current selection is the one that you want, press the [ENTER] key to activate it.

In lieu of highlighting the desired function and pressing [ENTER], you may simply press the key corresponding to the first capitalized letter of the function.

The MAIN MENU displayed below is an example of one type of menu provided by MC-GPS.  It is automatically displayed when you first turn on the MC-GPS.  It lists the major functions of the software.

Use the direction keys to highlight “Help” on the MAIN MENU.  If you press [ENTER] now, a help message will be displayed.

15:37	MAIN MENU

Collect data

Navigate

Setup

Help

SV PRN:  02  06  19  21  24  30

N3D3

LISTS

OFF

XFER

VER

Some menus allow you to choose several items from a list.  Such a list is displayed when file transfers or file deletions are to be performed.  You may use the direction keys to move the cursor to any item on the list.  At the bottom of such screens, there usually is a set of Functions provided in the form of a Softkey Menu:

Press [F1](MARK) to make a choice; press the same key to cancel the choice.  A chosen item is indicated with a chevron “»”.  When all of your choices are made, pressing the appropriate Function key will initiate the specified operation on the selected files.  In the above example, Routes “02069601” and “02069608” have been marked for deletion.

15:37	ROUTES

<NEW>	PDXSEA

»02069601	02069606

02069602	02069607

02069603         »02069608

02069604	02069609

N3D3

HELP

MARK

DEL

### 1.3.2  Using Softkey Commands

The Function keys enable you to issue commands by pressing just one or two keys.  The commands corresponding to the normal Function keys are displayed at the bottom of the relevant display screen.  To execute such a softkey command, simply press the corresponding Function key.  For example, when the MAIN MENU is displayed, you may check into the version number of the MC-GPS software by pressing [F5](VER) to call up the following screen.  Press any key to return to the Main Menu.

The commands corresponding to the shifted Function keys are identified by the labels placed above the Function keys.  The definition of the shifted Function keys are generally independent of the context of the current screen.  However, certain shifted Function keys are active only within the MC-GPS application.

MC-GPS

VERSION 2.50-P

(ENGINE:  MVX E350 004)

(OPTION:  RTCM + NMEA)

(c) COPYRIGHT 1993-1996

CORVALLIS

MICROTECHNOLOGY, INC.

N3D3

[SH2][F1](HELP)	Display the general message about keyboard operations within the GPS application.

[SH2][F2](SATS)	Display the SV STATUS screen within the GPS application (Section 1.5.2).

[SH2][F3](CMPAS)	Pop up an on-screen compass (Section 6.1).

### 1.3.3  Getting On-Screen HELP

MC-GPS provides on-screen instructions in the form of 'help messages' for fields that require data input.  There are two types of help messages:

- Press [SH2][F1] to get general help about how to use the MC-GPS keyboard.

- Whenever you see the softkey label “HELP” displayed at the bottom of a screen, press [F5](HELP) to call up a message relating to the operations to be performed in the current screen.

When a long help message is displayed, you may scroll it up or down one line at a time by using the [Ú] and  [Ù] direction keys.

### 1.3.4  Aborting an Operation

In general, pressing [ESC] aborts an operation and takes you back to a previous screen.

If you press [ESC] when the MAIN MENU is displayed, MC-GPS will ask you if you really wish to exit the MC-GPS application.  Select [F1](NO) to stay within the MC-GPS application.

## 1.4  Entering/Editing Data

MC-GPS automatically acquires and computes position data and records it into the appropriate data fields.  You may enter additional information by using the keyboard or an external input device such as a bar code reader.

MC-GPS employs some special data input methods to help speed up the data entry process:

- The data in certain data fields will automatically increment by default.  For example, Waypoint record numbers will automatically advance by 1 for each new Waypoint record added.

- Many data entry fields contain a pre-defined list of items from which you may make a selection by using the [×] and [Ø] keys.  These fields can be easily recognized by the small triangular pointer placed at the end of the data input field prompt (e.g. DATUM   ).

In MC-GPS, blanks for data entry are displayed in inverse video on the screen.  Cursor position is indicated by an underline character.

After entering each piece of data into a data field, press [ENTER] to confirm and go to the next field.  In most fields, the cursor will automatically advance to the next data field when the current line is full.  Before you press [ENTER], you may edit the data.

In most data fields, you may edit the data by using [×] and [Ø] to place the cursor ÙÚ×Øat the position of the character or digit to be edited and then type over it.  To insert one or more characters, use [SH1][ç](INS) to toggle into the “INSERT” mode before entering the character.  To delete the character to the left of the cursor position, simply press [ç].

Some data entry forms are presented in a tabular format.  The columns define different variables/fields (e.g. Attributes and Values) and each row is an individual record.  In such a screen, the [×] and [Ø] keys are primarily used for moving the cursor to another column.  To edit a data field in such a screen, press [ç] when you first get to that data field.  Then you will be able to use  [×] and [Ø] to position the cursor at the character you wish to change.

The contents of the numeric fields in the various setup screens discussed in Section 3 may be changed by typing over the existing data.  In fact, as soon as you type in the first character, the field will be blanked and ready to receive the new input.

You will notice that the characters in the currently highlighted data entry field are somewhat magnified.  If, as a result, not all the characters of a long string can fit into the displayed data area, then the characters you already typed in will scroll to the left as you type in additional characters, up to the maximum permitted in that data field.

In the normal operational mode, you may use the [Ù] and [Ú] keys to move the cursor to the various data entry fields.  When a data entry form is bigger than the physical screen of the MC-GPS, you can use the direction keys to scroll it up, down, left or right.  Pressing [ENTER] or [Ú] at the last field of such a data entry form will take you to the next page of the entry form.

### 1.4.1  Automatic Data Entry

MC-GPS can accept data input directly from a number of devices.  Following are some examples:

- It automatically obtains and displays the information gathered by the GPS receiver.  All you have to do start the data logging process is press one key (Section 5.2.3).

- It accepts input from an external device such as a bar code reader.  When you activate the external device, the data that you normally would have to key in will automatically go into the appropriate data fields (Section 3.5.1).

- It automatically supplies default values for various setup parameters based on a standard configuration file.  In many cases, you will need to make little or no change to these values.

### 1.4.2  Error Checking

MC-GPS has various error checking facilities.  It rejects invalid data by issuing a series of beeps and, in most cases, an error message.  Typically, the error message will be displayed in inverse video on the bottom line of the display. Other invalid data entries such as attempting to enter alpha characters into a numeric field, or attempting to start a selection menu option with the wrong key, will be met with error tones from MC-GPS.

## 1.5  Status Indicators and Screens

MC-GPS provides a number of screens and indicators to help you quickly answer questions such as the following:

- Is the GPS receiver on?

- Is the GPS receiver tracking satellites?

- Which satellites are being tracked by the GPS receiver?

- What was the last position computed by the GPS receiver?

- What is the battery power level?

- How much memory space is there for storing additional data?

- What are the current settings for the communications ports?

### 1.5.1  Tracking Status Indicator

As long as the GPS receiver is turned on, the Tracking Status Indicator is visible in every screen of the MC-GPS programs and gives you an idea of the quality of the constellation of the satellites being tracked by MC-GPS.

When the GPS receiver is ON, MC-GPS displays the tracking status indicator in the upper-right corner of the screen.  It is updated frequently (at least every 2 seconds) and displays the following status information:

???	The GPS receiver is in an unknown state for a brief period of time.

OFF	The GPS receiver has automatically turned itself off.

IDL:  IDLe	The MC-GPS is not searching for satellites and is not computing a position.

STS:  Search The Sky	MC-GPS is searching the sky for any visible satellites.  It will enter this state if it has been on for 30 minutes and has not found any satellites to track.  It is usually in this mode if you are indoors or if the antenna is disconnected and you haven't turned off the GPS receiver.

ALT: ALTernate constellation	MC-GPS has determined that its position has changes significantly (300+ miles), and is re-calculating which satellites to track.

IAC:  Initial ACquisition	MC-GPS is determining the location of the available satellites, and which satellites to track.

ACQ:  Satellite ACQuisition	The MC-GPS is acquiring signals from one or more satellites.

TRK:  TRacKing	The MC-GPS is tracking at least one satellite.

N2D:  Navigating in 2D	The MC-GPS is navigating in 2D and is tracking at least 3 satellites.  The altitude used will depend on your selection in the ALT. CONTROL field of the SETUP RECEIVER screen.

N3D:  Navigating in 3D	MC-GPS is navigating in 3D and tracking 4 or more satellites.

F2D:  Featuring in 2D	MC-GPS is logging 2D RAW data and Feature data.

F3D:  Featuring in 3D	MC-GPS is logging 3D RAW data and Feature data.

F3C:  Realtime Corrections	MC-GPS is receiving corrections data and storing 1-5m accuracy data in real time.

When MC-GPS is navigating or logging data, the Tracking Status Indicator will display the current PDOP (Position Dilution of Precision) as the last character.  PDOP values are displayed to the nearest whole number.  For example, “N3D4” means MC-GPS is navigating in 3D and tracking 4 or more satellites, and the PDOP is 4.

If you are using the RTCM_104 function (Section 3.5.3), the “D” in the Tracking Status Indicator will become a “C” when the RTCM_104 message is being received and used by the MC-GPS.

IAC, IDL and STS are the three possible states the MC-GPS will enter when you first turn it on.  If you have used the MC-GPS before at all, it will most likely start out with IAC and advance to TRacKing (TRK) within a minute or so.

If the status indicator is blank for more than 5 seconds, this indicates that the GPS receiver power is OFF.  Press [ON] to go to the GPS STATUS screen, then press [F1](ON) to turn on the GPS receiver.

If the status indicator reads STS, it may be due to the following reasons:

- The antenna is obscured from the satellites.  This usually happens when the MC-GPS is inside of a building (during download, for example), but can also happen if you are trying to work in a particularly poor environment.  Always remember to turn off your GPS receiver if you will be working inside for any length of time.

- The GPS receiver  does not know one or all of the following things:  It’s current location, the location of the satellites, the current UTC time and the current UTC date.  All of this information is necessary for the GPS receiver to track satellites.

In either case above, you can remedy the problem by moving to the Setup screens.  From the Main Menu select Setup to move to the Setup Menu.  First select Time/Date.  In the TIME/DATE screen you will set the current time, the current date and the UTC offset for your current position, then press [F4](SAVE).  Next, select Receiver and move to the RECEIVER screen.  In the RECIEVER screen you will find fields to enter your current location (within 1-2 minutes is accurate enough).  You will also notice the SEND ALM field.  Toggle this field to YES using the rignt arrow key and then press [F4](SAVE).

In the MAIN MENU screen, the PRN numbers of the tracked satellites are shown near the bottom of the display while the GPS receiver is on.  When MC-GPS loses track of a tracked satellite, it will issue a beep to let you know.

Time until First Fix

When you first turn on MC-GPS (with the receiver ON), it immediately begins to compute its position.  This process is called start-up.

MC-GPS has two start-up modes:  Cold Start and Warm Start.

Cold Start	In cold start, the GPS receiver doesn't know its current position, time/date and enters Search the Sky (STS) mode.  When it acquires one satellite, it begins to collect an almanac.

There is rarely a reason why your MC-GPS should have to perform a cold start.  Even if it is brand new, it should have a current almanac and correct UTC time/date.

While the GPS receiver is tracking satellites, it also collects the most recent available ephemeris data.  Ephemeris data is precise orbit information that is broadcast by each satellite.

After the MC-GPS is in N2D/N3D/F2D/F3D modes, it updates the almanac/ephemeris data and stores the current position for future start-ups.

Warm Start 	Typically, your MC-GPS will do a warm start. In warm start, the GPS receiver has a reasonably current almanac and an approximate start-up position, date and time.  If that is the case, then the receiver will begin tracking satellites in a couple of minutes.

If MC-GPS has current ephemeris data (has tracked satellites in the past 2 hours), then it may begin tracking satellites sooner.

### 1.5.2  STATUS Screen

The GPS STATUS screen can be popped up from anywhere in the MC-GPS programs by pressing the [ON] key.

From this screen, you may turn the GPS receiver on or off.  You may also view the following information.  To return to the previous screen, simply press [ESC].

FIX:NEW	STATUS

MEM:  1021952    BYTES  (98%)

SYS/RCVR: 90/75      RCVR:  ON

LAT: 	       44   33  28.92	N

LON:	123   15  24.06	W

ALTITUDE:  753.38   FEET  MSL

N3D3

HELP

UTM

SATS

OFF

FIX	Data in this field indicates whether the receiver is computing new positions or if the position on the screen is from an old position fix (NEW/OLD).  If FIX is NEW, then it displays the current position.

If FIX is OLD, then it displays the last position output by the GPS receiver.

MEM	This field shows the number of bytes of available memory in the current file system.  The percentage of remaining memory is enclosed in parentheses.

SYS/RCVR	The displayed value is the percentage of power remaining in the MC-GPS system and GPS receiver batteries.

RCVR	This field indicates whether the GPS receiver is ON or OFF.

LAT, LON	The last position computed by the GPS receiver is displayed in

(or NORTH,	Latitude and Longitude if the coordinate system selected is LLA.

EAST)	If the coordinate system selected is UTM, the position data is displayed in Northing and Easting.  For the State Plane Coordinate (SPC) system, the position is displayed as “X” and “Y” values.

ALTITUDE	The altitude will be displayed as MSL (Mean Sea Level) or HAE (Height Above Ellipsoid) depending on how your MC-GPS is setup (Section 3.3).

ZONE, STATE, R	Zone data will be displayed only if the currently selected coordinate system is UTM or State Plane.  State and Region data will be displayed only if the currently selected coordinate system is State Plane.

Function Key Assignments:

[F1]OFF/ON	Turn power OFF or ON for the GPS receiver.  If you do not need a position and do not need to log Features, you may turn off the GPS receiver.  This will substantially reduce the power drain on the battery pack.  When the receiver is OFF, the status indicator (at the upper-right corner of the screen) will not display a message.  If you choose to turn off the receiver, MC-GPS will display a message to prompt for your confirmation.  Press [F5](YES) to turn off the receiver.  When the receiver is OFF, MC-GPS will not read data from the GPS receiver.

Note:	Turning the MC-GPS off will always turn off its built-in GPS receiver.  However, this will not change the GPS receiver status setting in the STATUS screen.  If RECEIVER status was “ON” before you turned off MC-GPS, turning on MC-GPS will automatically turn on the GPS receiver.  If RECEIVER status was OFF before you turned off MC-GPS, turning on MC-GPS will not turn on the GPS receiver.

[F2](SATS)	Display the SV STATUS screen to show the status of the satellites being tracked by the GPS receiver.  See Section 1.5.3 for details.

[F3](UTM),	Toggle between LLA, UTM, and SPC Coordinate systems for position display.

(SPC),(LLA)

[F5](HELP)	Display a helpful message with respect to the selected data field.

### 1.5.3  Satellite Status

Use either of the following methods to view the status of the satellites being tracked by the GPS receiver.  The SV STATUS screen will display only if the GPS receiver is turned on.

- Press [F2](SATS) from the STATUS screen (Section 1.5.2).

- Press [SH2][F2] from anywhere in the MC-GPS programs.

.

Press [F4](PGDN] to display any additional satellites tracked.

15:37	SV STATUS

CH    PRN   AZI    ELEV    SIG/LVL

1      03     98       12        80.00

2      09   134       62        63.00

3      12     62       45        46.00

4      18    332      87        50.00

5      24    227        6        75.00

N3D3

HELP

HLTH

DOPS

LOC

PGDN

CH	This is the channel number of the GPS receiver corresponding to the tracked satellite.

PRN	This column shows the PseudoRandom Number of each tracked satellite.

AZI	This column shows the direction of each satellite from North.

ELEV	This column shows the elevation (in degrees) of the satellite above the horizon

SIG/L	Signal level of the satellite indicates how good the signal is that you are receiving from each satellite.  Low signal strength can be due to a faulty connection with the antenna.

MD	This field shows the GPS receiver Tracking Mode.  To ensure accurate corrected data, the differential correction function in PC-GPS will only accept raw data collected in Mode 08 (phase-lock maintained, producing carrier-smoothed data).

All data on the screen is updated once per second regardless of the data logging interval.

Function Key Assignments:

[F1](LOC)	Display the location of the satellites in relation to your position.  This is a two-dimensional graphical representation of the information in the SV STATUS screen.  It is what you would see if you were high up and look down upon the satellites.  In the diagram, you are located at the center.  “N” is “true North”.  The crosses represent the satellites in view.

[F2](DOPS)	Display the first four satellites being tracked as well as the VDOP and PDOP of the satellites.  Also show

15:37	LOCATION

N3D3

S

E

W

N

HELP

[F3](HLTH)	Display the health of each of the satellites based on the almanac.

15:37	DOPS

CH	PRN	  AZI      ELEV    SIG/L

1	03         98      12       41.00

2	09       134      62       33.00

3	12         62      45       36.00

4	18       332      87       50.00

HDOP: 2.4  VDOP: 2.9  PDOP:  3.8

SAT VIS#:  8   SAT TRK#:  4

N3D3

Press [F4](PGDN) to display any additional satellites tracked.

15:37	SV HEALTH

1      2      3      4      5      6       7

A     A      A     A      A      A      A

U     H      H     U     H      U      U

8      9	  10    11    12    13     14

A     A      A     A      A      A      A

U     H      H     U     H      U      U

N3D3

SAVE

PGUP

PGDN

The ALMANAC is a data file (ALM.MVX) containing information gathered from the satellites.  This information is used by the GPS receiver at startup to determine which satellites to search for.  Satellites that are functioning and available are considered healthy.  Satellites that are either not functioning or otherwise not available are declared unhealthy.

You may let the GPS receiver automatically compute the health status of each of the satellites by using the almanac, or you may override some or all of the satellite health indicators. As a general rule, you should let the GPS receiver determine the healthiness of the satellites unless you have a special circumstance and know exactly what you are trying to accomplish.  Otherwise, you may adversely affect the ability of the MC-GPS to track satellites.

The GPS receiver will collect updated information on the health of the satellites automatically when a new almanac file is collected.

Satellite PRN numbers are displayed on the second and fifth lines.  Satellites are numbered 1-32.  Only 14 satellites can be shown on this screen at one time.  You can press [Ú] to move to the next screen.

The 3rd and 6th lines allow editing.  You may enter 'A', 'I', or 'E'.

A = Automatic	This setting lets the GPS receiver automatically read the health status of the satellite.

I = Include	This setting requires the GPS receiver to include this satellite for  position computations.

E = Exclude	This setting requires the GPS receiver to exclude this satellite from position computations.

The 4th and 7th lines of the display reflect the GPS receiver's report of the satellite health status.
		H = Healthy
		U = Unhealthy

The following function keys are available from the SV HEALTH screen:

[F3](PGUP)		Go back to the first page of satellite health information.

[F4](PGDN)		Display the next page of satellite health information.

[F5](SAVE)		Press this key to save any changes you have made to this screen.

[F4](PGDN)	Press [F4](PGDN) to display the next page of satellite status information, if any.

[F4](PGUP)	Press [F4](PGUP) to go back to the first page of satellite status information.

[F5](HELP)	Display a help message with respect to the SV STATUS screen.

### 1.5.4  MC-GPS System Status

Use the [SH1][ENTER] key sequence to call up the SYSTEM STATUS screen.  From this screen, you can view the current settings of the COM ports and the display contrast.  This screen also reports whether the display backlight, key-repeat and key-beep are turned on.

The functions displayed at the bottom of this screen permit you to view the system battery level as well as adjust the screen contrast or turn the backlight, key-repeat or key-beep on or off.

BEEP

REP

PREV

F1           F2                           F5

- SYSTEM STATUS -

COM1:	19200, N ,8, 1

COM2:	4800, N, 8, 1,X

Contrast:	43.7%

Backlight:	OFF

Key Repeat:	OFF

Key Beep:	OFF

SYS

SECND

CONT

MORE

BKLIT

Function Key Assignments:

[F1](SYS)	Display the power meter for the system battery.

[F2](SECND)	Display the power meter for the GPS receiver battery.  You can adjust this battery meter when you replace the GPS receiver battery by using the [+] and [-] keys.

[F3](CONT)	Display the current contrast level of the LCD display.  You can adjust the contrast of the display in this screen using the [+] and [-] keys.

[F4](BKLIT)	Turns the LCD backlight ON and OFF.

[F5](MORE)	Displays a second set of function key labels, described below.

Second set of Function Key Assignments:

[F1](BEEP)	Turns the keybeep function ON and OFF.  If the keybeep function is ON, a beep will sound each time a key is pressed.

[F2](REP)	Turns the key repeat function ON and OFF.  If the key repeat function is ON, when a key is pressed and held the keystroke will be repeated.

[F5](PREV)	Return to the original set of function key labels.

To exit the SYSTEM STATUS screen, simply press [ESC].

Note:  	You will not be able to use the {SH2][ON] key sequence to turn off the unit from the SYSTEM STATUS screen.  To turn of the unit, first press [ESC] to exit the SYSTEM STATUS screen, then press [SH2][ON].

# Section 2 - Tutorial

This tutorial is designed to quickly familiarize you with the most important aspects of the MC-GPS field data collection programs and its companion product, the PC-GPS Windows software.  The goal is for you to begin using the system as soon as possible.  This tutorial highlights the essential functions of GPS data collection and post-processing, but does not cover all the functionality provided by MC-GPS and PC-GPS.  For additional information, please refer to the other sections of this manual as well as the PC-GPS manual.

Please read Section 1 in its entirety.  Then go through the lessons in this section in the order presented.  After completing these lessons, you should have a good idea of how to do the following:

- Set up the MC-GPS receiver.

- Collect GPS field data, making use of a Feature List.

- Transfer field data to a PC.

- Perform differential correction on the PC.

- Create a Feature List on the PC and transfer it to MC-GPS.

## 2.1  Lesson 1 - Getting Started

### 2.1.1  Turn on MC-GPS (MAIN MENU)

The following top-level functions are listed on the MAIN MENU:

15:37	MAIN MENU

Collect Data

Navigate

Setup

Help

SV PRN:  02  06  19  21  24  30

N3D3

LISTS

OFF

XFER

VER

Collect Data		GPS data logging (Section 5)
	Navigate		Navigation functions (Section 6)
	Setup 			Setup for various important parameters (Section 3)
	Help			A general help message about the MAIN MENU

The following functions are provided through the Function keys [F1] - [F5]:

[F1](OFF)	Turn off the MC-GPS Field Data Collector as well as the GPS receiver.

[F3](LISTS)	Create and maintain Jobs, Feature Lists and Routes (Section 4).

[F4](XFER)	Transfer files to and from a PC (Section 7).

[F5](VER)	Display information about the software and the GPS receiver.

Please do the following warm-up exercise now:

- Press  [Ú] to highlight “Help”, then press [ENTER].

- Read the displayed message.  Press [Ú] or [ENTER] to scroll additional lines into view.

- Press [ESC] to return to the MAIN MENU.

- Now, press the [F5] key which corresponds to the softkey label “VER”.  Look at the MC-GPS software version number displayed on the screen.

- Press [ESC] to return to the MAIN MENU.

### 2.1.2  Turn the GPS Receiver ON/OFF

MC-GPS can track satellites and collect GPS data only when its GPS receiver is turned on.  After doing a job, you would want to turn off the GPS receiver to save battery power.

Procedure for turning off the GPS receiver

- Press the [ON] key while the MC-GPS is turned on.  This pops up the GPS STATUS screen.  If [F1] is labeled "ON", your GPS receiver is already OFF.  If this is the case, skip to step 3.

- Press [F1](OFF) to turn off the GPS receiver.  You will be prompted to confirm that you wish to turn the GPS engine off.  Press [F5](YES) to confirm.  Note that the Tracking Status Indicator in the top right corner has disappeared, and the [F1] key is now labeled "ON".

- Press [ESC] to return to the previous screen.

- If you wish to turn off both the GPS receiver and your MC-GPS, press [ESC] until you are back at the MAIN MENU.  Then press [F5](OFF).

Procedure for turning on the GPS receiver

- Press the [ON] key while the MC-GPS is turned on.  This pops up the GPS STATUS screen.  If [F1] is labeled "OFF", your GPS receiver is already on.  If this is the case, skip to step 3.

- Press [F1](ON) to turn on the GPS receiver.

- Press [ESC] to return to the previous screen.

### 2.1.3  Check the Tracking Status Indicator

Before you use MC-GPS for a job you should take it outside and verify that it will track satellites.  The Tracking Status Indicator in the upper-right corner of every MC-GPS screen shows the quality of satellite reception.  The Tracking Status Indicator is only visible when the GPS receiver is ON.  Please refer to Section 1.5.1 for a more detailed explanation of the Tracking Status Indicator.

If you are getting good satellite reception, the Tracking Status Indicator will say N3D#. "#" is a number that shows the current PDOP, an indication of the quality of the GPS satellite constellation the MC-GPS is tracking.  Any number less than 6 is an acceptable PDOP number.

When you first turn on your MC-GPS Field Data Recorder, you will see the Tracking Status Indicator progress from ACQ (Satellite Acquisition) through TRK (Tracking), N2D# (Navigating in 2D), then finally N3D# (Navigating in 3D).  It should take only a few minutes for the unit to start tracking satellites.

### 2.1.4  Set up the GPS Receiver

If the MC-GPS has a problem tracking satellites on a clear day, it may not have been properly set up.  In that case, perform the following procedure:

- Start in the MC-GPS MAIN MENU.  (If you are not at the MAIN MENU, press [ESC] until you are.)

- Check to see that the GPS receiver is turned ON (Section 2.1.2).  Also check that the Time/Date are correct (Section 3.4).

- Highlight “Setup” and press [ENTER].

- Highlight "Receiver" and press [ENTER] again.

Enter your Latitude and Longitude in the appropriate fields.  Approximate values are good enough; you only need to be precise to within a few degrees.

15:37	    RECEIVER

LAT:	44	33	49.35	N

LON:	123	15	49.50	W

MSL: 125.1        M	EL. MASK:  5

PDOP    7    VDOP    6    HDOP     3

ALT CTRL    AUTO      ALM?  NO

SEL CONSTELLATION    CANYON

N3D3

OFF

HELP

SAVE

- Press [F4](SAVE) to send the information displayed on this screen to the GPS receiver.

- Press [ESC] until you are back to the MAIN MENU.

## 2.2  Lesson 2 - Field Data Collection

After you have verified that the MC-GPS Field Data Recorder will track satellites, the next step is to use it to collect field data.  Assume that you wish to map a sidewalk and record the location of a water fountain by that sidewalk.  This means logging a Line Feature, temporarily interrupting it to record a Nested Point (Section 5.2.3) Feature and then continuing with the Line Feature.

In this lesson, we will make use of the PARK.FBR Feature List that was loaded into your MC-GPS at the factory.  A Feature List is a list of the Features, Attributes, and Values you expect to encounter in the field.  Feature Lists provide a quick and convenient way to tag GPS field data.  If, for some reason, you do not have the PARK.FBR file in your MC-GPS system, you may transfer it from your PC-GPS package (Section 2.4.9).  If you do not have the PC-GPS software available, you could follow the instructions in Section 4 to setup a simple Feature List.  If you choose not to use any Feature List, you could define each individual Feature just before logging GPS data for it.

Please note that the examples in this section do not involve Carrier Phase GPS data logging.

### 2.2.1  Start a New Job

- At the MAIN MENU, highlight “Collect Data” and then press [ENTER].  The JOB DATA screen will be displayed.

Select “NEW” for Job File, “PARK” for Feature List and “NONE” for Route.  You may leave the other (optional) fields blank.  When done, press [F1](GO).  A default job file name similar to the following will pop up in a box.  You may edit it, if you wish.  For now, just accept it by pressing [ENTER].

15:37	JOB DATA

JOB FILE  NEW	    IDA

FEATURE LIST	PARK

ROUTE  NONE

CREW:

DEPT:

DESC:

N3D3

GO

BASE

SAVE

HELP[

VIEW

2.2.2  Select and Describe a Feature

15:37	JOB DATA

JOB FILE  NEW		    IDA

FEATURE LIST	NONE

ROUTE  NONE

CREW:

DEPT:

DESC:

N3D3

FILE NAME:  A061011A

GO

BASE

SAVE

HELP[

VIEW

- Once you have named your Job, you will move to the FEATURES screen.  Here you will see a listing of the Feautes in your Feature List.  Highlight “SIDEWALK”.  Note that it is a Line type Feature to be logged in DYNAMIC mode with a 1-sec interval between position Fixes.  Press [ENTER] to select this Feature and display its associated FEATURING screen.

- When you have selected your Feature, the Featuring screen will be displayed.  You will be prompted to select Values for the Attributes associated with the Sidewalk Feature.

To make a VALUE selection, use [Ù] or [Ú] to highlight the desired Value, then press [ENTER].  Select “GOOD” from the VALUE box for the “CONDITION” Attribute.  Then select “CONCRETE” for the “SURFACE” Attribute.

15:37	FEATURING

LINE:  SIDEWALK

ATTRIBUTE	VALUE

CONDITION

N3D3

GRAVEL

BLACKTOP

CONCRETE

SURFACE

SELECTEDIT

### 2.2.3  Store a Line Feature

After you have finished assigning appropriate Attributes and Values to the Feature, the FEATURING screen will look like the following:

Check the Tracking Status Indicator in the top right corner of the screen.  N3D# (where # is less than 6) indicates good reception.  It means that MC-GPS is tracking satellites correctly and is ready to store 3D (three dimensional) data but is not storing it yet.  If Navigation Beep is turned on (Section 3.2), you will hear a high-pitched beep once per second whenever the Tracking Status Indicator displays N3D#.

15:37	FEATURING

LINE: SIDEWALK

ATTRIBUTE	 VALUE

CONDITION	GOOD

SURFACE	CONCRETE

N3D3

DEL

HELP

STORE

OFFSET

NEW

- Now, press [F4](STORE) to begin data logging.  The Tracking Status Indicator changes to F3D#, which means that MC-GPS is storing your Feature descriptions along with 3D GPS data.  At the same time, the sound of the beep changes to a lower pitch.  A pop-up screen reports the number of position Fixes collected so far.  This figure is updated once per second.

Walk along the sidewalk for a minute or two at a normal pace.

15:37	DYNAMIC

LINE:  SIDEWALK

F3D3

STORING DATA NOW...

COLLECTED:67

INTERVAL:1 SEC

PAUSE

POS

EDIT

END

PNT

### 2.2.4  Store a Nested Point

- Let's say you now come upon a water fountain.  Stop walking and press [F2](PNT).  You will see the Nested Point screen, as shown below.  If you wish to change the data logging session time for one of the listed Features, highlight that Feature and press [F3](EDIT).  For this example, we will just use the displayed session time.

Highlight "FOUNTAIN" and press [ENTER].  Then select the desired Values for the Attributes “WORKS?” and “QUALITY”.  When done, the screen will look like the following:

15:37	NESTED POINT

FEATURE	  	       M	SESS

TREE		   S	80

CAR		   S	20

FOUNTAIN		   S	20

STATUE		   S	15

CORNER		   S	60

N3D3

EDIT

NAV

HELP[

Press [F4](STORE) and stay at that spot for 20 seconds to record the GPS data for the water fountain.

15:37	NESTED POINT

POINT:  FOUNTAIN

ATTRIBUTE	 VALUE

WORKS?	WATER ON

DEPTH	GOOD

N3D3

QUALITY

DEL

OFFSET

STORE

HELP

NEW

### 2.2.5  Continue with the Line Feature

When MC-GPS has finished logging the Nested Point, it will ask if you want to continue the sidewalk Feature.

Press [F1](CONT) to continue storing coordinates for the sidewalk.

15:37	NESTED POINT

POINT FEATURE:  FOUNTAIN

F3D3

CONTINUE LINE/AREA FEATURE OR ENTER ANOTHER NESTED POINT.

CONT

PNT

- Begin walking again along the sidewalk.  The DYNAMIC screen will again be displayed to show you the progress of data logging.

- After a few minutes, press [F5](END) from the DYNAMIC screen to end the data logging session.

### 2.2.6  Turn OFF GPS Receiver

Since you are done collecting GPS data, you should turn off the GPS receiver so as to save battery power.  To do so, press [ON] to display the GPS STATUS screen .  Then press [F1](OFF).  Press [ESC] until you are back at the MAIN MENU.

## 2.3  Lesson 3 - Transfer a Job to PC-GPS

You have now collected a basic job.  The next step is to transfer this job to PC-GPS on your desktop computer and work with it there.  You must have PC-GPS installed on your desktop computer to continue.  Please refer to the PC-GPS manual for installation instructions.

- Connect COM1: on MC-GPS to COM1: (or , in some cases, COM2:) on your desktop computer, using the cable and adapters provided with your system.

- Start your PC-GPS software.  From the PC-GPS 2.5 Main Menu, select the FILE TRANSFER pull-down menu and the CONFIGURE function to verify the PC communication settings.  From the PC-GPS 3.2 Main Menu, select the GPS pull-down menu, then Transfer and then Settings.  The default communications port is COM1, and the default baud rate is 19200 baud.

- Click on the RECEIVE button.  This will place the PC into KERMIT server mode, ready to receive files from MC-GPS.

- On MC-GPS, press [F4](XFER) from the MAIN MENU.  Then press [F1](TYPE) until the FILE TYPE field shows “JOB”.  Press [F2](BAUD) until the BAUD RATE field shows “19200”.

Highlight the job you want to transfer (e.g. “A061011A”) and press [ENTER] to mark it.  You may mark more than one file for transfer.

5:37	FILE TRANSFER

FILE TYPE	JOB

BAUD RATE	19200

F3D3

A061011A

TYPE

RECV

SEND

BAUD

SUM

- On MC-GPS, press [F3](SEND) to start the transfer process.

- When the transfer is complete, press [ESC] on MC-GPS to return to the MAIN MENU, then press [F1](OFF).

## 2.4  Lesson 4 - Using PC-GPS

PC-GPS can perform many tasks on the GPS field data you have collected.  We will go through a few important PC-GPS functions by using the sample job, CMTTUT.FTR, that is provided with PC-GPS.  We recommend working through the following exercises with the sample job.  Once you have a good understanding of the operation of PC-GPS, feel free to work with your own Jobs.

Throughout the remainder of this tutorial, command sequences involving the PC-GPS pull-down menus will be presented in the following format: ”Command/Sub-level Command”.  For example, “FILE TRANSFER/RECEIVE” is to be interpreted as “Click on the FILE TRANSFER pull-down menu, then click on RECEIVE.”

Across the top of many screens in the PC-GPS utility, you will see a collection of icons referred to as the Tool Bar.  Each of these icons performs a useful function.  You can move the mouse pointer to a Tool Bar icon to see a yellow HELP MESSAGE that names the function of that icon.

Tool Bar  -  The Tool Bar is a way to quickly access frequently used functions.  For instance, the ZOOM FIT icon performs the same action as EDIT/ZOOM FIT function.  Note that if you hold the mouse on an icon, you will see a helpful message displayed on the  screen.

For more detailed information on the operation of PC-GPS, please refer to your PC-GPS User’s Manual.

### 2.4.1  View Field Data

From the PC-GPS Main Menu bar, select JOB/OPEN (FILE/OPEN for PC-GPS 3.2), and then double-click on “CMTTUT.FTR” in the listing of Jobs.

- You will see a mixture of small squares, dots, and color-filled areas.  Each small solid square represents a one-second GPS coordinate position fix in a Line or Area Feature.  Round dots are Point Features, and a color-filled region is the space inside an Area Feature.  The color of a dot or square is important.  Olive Green points are uncorrected, blue points are differentially corrected, and red points are offset (Section 5.2.4).  Offset points are colored red whether they are corrected or uncorrected.  (You check the correction status in the COORDINATES screen.)

- Do not do any editing until after you have differentially corrected the Job (Section 2.4.2).

- Click EXIT on the right side of the tool bar.  If PC-GPS asks you if you want to save changes, choose "NO".

### 2.4.2  Perform Differential Correction

Differential correction is the process of applying to your GPS field data a correction factor determined from the data logged by a GPS base station at a known location within the same time period.  Differential correction improves the accuracy of Coarse Acquisition code GPS data to 1-5 meters.  This section describes how to differentially correct CMTTUT.  You should be able to apply the same principle to differentially correct your own job.

To differentially correct a MC-GPS Job, you must have base station data for the entire time range of the field work.  For example, CMTTUT was started at 16:00 hours Pacific Daylight Time (PDT) and completed by 17:00 hours PDT on September 21st, 1994.  If you wish to differentially correct this job, you must have base station data for the same time range.  The file 092116.DIF is a one-hour base station file that covers the above time period.  The base file 092116.DIF will differentially correct the rover file CMTTUT.  092116.DIF was recorded with a 12-channel base station.

Select multiple base station files to differentially correct a job that spans a period of over one hour.  Detailed instructions on differential correction are provided in the PC-GPS User’s Manual.

- Select DIFFERENTIAL/CMT DIFFERENTIAL from the PC-GPS Main Menu bar (GPS/DIFFERENTIAL/CA CODE for PC-GPS 3.2).  You will be presented with the following screen:

- .

- BASE TYPE must be Leica MX9000, since 092116.DIF was collected using data from a Leica base station.  If you wish to learn how to differentially correct with a Trimble Pathfinder base station, please refer to the PC-GPS manual.  ROVER TYPE must also be Leica Engine, since MC-GPS uses an integrated Leica GPS receiver.

- Select CMTTUT.FTR as the INPUT FILE (the file you wish to correct), and select 092116.DIF as the DIFFERENTIAL FILE (the base station file).  Only certain base station types require an EPHEMERIS FILE.  You do not need one for Leica MX9000 base station data, so leave it blank.  Notice that the OUTPUT FILE in the lower-right quadrant of the screen now shows the same name of the INPUT FILE.  Since you want to save the original field data, change the name of the OUTPUT FILE to CMTTUTC.FTR.  Note that a corrected file called CMTTUTD.FTR is already in your PC for your reference.  If you forget to change the name of the output file, PC-GPS will remind you.

- Click the SETUP button to move to the Differential Setup screen. It is very important that you enter the correct coordinates for the base station you are using.  The coordinates of the base station that 092116.DIF came from are:
		Latitude:	 44	33	47.2204	N
		Longitude:	123	15	47.4256	W
		HAE:		63.51  Meters

- Click OK to return to “GPS differential” screen.  Now click DIFFERENTIAL to start the correction process.

The DATA TIME RANGE screen is first displayed to tell you whether you have selected the correct base station data.  It probably says "Time range matches correctly".  If this screen ever says "Warning! Time range does not match!", then you probably selected the wrong base station file.  Click on the CANCEL button and try again to select the correct base station file from the “GPS differential” screen.  If your time ranges match correctly, proceed.

- Click OK.  The differential correction process takes a few minutes.  The actual processing time depends on the computer you are using and the size of the files involved.  PC-GPS will display a window to report on the progress of the differential correction process.  There are four stages, and PC-GPS will display an updated “percentage completed” value for each stage.

- The COORDINATE UPDATING window will report on the correction process when it is finished.  It tells you that, of 63 total coordinates, all 63 coordinates of CMTTUT were corrected.  Click OK to return to the “GPS differential” screen.  Now click CANCEL until you are back to the MAIN MENU.

### 2.4.3  View and Edit a Job

Go back to JOB/OPEN and choose CMTTUTC.FTR.  Then open CMTTUT.FTR also.  Select WINDOW/TILE.  The corrected and the uncorrected version of CMTTUT are now side by side.

- Uncorrected GPS coordinates typically have 50-100 meters of error.  Notice that CMTTUT looks pretty good when compared to the differentially corrected version of the same job, CMTTUTC.  This is because Selective Availability (SA), the deliberate scrambling of GPS signals by the US Government, happened to be turned off when this sample was collected.  SA is usually on, so your uncorrected GPS jobs will typically look worse than CMTTUT before differential correction.

- Click on CMTTUTC, and the label at the top of that window will darken.  It is now the active window.  You can only edit the active window.

### 2.4.4  Cut Uncorrected Points

Often, the first thing you do after differentially correcting a job is to remove all uncorrected points by using EDIT/CUT/CONDITIONAL/CUT ALL UNCORRECTED.  If there are some points in one of your Jobs that do not differentially correct and you want to remove them, this is the most efficient way to do so.  In the sample job, 100% of the points will correct; so there will be no uncorrected points.  Click on the EXIT icon to return to the Main Menu.

### 2.4.5  Combine Several Job Files into One

The following exercise demonstrates how to copy several GPS Jobs into a single file.

- Use JOBS/FILE/OPEN to open CMTTUTC.FTR and then also open CMTTUT.FTR.

- Use FILE/NEW to create a new file in which to store the combined Job.

- To display the three open Jobs at the same time, select WINDOW/TILE.

- When all three Jobs are displayed (the new Job, labeled "Untitled", will be blank), click anywhere on the CMTTUT window to activate that window and then click the ZOOM OUT icon twice.  Do the same for CMTTUTC.  Now you are ready to copy.  Select EDIT/COPY, or click on the COPY icon.  Position the mouse above and to the left of all of the points displayed in the CMTTUTC window.  Click and drag the mouse below and to the right of all of the points (i.e. draw a stretch box around all the data in CMTTUTC).  Now copy the data to “Untitled” by first clicking on the Untitled window and then selecting EDIT/PASTE or clicking on the PASTE icon.  Now perform the same operation for CMTTUT.

- Once you have completed appending CMTTUT to “Untitled”, select FILE/SAVE AS and save the new combined job as CMTTUT2.  You have now copied both jobs into a single file and saved the results.

- Double-click the CLOSE BOX in the upper-left corner of the CMTTUT window to close CMTTUT.  Close the CMTTUT2 window the same way.  Select WINDOW/TILE.  Select EDIT/ZOOM FIT.  CMTTUTD now fills the screen.

### 2.4.6  Measure Distance

PC-GPS can display the horizontal distance and slope distance between any two locations in your Job.

- Select MEASURE/DISTANCE or click the DISTANCE tool bar icon to display the following dialog box:

- If the DIRECT DISTANCE window is covering the points you wish to select, you can move it by grabbing the Direct Distance bar with the mouse and dragging it out of the way.

- Click on two Features and the Direct Distance window will show the distance and direction between them.

- Close the DIRECT DISTANCE box by clicking the OK button.

### 2.4.7  View the Details of a Feature

- Double-click on the TREE feature near the south-east corner of the map.  PC-GPS will always grab the Feature nearest to where you double-click.  The FEATURE UPDATE diaog box shows the Attributes and Values of the selected Feature.  It also gives access to other information about that Feature.  You may change the Feature name and the Feature ID.

- Click on the COORDINATES button.  The COORDINATES window shows detailed information about the GPS position.  Besides the coordinates, it shows whether the GPS position is 2D or 3D, whether it has been differentially corrected, the DOPS (quality of satellite geometry), the session time (# of GPS fixes used in determining the position), SV NUM (number of satellites tracked), and RESIDUAL (a measure of uncorrectable error).

Note:	If you change the coordinate values, you will actually move the point.  CANCEL both the COORDINATES and FEATURE UPDATE window.  Now double-click on one of the red (offset) point features.  Click on the OFFSET button.  Any offset you give a Feature while you are in the field will be displayed here.  You can edit the OFFSET parameters in this screen.

### 2.4.8  Creating and Editing a Feature List

You have seen in Section 2.2.2 how easy it is to attach names and descriptions to position data if you have a Feature List handy.  Typically, you would create a Feature List in PC-GPS and download the Feature List to your MC-GPS before you go into the field.  (In fact, you would normally use PC-GPS to create your MC-GPS Job files and Route files as well.)  Using PC-GPS, you can easily create any number of Feature Lists or open an existing Feature List and modify it as you wish.

Create a new Feature List as follows:

- Select LISTS/FEATURE LISTS (GPS/FEATURE LISTS for PC-GPS 3.2) from the Main Menu to display a File Open dialog box.

- Enter a file name such as “mylist.fbr”, then click OK.

- In the Feature List screen, click the NAME field and enter the name of a Feature, such as “HOUSE”.

- Click the TYPE field then click the little arrowhead.  Specify “POINT” as the type of the Feature.

- Click the ATTRIBUTE field and enter “LEVELS?”.

- Click the VALUE field and enter the Value “1”.  Press [Ú] to add a new blank record, then enter the Value “2”.

- When done, click OK to save the information and return to the MAIN MENU.

- Repeat steps 3 through 7 to add any additional Features that you wish to include in this Feature List.  The Features in the same Feature List may be of different types.

Later on, you may open MYLIST.FBR to edit an existing Feature or to incorporate additional Features into this Feature List.  Let’s say  you wish to add a new Feature named “ROAD”.

- Select LISTS/FEATURE LISTS from the MAIN MENU and click on “mylist.fbr” to open this file.

- Click on the bottom-most record.  Then press [Ú] to add a new blank record.

- Input the information about the new Feature you wish to add.  Enter “ROAD” as the Feature Name and select “LINE” as the Feature Type.  Enter the appropriate data logging mode and time parameters.  Input Attributes and Values as you please.

- If you wish, repeat steps 2 and 3 to add additional Features.

- Click OK to save the changes you have made.  If you do not wish to save the changes, click CANCEL.  You are now back at the MAIN MENU.

### 2.4.9  Transfer a Feature List from PC-GPS to MC-GPS

You will now transfer the Feature List you created in Section 2.4.8 to MC-GPS.  You may use the same procedure to transfer any other Feature List (such as PARK.FBR) to MC-GPS.

- Make the proper cable connections according to the instructions provided at the beginning of Section 7.

- From the MC-GPS MAIN MENU, press [F4](XFER).  Check the displayed FILE TRANSFER screen to make sure that the Baud Rate is 19200.  Then press [F4](RECV).  This puts MC-GPS into KERMIT server mode, ready to receive files from the PC-GPS utility.

- From the PC-GPS Main Menu bar, select the FILE TRANSFER pull-down menu.  Then select the CONFIGURE function to verify the PC communication settings.  The default communications port is COM1, and the default baud rate is 19200 baud.  Use the mouse to click the “OK” button.

- From the PC-GPS FILE TRANSFER pull-down menu, select the SEND function.  Click on “mylist.fbr” and then click  “OK”.

- When the file transfer is completed, PC-GPS will automatically break MC-GPS out of server mode.  On MC-GPS, press [ESC] to return to the MAIN MENU.

You are now ready to go out and collect some data using your new Feature List.  Follow the example in Section 2.2 to log a Line Feature (ROAD) and a couple of Nested Point Features (HOUSE).  If you have the appropriate base station data file available, you should be able to use the procedure in Section 2.4.2 to obtain differentially corrected position data.

Before doing any serious work, please read Section 3, Section 5 and any other relevant sections of this Manual.

# Section 3 - Setup

This section will guide you through the setup process for MC-GPS.  Setup parameters are important from the standpoint of MC-GPS performance and reliability of the collected data. The GPS receiver needs to be turned on for the Receiver and Time/Date setup functions to work properly.

When you select “Setup” from the MAIN MENU, the following menu appears:

Receiver	Send to the GPS receiver important information such as the initial position as well as DOP constraints and, optionally, an almanac.

15:37	SETUP

Receiver

Data Logging

Datum/Coord. System

Time/Date

RS-232 Input/Output

N3D3

Data Logging	Set data logging navigation parameters.

Datum/Coord. System	Specify the units in which measurements and information will be displayed.

Time/Date	View the UTC time/date used by the GPS receiver, or set the local time/date.

RS-232 Input/Output	Set up communications parameters for the specified port and external device.

The various setup screens initially contain the factory default parameters.  You may change them to meet the requirements of your applications.  Once you have saved your entries, they remain active until you change them again.  These parameters are stored in the MC-GPS system as long as its batteries are good.  Resetting the MC-GPS system will not alter the values of the setup parameters.

Many setup parameters may be changed by simply pressing [Ø] to select from available options.  Before pressing [ESC] to leave the data entry screen, remember to use the Function key labeled “SAVE” to save the data you have entered.

## 3.1  GPS Receiver Setup

When the MC-GPS is turned on, it immediately attempts to compute its position.  Without any initial information, MC-GPS can compute its position, but the process can take up to 24 minutes.  You can speed up this process by sending some information to the GPS receiver the first time you use it.  (Normally, this has been done at the factory prior to shipping).

Select “Receiver” from the SETUP menu screen.  The following screen appears:

Information displayed in this screen reflects the default setup parameters for the GPS receiver.  If the information in this screen is incorrect, you may change the data and then send the new setup to the GPS receiver by pressing [F4](SAVE).  The GPS receiver must be ON in order to save and send any changes.  The GPS receiver will automatically use the constellation of satellites that results in the best PDOP.

15:37	    RECEIVER

LAT:	44	33	47.22	N

LON:	123	15	47.43	W

MSL: 141.08      FT	EL.MASK:  5

PDOP    7  VDOP     6  HDOP    3

ALT.CTRL   AUTO   ALM  NO

SEL. CONSTELLATION   CANYON

N3D3

OFF

HELP

SAVE

LAT, LON, MSL	These data fields are for recording the Latitude  (LAT) and Longitude  (LON) in degrees, minutes, and seconds as well as the elevation above Mean Sea Level (MSL) of your beginning position.  An exact location is not necessary; even a close approximation (within 300 miles or so) can save much time in assisting the GPS receiver to track the correct satellites.

EL.MASK	Enter an elevation mask to mask out satellites that are low on the horizon.  To minimize multi-path error and to minimize delayed reception of satellite signals, you should not use signals from satellites that are too low on the horizon.  Satellites below the elevation in this field will not be tracked.  MC-GPS defaults to an elevation mask of 5 degrees.  For doing a carrier phase Job, specify an elevation mask of no less than 15 degrees.

PDOP, VDOP, HDOP	These three fields are for controlling the upper allowable limits of horizontal (HDOP), vertical (VDOP) and position (PDOP) dilution of precision.

(PDOP)2 = (VDOP)2 + (HDOP)2

The GPS receiver will not compute a position if the PDOP is greater than the value entered.  If the VDOP limit is exceeded, the GPS receiver may still compute a position based on the entry in the ALT. CTRL field.  MC-GPS defaults to 6 for VDOP and 7 for PDOP, respectively.  You may change these values if you wish.

ALT.CTRL	The altitude portion of the position that is displayed on the NAVIGATION, POSITION and STATUS screens can be determined by using one of three methods.  You may press [Ø] to select the appropriate method.

3D ONLY - In this setting, MC-GPS computes a 3D position only when 4 or more satellites are available.  Otherwise, it does not compute or store a position.  Set ALT.CTRL to “3D ONLY” when you require very accurate data.

AUTO - MC-GPS computes 3D positions when 4 or more satellites are available, and 2D positions when only 3 satellites are available.

MANUAL - MC-GPS computes all 3D positions based on the value you entered into the “MSL” field as long as 3 or more satellites are available.

Note:	If you will be relying upon a position containing a manually entered altitude, it is important to enter an accurate altitude.  If the altitude is not correct, it can introduce significant errors into the horizontal position displayed by MC-GPS.

SEND ALM?	Select “YES” if you wish to send the almanac in the RAM Disk (drive I:) of the MC-GPS to its GPS receiver when you press [F4](SAVE).  After the almanac is sent to the GPS receiver, this field will be automatically reset to the default “NO”.

An almanac is a file that tells the GPS receiver which satellites are visible at a given time, date, and location.  If the MC-GPS will not track satellites, it may be because the GPS receiver has lost its almanac.  A quick way to fix the problem is to send the backup almanac in the MC-GPS RAM Disk to the GPS receiver.

Normally, the MC-GPS automatically collects an almanac if it is tracking at least one satellite for at least 12.5 minutes continuously.

Function Key Assignments:

[F1](OFF/ON)	Turn power OFF or ON for the GPS receiver.  While the receiver is on, [F1] is labeled "OFF" and you may press [F1](OFF) to turn it off.  While the receiver is off, [F1] is labeled "ON" and you may press [F1](ON) to turn it on.

It is advisable to turn off the GPS receiver when you do not need to compute your position (e.g. when you are indoors or when you are just setting up the time/date or data logging parameters).  This will reduce the power drain on the battery.  When the GPS receiver is turned on, MC-GPS is constantly attempting to track satellites and consumes much more power than when it is turned off.

You can quickly tell if the GPS receiver is on or off by looking for the Tracking Status Indicator at the upper-right corner of the screen.  The Tracking Status Indicator will not be displayed when the GPS receiver is turned off.

When you press [F1](OFF), MC-GPS will prompt you for confirmation.  If you really want to turn off the GPS receiver, press [F5](YES) to confirm.

Note:	After you turn on the GPS receiver, it will need a few seconds to establish communication with the MC-GPS system.  During this time, a “WAIT” annunciator is displayed at the upper-right corner of the screen.  Wait until this annunciator goes away before pressing any keys.

[F4](SAVE)	Send all RECEIVER setup screen information to the GPS receiver for initialization.  If the “SEND ALM“ field shows “YES” then the almanac in the MC-GPS RAM Disk (drive I:) will also be sent to the GPS receiver.

If a backup almanac does not exist in the MC-GPS RAM Disk, then MC-GPS displays the message: “COULDN’T OPEN ALMANAC DATA FILE”.  In that case, you will need to collect an almanac or get one from your PC-GPS or another MC-GPS.

This function works only when the GPS receiver is turned on.  If the GPS receiver is off when you press [F4](SAVE), MC-GPS will issue a message to let you know.  In that case, press [ON] to display the STATUS screen and then press [F1](ON) to turn on the GPS receiver.  Now, press [F4](SAVE) to send the information to the GPS receiver.

[F5](HELP)	Display a helpful message about the currently highlighted data field.

## 3.2  Data Logging Setup

You can direct the way MC-GPS performs data collection by entering the pertinent information into the DATA LOGGING setup screen and the EDIT FEATURE LIST screen.  The parameters in the DATA LOGGING setup screen are global defaults that will be automatically supplied by MC-GPS when you create a new Feature.  You may override these by entering other values and then pressing [F4](SAVE).

You may also instruct your MC-GPS to beep each time it computes or stores a position, or to alert you when you are within a specified range of the next Waypoint while you are navigating.  Navigation involves moving from Waypoint to Waypoint along a pre-defined Route.

From the SETUP MENU screen, select "Data Logging", and the DATA LOGGING setup screen will be displayed.  If this is the first time you are using your MC-GPS, you will see the following default parameters:

Press [Ú] to display additional fields in the following screen:

15:37	 DATA LOGGING

LOG RAW DATA    YES

SETTLE TIME:  5

LOGGING MODE    DYNAMIC

STATIC SESSION:  20

DYNAMIC INTVL:  1

NAV BEEP    Y      FTR BEEP     Y

N3D3

HELP

SAVE

Field Descriptions:

15:37	DATA LOGGING

WARN RANGE:       0      FEET

WARN MSG    N  WARN BEEP    N

N3D3

HELP

SAVE

LOG RAW DATA	This field is a YES/NO toggle.  If you aren't sure how to set it, then make sure it is set at YES.  This means that MC-GPS will collect raw data for post-processing differential correction.  The only reason to set it to NO is if you want to save disk space because you are doing real-time DGPS (differential GPS) and do not need to perform post-processing differential correction.  If you select “NO”, MC-GPS will display the following message: “You will not be able to differentially correct your data in PC-GPS. Please confirm.”.  Press [F5](CONFIRM) or [F1](CANCEL), as appropriate.

SETTLE TIME	This field is for entering a time, in seconds, for allowing the receiver to "smooth" the data.  Once MC-GPS is tracking at least 3 satellites, you will not be able to store any data until this time has passed.  Please note that if you leave the FEATURING screen, the settle time process will start over again.  It will show the message: ”WAIT XX SECONDS...”.  In some cases where the most precise accuracy is not required and convenience is important, you may select a settle time figure as low as 1 second.  The default is 5 seconds.

LOGGING MODE	MC-GPS can use two different techniques for logging positions, called DYNAMIC mode and STATIC mode.

DYNAMIC mode assumes you are moving.  MC-GPS computes each coordinate position from a single position fix, and will not average coordinates.  This mode is appropriate for collecting data while walking or driving.

STATIC mode assumes you are stationary.  Each coordinate location is computed from an average of several position fixes.  The number of position fixes to be averaged is determined by your entry in the SESSION FOR STATIC POINT field described below.  If you move while recording points, the coordinate average will be affected accordingly.

When MC-GPS has collected the number of fixes entered in the SESSION FOR STATIC POINT field, you may then move to the next Feature you want to record.  When the Job is differentially corrected, the positions will be averaged to produce one coordinate point.

STATIC SESSION	This field is for setting the number of position fixes the STATIC mode will use to compute one coordinate position.  MC-GPS will automatically stop logging data when the number of recorded fixes reaches this number.  A minimum of 300 seconds are required for Carrier Phase data collection (Section 5.2.8).

DYNAMIC INTVL	This field is for entering the interval, in seconds, for logging data in DYNAMIC mode.  For example, entering 5 into this field means that the MC-GPS will record a coordinate position every 5 seconds while you are traveling along a Line Feature or around the perimeter of an Area.

NAV BEEP	Enter 'Y' to hear a Navigation Beep each second when the MC-GPS GPS is in N2D or N3D modes.  Note that the MC-GPS is not storing data at this point; it is only telling you that it is tracking a sufficient number of satellites to store a position.

FTR BEEP	Enter 'Y' to hear a lower pitched Featuring Beep each second when the MC-GPS is logging data in F2D or F3D modes.

WARN RANGE	Enter a distance value as the Warning Range.  The Warning Message you stored with each Waypoint and the warning beep will activate when the computed position is within Warning Range of the destination Waypoint in the NAVIGATION screen (Section 6).  The unit of measure (Feet, Meters, Miles, Kilometers) is determined by the UNITS field in the DATUM setup screen (Section 3.3).  For example, if UNITS is “Feet”, and you enter 300, then when the computed position is within 300 feet of the destination Waypoint in the NAVIGATION screen, a warning beep and the user-defined Warning Message will be issued.  Note that real time uncorrected GPS positions are accurate to only about 100 meters, so a Warning Range much smaller than this isn't practical.

WARN MSG	If you would like the Warning Message for the destination Waypoint to display in the NAVIGATION screen (Section 6) when you are within the Warning Range, then enter 'Y'.  Otherwise enter 'N'.  You may store a Warning Message with each Waypoint (Section 4.3).

WARN BEEP	If you would like MC-GPS to issue a Warning Beep when you are within Warning Range of the destination Waypoint in the NAVIGATION screen, enter 'Y'.  Otherwise enter 'N'.

Function Key Assignments:

[F4](SAVE)	Save the settings in this screen.

[F5](HELP)	Display a pertinent help message for the current data field.

## 3.3  Setup Datum/Coord. System

From the SETUP screen, select “Datum/Coord. System” to define the way information will be displayed. The DATUM screen initially contains the factory default values, as shown below.  You may use the [Ø] key to specify the appropriate datum, measurement units, north reference, time mode and coordinate system.  When done, press [F4](SAVE), then press [ESC] to return to the SETUP screen.  Press [ESC] again to return to the MAIN MENU.

Field Descriptions:

15:37	DATUM

DATUM      WGS 84

UNITS        FEET	HEIGHT      MSL

NORTH       TRUE

TIME MODE       UTC

COORD       LLA

N3D3

HELP

SAVE

DATUM	Select the datum used to display your position.  The GPS receiver collects data in the LLA coordinate system and the WGS-84 datum.  If you select another coordinate system/datum, MC-GPS will display positions in that coordinate system/datum.  MC-GPS offers about 50 datums, and you can also define your own.  When the cursor is on the DATUM field, you may use [Ø] or [×] to scroll through them.  Instead of scrolling through the datums to find the desired one, you may enter the first letter of the datum you would like, and you will be taken to the first datum that begins with that letter.   When you change the datum, it will result in different Latitude, Longitude and elevation values.

UNITS	Select the distance unit for all distance fields.  Use [Ø] or [×] to scroll through FEET, MILE, METER, and KM (kilometer).  The distance unit will also affect the velocity unit.  The following table applies:

DISTANCE UNITS		VELOCITY UNITS		
	METER				Meters per Second (M/S)  
	FEET				Feet per Second (FT/S)
	MILE				Miles per Hour (MPH)
	KM				Kilometers per Hour (KPH)

HEIGHT	Toggle between MSL (Mean Sea Level) and HAE (Height Above the Ellipsoid).

NORTH	Press [Ø] to toggle between TRUE and MAGNETIC north as the North reference.

DEG	Enter the degrees of declination (only displayed if MAGNETIC north is chosen).

TIME MODE	Press [Ø] to toggle between “LOCAL” and “UTC” time display modes.

COORD	Choose the coordinate system.  You may use [Ø] or [×] to toggle among these choices:

LLA	Latitude, Longitude, and Altitude
UTM	Universal Transverse Mercator
SPC	State Plane Coordinate

If you select UTM, you may enter the reference coordinates into the F.N. (False Northing) and F.E. (False Easting) fields.  The default False Northing is 0 and the default False Easting is 500000.

If you select SPC, you must set DATUM to either NAD 27 or NAD 83.  Otherwise, you will get an “ILLEGAL DATUM” warning message when you try to display SPC coordinates.

ZONE - This field is for entering the State Plane Coordinate System Zone number.  It is very important to enter the correct zone number for your location. Note that when a zone is selected, the STATE field will automatically be updated. The ZONE field is only displayed when SPC is selected as the coordinate system.

STATE - This field is for entering the state for the State Plane Coordinate System.  It is only displayed when SPC is selected as the coordinate system.  Use this field to find the SPC Zone number if you do not know it.

R - This field is for entering the Region in the state specified for the State Plane Coordinate System. It is only displayed when SPC is selected as the coordinate system.  Use this field to find the SPC Zone number if you do not know it.  MC-GPS will automatically determine the correct ZONE code for the SPC coordinate system.

Function Key Assignments:

[F4](SAVE)	Save the settings in this screen.

[F5](HELP)	Display a pertinent help message for the current data field.

## 3.4  Time/Date Setup

The date and time used by the GPS receiver is UTC (Universal Time Coordinated) Time.  It is the same as Greenwich Standard Time.  Following are examples of conversion to UTC time:

Local Time and Date				UTC Time and Date
	10:19 PM 	EST/USA  06-01-97            	03:19   UTC     06-02-97
	11:33 AM	PDT/USA  06-01-97            	18:33   UTC     06-01-97
	01:19 AM	WDT/AUS 06-01-97           	16:19   UTC     05-31-97

The correct UTC time/date, local time/date and local time offset have been entered for Pacific Standard Time.  If you are not in the Pacific Standard Time  zone, then you should enter the correct local time and local time offset. Note that the UTC time/date is the same anywhere in the world.

Select “Time/Date” from the SETUP screen, then enter the local time, local date and UTC time offset into the following screen.  The UTC time and date are displayed for your reference.  These fields may not be directly edited, but the data will change in accordance with your entries for the local time/date and UTC time offset.  After entering the time/date information, press [F4](SAVE) to save the data and return to the SETUP screen.

Field Descriptions:

15:37	TIME/DATE

UTC TIME:	00:37:42

UTC DATE:	02-08-96

LOCAL TIME:	16:37:42

LOCAL DATE:	02-07-96

UTC TIME OFFSET:	 -8

N3D3

HELP

SAVE

UTC TIME, DATE	This is the time and date that will be used by the GPS receiver to determine the current location of the satellites.

LOCAL TIME, DATE	The displayed time and date are those of the MC-GPS system.  You may edit it now.  Enter the correct time and date in the displayed format.  The MC-GPS system time/date will be set to agree with your entries.

UTC TIME OFFSET	The data in this field is used in conjunction with the local time and date to set the UTC time and date values.  Enter the difference between local time and UTC time.  Enter the number of hours to add to UTC time for local time.  For example, enter “-8” as the offset between UTC time and Pacific Standard Time.  For Pacific Daylight Saving Time, enter “-7” as the UTC time offset.

Function Key Assignments:

[F4](SAVE)	Save the settings in this screen.

[F5](HELP)	Display a helpful message for the current data field.

## 3.5  Input/Output Setup

Select “RS-232 Input/Output” from the SETUP menu screen to display the INPUT/OUTPUT screen.

Use the above screen to define input/output parameters for various external devices.  The appropriate data fields will be displayed for the selected external device.  If there are more data fields than can fit into one screen, press [Ú] to get to the second screen.

15:37	 INPUT/OUTPUT

DEVICE     RTCM_104

PORT        COM1: 	INPUT

BAUD        19200    	DATA      8

PARITY      NONE 	STOP      1

HS   DTR/CTS   RTCM_104   AUTO

AGE: 15     TIME ELAPSED: 1657

N3D3

HELP

SAVE

Note:	Power consumption is higher when a COM port is open for communication with an external device.

Field Descriptions:

DEVICE	Press [Ø] or [×] to select the device that will be used for data input or for receiving data output by MC-GPS.  Valid choices are: BARCODE(Section 3.5.1), NMEA 0183 (Section 3.5.2) and RTCM_104 (Section 3.5.3).  If you are not using RTCM_104 input or NMEA 0183 output, set this field to BARCODE.

PORT	Press [Ø] or [×] to specify the communications port to which the selected device will be connected.  Valid choices are: COM1: and COM2:.  MC-GPS will automatically display INPUT for all types of devices except NMEA 0183.  It will display OUTPUT if NMEA 0183 is selected as the external device.

If the selected device is RTCM_104, then MC-GPS will automatically select COM1: as the input port and reroute BARCODE input to COM2:.

BAUD	Press [Ø] or [×] to select the appropriate data transfer rate in bits per second.  Valid choices are:  300, 600, 1200, 2400, 4800, 9600, 19200 and 38400.

DATA BITS	Enter “7” or “8” as the number of data bits.

PARITY	Press [Ø] or [×] to select among NONE, EVEN and ODD.

STOP	Enter “1” or “2” as the number of stop bits.

HS	Press [Ø] or [×] to select XON/XOFF, DTR/CTS or BOTH as the handshake mode.

OUTPUT FREQ	This field will only be displayed for the NMEA 0183 device (Section 3.5.2).  Enter a number to specify how often the NMEA message will be output.  The minimum setting is 1 second.

RTCM_104	This field will only be displayed for the RTCM_104 device (Section 3.5.3).  It lets you specify how MC-GPS makes use of the  RTCM_104 message.  Available modes are: AUTO, YES and NO.

AGE	This field will only be displayed for the RTCM_104 device.  Enter the maximum acceptable age (in seconds) of the RTCM_104 message.

TIME ELAPSED	This field will only be displayed for the RTCM_104 device.  This field displays the time that has passed since MC-GPS last received an RTCM_104 message.

Function Key Assignments:

[F4](SAVE)	Save the information entered.

[F5](HELP)	Display a helpful message about the currently highlighted data field.

### 3.5.1  Bar Code Reader Setup

The bar code wand is a powerful data input device.  It can emulate MC-GPS keystrokes as well as enter information into data fields.  For example, you could create one bar code so that when you scan it, the following things would happen:

- Enter "STOP SIGN" into FEATURE NAME field.

- [ENTER] key is pressed.

- Enter "P" into the FEATURE TYPE field.

- Enter [F5](CNFRM).

- Enter "CONDITION" into attribute field.

- [ENTER] key is pressed.

- Enter "NEEDS REPAIR" into value field.

- [F4] (STORE) key is pressed to store the data and record the GPS position.

The result is that with one swipe of a bar code (taking less than a second), you have recorded a Feature called STOP SIGN with a GPS location and noted that it needs repair.  In a realistic situation, you would probably be scanning bar codes from a list of Feature bar codes, Attribute bar codes or Value bar codes.

CMT offers a bar code wand that is compatible with the MC-GPS and the default settings in this screen are correct.  If you wish to use another bar code reading device, please consult with CMT about the compatibility requirements.

In creating your bar codes to emulate the MC-GPS keystrokes, you will need to use EXTENDED CODE 39 (full ASCII conversion).  For example:

MC-GPS KEY TO EMULATE		CODE 39 BARCODE EQUIVALENT

F1						%U%F
		F2						%U%G
		F3						%U%H
		F4						%U%I
		F5						%U%J
		ENTER key					$M
		ESC key						%A

The procedure for using a bar code with your MC-GPS is as follows:

- Connect the bar code wand supplied by CMT to COM2 or COM1 of your MC-GPS Data Recorder.  If COM1 is being used for receiving RTCM_104 messages, then connect your bar code wand to COM2.

- In the INPUT/OUTPUT setup screen, set DEVICE to “BARCODE”, and set PORT to “COM2:” or “COM1:”, depending on which port is connected to the bar code wand.  The tip of the bar code wand should light up.

- Press [F4](SAVE) to save the input/output setup.  You may now use the bar code wand to enter information into the MC-GPS data fields.

### 3.5.2  NMEA 0183 Setup

The NMEA function enables MC-GPS to output its current calculated coordinates through a COM: port for use by an external device (such as a video camera).

It is important that the baud rate and other communication parameters match exactly that of the device accepting the NMEA 0183 message.  The MC-GPS simply outputs the NMEA message and has no way of knowing if the attached device is using the message.  Therefore, you should check that the receiving device is correctly using the NMEA message.

For the OUTPUT FREQ field, specify how often the NMEA message will be output.  The minimum setting is 1 second.  Please note that the output frequency depends upon the baud rate.  Please refer to the chart below:

NMEA Output Baud Rate	Maximum Output Frequency

19200	Once per second

4800-9600	Once per 3 seconds

1200-2400	Once per 5 seconds

300-600	Once per 7 seconds

The NMEA and RTCM_104 functions can be used in conjunction with each other for a powerful solution. For example, it is possible to receive an RTCM_104 message on COM1:, output an NMEA message with a corrected GPS position on COM2:, and collect data at the same time.

### 3.5.3  RTCM_104 Setup

MC-GPS can compute corrected coordinates by applying the GPS differential correction data received from a device that outputs RTCM_104 messages.  It is important that the baud rate and other communication parameters match exactly that of the device providing the correction (e.g. FM-pager, RF MODEM, beacon receiver).  You must specify to which COM port your RTCM_104 device will be connected.  If there is a conflict with another device (e.g. bar code reader), you will be notified with a pop-up message and required to change the settings in the INPUT/OUTPUT screen before saving.

The Tracking Status Indicator will tell you if the MC-GPS is receiving and using the RTCM_104 correction.  In N2D/F2D/N3D/F3D, the D will change to a C (e.g. N3C).

For the RTCM_104 field, select the appropriate mode in which you wish the MC-GPS to operate.

- AUTO - MC-GPS will use the RTCM_104 message whenever it is received.  If it is not received, the MC-GPS will continue to collect Feature data with uncorrected coordinates.  You will be notified during the transition between picking up and losing the RTCM_104 message, but the data collection process will not be interrupted.  If the LOG RAW DATA field in the DATA LOGGING setup screen (Section 3.2) is set to “YES”, then you will also be able to perform post-process differential correction.

- YES - MC-GPS will only allow you to collect data while an RTCM_104 message is being received.  If the RTCM_104 message is lost, then you will be warned and the MC-GPS will halt the data collection session until the RTCM_104 message is received again.

- NO - MC-GPS will not use any RTCM_104 message.  In this case, raw data will always be logged and you will not see the “LOG RAW DATA” field in the DATA LOGGING setup screen (Section 3.2).

For the AGE field, enter the maximum age (in seconds) of the RTCM_104 message you are willing to accept.  Please note that the older the RTCM_104 message, the more the potential for position error there is, especially when you are moving.  The maximum setting is 90 seconds.

# Section 4 - Database Management (LISTS)

MC-GPS provides functions for creating and managing databases for Job Information files, Feature Lists, and Routes.  You may also prepare these databases by using PC-GPS in the convenience of your office before heading out for a job.  In the field, simply select from the pre-defined data items when you are prompted for an entry for Job File, Feature List, or navigation Route.  By making good use of this functionality, you can significantly improve the efficiency of your GPS data collection job.

The LISTS management function lets you do the following:

- Define and maintain Job information, Feature List and Route files.

- Quickly view all the Job information, Feature Lists and Routes that are currently stored in the MC-GPS system.

- View and plot the GPS data collected for a specified Job.

- View summary information for a selected Job.

The GPS receiver does not need to be turned on for the database management functions to work properly.  To save battery power, you may want to turn off the GPS receiver while setting up the databases.

The contents of some of the data fields provided by the database management functions may be changed by simply pressing [Ø] to select from available options.  To save the data you have entered, please remember to press the Function key labeled “SAVE” before pressing [ESC] to leave the data entry screen.

When you press [F3](LISTS) from the MAIN MENU, the following screen will be displayed.  Select “Jobs”, “Feature Lists” or “Routes” to view or change the corresponding database.  Instructions for working with these databases are provided in the following sections.

4.1  Job File Maintenance

15:37		SELECT

SV PRN:  02  06  19  21  24  30

N3D3

Jobs

Feature lists

Routes

LISTS

OFF

XFER

VER

To manage your Job databases, press [F3](LISTS) from the MAIN MENU and then select “Jobs”.  A screen similar to the following will be displayed:

15:37	JOBS

<NEW>	G061412A

G061412B	G062014A

G062115A	G062115B

G062115C	MARSH3

N3D3

SUM

DEL

MARK

HELP

The JOBS screen lists all existing Job Names along with the selection “NEW”.  The following Function keys act on the currently highlighted Job.

Function Key Assignments:

[F1](MARK)	Tag the Job file for batch deletion.  A marked file will display with a chevron “»” in front of it.  To un-mark a marked file, press [F1](MARK) again over the highlighted file.

[F2](DEL)	Delete the currently highlighted Job after prompting for your confirmation.  If you have marked one or more Jobs, this function will delete only the marked Jobs after your confirmation.

WARNING:	Before deleting Job files, make sure you have selected the correct files to delete and that you really want to delete them.  Deleting a Job erases all information associated with that job, including any data collected for that Job.  When a Job is deleted, all the files with file name extensions “.DEF”, “.FTR” and “.RAW” are deleted.  However, the Feature Lists or Routes selected to be associated with the Job will remain intact.

[F4](SUM)	Display the JOB SUMMARY screen or BASE SUMMARY screen to provide summary information about the Job (Section 4.1.2).

[F5](HELP)	Display a helpful message about this screen.

### 4.1.1  Defining a Job and Editing Job Definition

To create a new Job, select “NEW” from the JOBS screen.  To view or edit an existing Job, select the name of the Job from the list on the screen.  In either case, a screen similar to the following will be displayed:

Field Descriptions:

15:37	JOB INFO

JOB FILE:   MARSH3	    ID    G

FEATURE LIST    WETLND

ROUTE    NONE

CREW:  GIS GROUP #32

DEPT:    MAPPING

DESC:   WETLAND SURVEY

N3D3

VIEW

SAVE

CANCEL

JOB FILE	This field displays the currently selected Job file name.  If you have selected “NEW” from the JOBS screen, MC-GPS will automatically supply a default 8-character Job file name in this data field.  The format is “IMMDDHHC”:

I	GPS receiver ID assigned to you by your system administrator

MM	Current UTC month

DD	Current UTC day

HH	Present UTC hour

C	Sequence letter ranging from A-Z.

For example, assume you have previously entered your ID code “G” into the “ID” field in the JOB DATA screen (Section 5.1).  If you are creating a “NEW” file on June 10 at the 15th hour UTC time, the first Job in that hour would be named G061015A.  The second Job in the same hour would be named G061015B, and so on.

You may change the default Job name as you wish.  If the name you enter already exists, MC-GPS will reject it and let you enter a different name.

ID	This field displays the ID code you have previously entered into the “ID” field in the JOB DATA screen (Section 5.1).

FEATURE LISTØ	Pressing [Ø] or [×] will scroll through the Feature Lists that you have defined and stored on MC-GPS.  You may select a Feature List to be associated with the Job so that the Feature identification tags it contains are handy whenever you need to use them for identifying collected data.  You may switch to a different Feature List at any time.  Select “NONE” if you do not wish to associate a Feature List with your Job.

ROUTEØ	A navigation Route does not have to be tied to any particular Job.  However, you may designate a Route to be associated with a Job so that you may access the navigation functions while doing the data collection Job.  Press [Ø] or [×] to scroll through the Routes that you have previously defined and stored on MC-GPS.  You may change the Route for the Job at any time.  Select “NONE” if you do not wish to associate a Route with your Job.

CREW	This field is for entering up to 31 characters for identifying the people or group responsible for collecting the data for this Job.  Entry into this field is optional.

DEPT	This field is for entering up to 31 characters for describing the department or emphasis of the project.  Entry into this field is optional.

DESC	You may enter up to 127 characters for a more detailed description of the Job.  Entry into this field is optional.

Function Key Assignments:

[F1](CANCEL)	Return to the JOBS screen.

[F2](VIEW)	Display or plot collected Feature data (Section 5.4).  This Function key is not available if the selected file is a base station Job file.

[F5](SAVE)	Save the Job definition and return to the JOBS screen.

### 4.1.2  Viewing Job Summary

If you press [F4](SUM) in the JOBS screen, MC-GPS will list the size of the files and the number of logged Features by type.  The screens for Job Summary are shown below.  To get to the second screen, press [Ú].

Field Descriptions:

15:37    JOB SUMMARY

JOB DEF SIZE:			499

RAW FILE SIZE:			134097

FEAT FILE SIZE:	64009

UTC START:		13:01:12	02-06-96

UTC STOP:		14:09:46	02-06-96

#POINTS:   3	#LINES:    2

#AREAS:   2	#COORDS:  199

N3D3

JOB DEF SIZE	Display the file size (in bytes) of the Job definition file (*.DEF).

RAW FILE SIZE	Display the amount (in bytes) of the raw data (*.RAW) logged for this file.

FEAT FILE SIZE	Display the file size (in bytes) of the Job Features file (*.FTR) generated for this file.

UTC/LOCAL START	Display the UTC or LOCAL start time of the raw data, depending on the TIME MODE setting in the DATUM setup screen (Section 3.3).  This is useful when matching with the base station file for differential correction.

UTC/LOCAL STOP	Display the UTC or LOCAL stop time of the raw data, depending on the TIME MODE setting in the DATUM setup screen (Section 3.3).  Please note that UTC START and UTC STOP only check the beginning and the end of the file.  If you had collected data a week ago and then finished it today, the raw data will appear to cover an entire week even though this is not the case.

#POINTS	Display the number of Point Features collected for this Job.

#LINES	Display the number of Line Features collected for this Job.

#AREAS	Display the number of Area Features collected for this Job.

#COORDS	Display the total number of coordinate records in all of the Feature records.  Each Point Feature has one coordinate record while each Line or Area Feature has more than one coordinate record.

If the Job file you selected is one that contains base station data (Section 5.5), the following screen will be displayed to show the time and location at which the base station data was collected.

Field Descriptions:

15:37 BASE SUMMARY

RAW FILE SIZE	:	134097

UTC START:		13:01:12	02-06-96

UTC STOP:		14:09:46	02-06-96

LATITUDE:         44	33     47.2204   N

LONGITUDE:    121	15     47.4256   W

ALTITUDE:       63.51 M  HAE

N3D3

RAW FILE SIZE:	Display the amount of the raw data (*.RAW) logged for this Base Job file.

UTC/LOCAL START	Display the UTC or LOCAL start time of the raw data, depending on the TIME MODE setting in the DATUM setup screen (Section 3.3).  This is useful when matching with the remote unit data file for differential correction.

UTC/LOCAL STOP	Display the UTC or LOCAL stop time of the raw data, depending on the TIME MODE setting in the DATUM setup screen (Section 3.3).

LATITUDE,	Display the location of the base station used for logging the raw data.

LONGITUDE,

ALTITUDE

## 4.2  Feature Lists, Attributes and Values

Features are essential identification tags for GPS position data.  A Feature may be one of three types: POINT, LINE, or AREA.  A POINT is a discrete item, such as a telephone pole.  A LINE is a series of connected points, such as a road.  An AREA is bounded by a line that ends back where it began, such as a land parcel.

When you define a Feature, you will specify the Feature Type as well as how data is to be collected for that Feature.  When that Feature is selected for data logging, MC-GPS will automatically use the associated data collection mode and time parameters to perform the job.

Besides identifying position data with a Feature type and name, you may also attach descriptive information to it.  The characteristics of a Feature are called Attributes.  For example, “HEIGHT” might be an attribute of interest for a POINT Feature called “TREE”.  The description of a Feature is complete when a Value is assigned to its Attributes.  A Feature may have multiple Attributes, each of which may have any number of Values.  Attributes and Values are not essential for GPS data logging.

With MC-GPS, you can group related Features into a Feature List.  Each Feature List with its associated Attributes and Values is maintained as a separate file.  MC-GPS gives you the capability of storing a large number of Feature Lists for use with different GPS Jobs.

When you set up a new Job, you may specify a Feature List to be associated with it (Section 4.1).  The Feature List serves as a "look-up" table that makes it very easy to attach descriptions to the GPS position information you collect.  The following example illustrates the relationship among the Features, Attributes, and Values in a typical Feature List.

FEATURE	TYPE		ATTRIBUTES		VALUES

Tree		Point		Species			Oak, Maple, Pine, Other
				Age			Old, Intermediate, Young
				Cut?			Yes, No

Road		Line		Lanes			1, 2, 4, Superhighway
				Paved?			Yes, No
				Travel			Heavy, Moderate, Light

Marsh	Area		Size			<1 ac., 1-5 ac., 5 ac.
				Beaver?			Yes, No

### 4.2.1  Maintaining Feature Lists

To manage your Feature List databases, press [F3](LISTS) from the MAIN MENU and then select “Feature lists”.  A screen similar to the following will be displayed:

Function Key Assignments:

15:37	FEATURE LISTS

<NEW>	WETLND

02069601	02069602

02129601	02269601

02269602	03129601

RIVERS

N3D3

HELP

DEL

MARK

[F1](MARK)	Tag Feature List files for batch deletion.  A marked file will display with a chevron mark (») in front of it.  To un-mark a marked file, press [F1](MARK) again over the highlighted file.

[F2](DEL)	Delete the marked Feature Lists after receiving your confirmation.

CAUTION:	Before deleting Feature Lists, make sure you have selected the correct files to delete and that you really want to delete them.  When a Feature List is deleted, all the Features in the Feature List and their associated Attributes and Values are also erased.

[F5](HELP)	Display a message about the Function keys available in this screen.

Creating a new Feature List

To create a new Feature List, highlight “NEW” in the FEATURE LIST screen and then press [ENTER].  A box will pop up to show the default Feature List name assigned by your MC-GPS.  The format for the default Feature List name is “MMDDYYnn” which is made up of the date and a sequence number.

You may enter your own Feature List file name.  Enter up to eight characters or digits as a name for the Feature List.  Blank spaces within the name and other illegal characters such as “.” and “/” will be rejected.  MC-GPS will reject duplicate Feature List file names.

When you have entered the name you want, press [ENTER].  The EDIT FT LIST screen will be displayed to permit you to add Features to this new Feature List.  The EDIT FT LIST screen is described in the following subsection.

Editing a Feature List

The EDIT FT LIST screen is displayed when you highlight an existing Feature List and press [ENTER], or after you select “NEW” and enter a new file name.

Deleting existing Feature Lists

The FEATURE LIST screen lists all Feature Lists currently stored in your MC-GPS.  You may mark one or several existing Feature Lists for deletion.  MC-GPS will ask you to confirm before it actually deletes the highlighted or marked files.

Caution:	Make sure you really wish to delete the highlighted or marked Feature List before confirming.  Once a Feature List is deleted, all the Feature tags and their associated Attributes and Values are erased and cannot be recovered.

Field Descriptions:

15:37	    EDIT FT LIST

FT LIST:  WETLND   	CHK    N

FEATURE                 TP  S/D    SESS

MARSH                       A     D      1265

GAME TRAIL              L      D       327

NESTING SITE           P     S         20

SAMPLING SITE         P    S     1800

N3D3

INS

SAVE

HELP[

DEL

ATTR

FT LIST	This is the name of the currently selected Feature List.  This field is for display only.

CHK	The program will initially skip this field because it is infrequently used.  You may press [Ù] to go back to this field.  Use [Ø] to toggle this field to "Y" if you would like MC-GPS to error-check Feature names.  This will force you to select a Feature from the Feature list for tagging GPS data.  While Feature error-checking is active, no Feature name may be manually entered unless it is identical to one of those in the Feature list.  The Feature type must also match that of the Feature in the Feature List.  However, you may assign a different data logging time parameter to the Feature you entered.  The default is “N” (no error-checking).

FEATURE	This column lists the names of the Features in the Feature List.  Up to 15 characters may be entered for each Feature name.

TP	This field defines the Feature type.  When the cursor is in this column, [F1], [F2] and [F3] are redefined as follows to permit you to quickly select one of the valid entries: 

[F1](PNT)	P - Point Feature
[F2](LINE)	L - Line Feature
[F3](AREA)	A - Area Feature

S/D	This field defines the mode of data logging to be performed for the associated Feature. The data logging mode defined for each Feature overrides that specified in the global default Data Logging settings (Section 3.2).  When the cursor is in this column, [F1], [F3], [F5] are redefined as follows to permit you to quickly select one of the valid entries:

[F1](STATIC)		S  - STATIC mode; default for Point Features
[F3](DYNAMIC)		D - DYNAMIC  mode
[F5](HELP)		Display a helpful message about this screen.

SESS	This field defines an important parameter for the data collection session.  For STATIC mode data logging, enter the number of position fixes to be recorded at each Point Feature.  For DYNAMIC mode data logging, specify the time interval between successive position fixes as you move along a Line or the perimeter of an Area Feature. The session parameter defined for each Feature overrides that specified in the global default Data Logging settings (Section 3.2).

Function Key Assignments:

[F1](ATTR)	Go to the EDIT ATTR screen (see 4.2.2) to define and edit Attributes for the highlighted Feature.

[F2](DEL)	Delete the highlighted Feature after receiving your confirmation.  MC-GPS will not permit you to delete the last remaining Feature in a Feature List.  If you really wish to remove it, simply delete the entire Feature List from the FEATURE LIST screen.

CAUTION:	Before deleting a Feature, make sure you have selected the correct item and that you really want to delete it.  All the Attributes and Values associated with that Feature will also be erased.

[F3](INS)	Insert a blank record at the current cursor position so you can add a Feature at that position.  This function is helpful for organizing and ordering the Features in the Feature List.

[F4](SAVE)	Save data and return to the FEATURE LIST screen.

[F5](HELP)	Display a message about the Function keys available in this screen.

To edit a Feature in a Feature List, use [Ù], [Ú], [×] or [Ø] to move to the field to be changed, then edit or type over the existing information.  Press [F4](SAVE) if you wish to save the changes in this screen.

To append a new Feature to the Feature List, use [Ú] to move the cursor to the last Feature in the Feature List, then press [Ú] one more time.  This adds a new blank line on which you may enter the definition for the new Feature.

To delete a Feature, simply highlight it and then press [F2](DEL).

If you wish, you may further characterize each Feature with Attributes and Values (Sections 4.2.2 and 4.2.3).

### 4.2.2  Defining Attributes

To add an Attribute to a Feature or to modify one of the existing Attributes of the Feature, highlight the Feature record in the EDIT FT LIST screen and then press [F1](ATTR).  A screen similar to the following will appear:

Field Descriptions:

15:37	    EDIT ATTR

FT: SAMPLING SITE   TYPE:	 P

ATTRIBUTE_NAME

NUMBER

DEPTH

SOIL TYPE

N3D3

INS

SAVE[

VAL

DEL

FT	The Feature name is displayed for reference only.

TYPE	The Feature type is displayed for reference only.

ATTRIBUTE_NAME	This column contains the names of the Attributes defined for the currently selected Feature.  Up to 15 characters may be entered for each Attribute name.

Function Key Assignments:

[F1](VAL)	Go to the EDIT VALUE screen (4.2.3) to define and edit Values for the highlighted Attribute.

[F2](DEL)	Delete the highlighted Attribute after obtaining your confirmation.

[F3](INS)	Insert a blank Attribute at the current cursor position.  This function is helpful for organizing and ordering the Attributes for the selected Feature.

[F5](SAVE)	Save the Attribute data and return to the EDIT FT LIST screen.

To edit a displayed Attribute, use [Ú]  or [Ù] to move to the Attribute name to be changed, then edit or type over the existing Attribute name.  Press [F5](SAVE) if you wish to save the changes in this screen.

To append a new Attribute to the displayed list of Attributes, use [Ú] to move the cursor to the last Attribute in the list, then press [Ú] one more time.  This adds a new blank line on which you may enter the name of the new Attribute.

To delete an Attribute, simply highlight it and then press [F2](DEL).

Press [F5](SAVE) if you wish to save the changes you made in this screen.

To assign one or more Values to an Attribute, highlight its name, then press [F1](VAL).  The procedure is discussed in Section 4.2.3.

### 4.2.3  Defining Values

To assign an additional Value to an Attribute or to modify one of the existing Values for an Attribute, highlight the Attribute in the EDIT ATTR screen and then press [F1](VAL).  A screen similar to the following will appear:

Field Descriptions:

15:37	EDIT VALUE

ATTR: WATER CONTENT

VALUE CK     N

VALUES

DRY

10

25

N3D3

SAVE

INS

DEL

VALCHK

ATTR	The Attribute name is displayed for reference only.

VALUE CK	The program will initially skip this field because it is infrequently used.  You may press [Ù] to go back to this field.  Toggle this field to "Y" if you would like MC-GPS to error-check the Values.  This forces you to select a Value from those that have been defined for the selected Attribute.  While Value error-checking is active, no Value may be manually entered unless it is identical to one of those already defined for the Attribute or within a range of Values specified for that Attribute.  The default is “N” (no error-checking).

VALUES	This column displays the Values assigned to the currently selected Attribute.  Up to 15 characters may be entered for each Value.

Function Key Assignments:

[F1](VALCHK)	Go to the VALUE CHECKING screen.

The VALUE CHECKING screen lets you define a default value (DEFAULT), a maximum value (MAX VALUE), and a minimum value (MIN VALUE).  The default, maximum and minimum Values you specify must be from the Values list for the selected Attribute.

15:37	VALUE CHECKING

DEFAULT

MAX VALUE

MIN VALUE

N3D3

CANCEL

CNFRM

The default Value will be automatically assigned to the associated Attribute when you tag a Feature with this Attribute.

The maximum and minimum Values are used to define a range of Values in addition to the discrete Values in the list.  You may assign any Value in that range to the associated Attribute when you use that Attribute to tag a Feature.  Maximum and minimum values are normally specified for error-checking numeric Values.

If you wish to limit the valid numeric Value entries to a specific range, enter just the maximum Value and the minimum Value (and no other Value) into the EDIT VALUE screen, then select the maximum value as MAX VALUE and the minimum value as MIN VALUE in the VALUE CHECKING screen.  With VALUE CK turned on, MC-GPS will not let you assign any Value that is not within the specified range to its associated Attribute.

Press [F5](CNFRM) to accept your entries in this screen or press [F1](CANCEL) to abandon the data.

[F2](DEL)	Delete the highlighted Value after receiving your confirmation.

[F3](INS)	Insert a Value at the current cursor position.  This function is helpful for organizing and ordering the Values in the list.

[F5](SAVE)	Save the Value data in this screen and return to the EDIT ATTR screen.

To edit a displayed Value, use [Ú]  or [Ù] to move to the Value to be changed, then edit or type over it.

To append a new Value to the displayed list of Values, use [Ú] to move the cursor to the last Value in the list, then press [Ú] one more time.  This adds a new blank line on which you may enter the new Value.

To delete a Value, simply highlight it and then press [F2](DEL).

Press [F5](SAVE) if you wish to save the changes you made in this screen.

## 4.3  Waypoints and Routes

A Waypoint is a single location with known coordinates.  It is described by a name and a description.  A Route is a sequence of Waypoints connected to create a path through which to navigate.  MC-GPS allows you to define a Route by collecting GPS position data at Waypoints along a path.  Later, MC-GPS can refer to that Route to help you or another user revisit each Waypoint.

Each Route is a file stored in MC-GPS.  A Route file contains one or more Waypoints in sequence.  The number of Routes you may create in MC-GPS is only limited by the amount of file storage memory space in the MC-GPS.

To manage your Routes database, press [F3](LISTS) from the MAIN MENU and then select “Routes”.  A screen similar to the following will be displayed.

Function Key Assignments:

15:37	ROUTES

<NEW>	PDXSEA

02069601	02069602

02069603	02209601

02209602	02269601

ROAD

N3D3

HELP

DEL

MARK

[F1](MARK)	Tag Route files for batch deletion.  A marked file will display with a chevron mark (») in front of it.  To un-mark a marked file, press [F1](MARK) again over the highlighted file.

[F2](DEL)	Delete the marked Routes after receiving your confirmation.

CAUTION:	Before deleting the Routes, make sure you have selected the correct files to delete and that you really want to delete them.  Deleting a Route means erasing all the Waypoints in the Route file.

[F5](HELP)	Display a message about the Function keys available in this screen.

Creating a new Route

To create a new Route, highlight  “NEW” in the ROUTES screen and then press [ENTER].  A box will pop up to show the default Route file name assigned by your MC-GPS.  The format for the default Route file name is “MMDDYYnn” which is made up of the date and a sequence number.

You may enter your own Route file name. Enter up to eight characters or digits as a name for the Route.  Blank spaces within the name and other illegal characters such as “.” and “/” will be rejected.  MC-GPS will reject duplicate Route file names.

When you have entered the name you want, press [ENTER].  The EDIT WAYPOINT screen will be displayed to permit you to add Waypoints to this new Route.  The EDIT WAYPOINT screen is described in Section 4.3.2.

Once the first Waypoint has been entered and saved, the new Route is established.  You will then be taken to the EDIT ROUTE screen to add more Waypoints or to use the other functions provided in that screen.  The EDIT ROUTE screen is described in Section 4.3.1.

Deleting Existing Routes

The ROUTES screen lists all Routes currently stored in your MC-GPS.  You may mark one or several existing Routes for deletion.  MC-GPS will ask you to confirm before it actually deletes the highlighted or marked files.

Caution:	Make sure you really wish to delete the highlighted or marked Routes before confirming.  Once a Route is deleted, it cannot be recovered.

### 4.3.1  Editing a Route

The EDIT ROUTE screen is displayed when you highlight an existing Route name in the ROUTES screen and press [ENTER].  It displays all the Waypoints currently recorded on the selected Route and lets you analyze, reorganize and modify them.  Additional information about these operations are provided in Sections 4.3.2, 4.3.3 and 4.3.4.

Field Descriptions:

15:37	EDIT ROUTE

ROUTE: PDXSEA	STOPS:   4

1	WPT1	M

2	WPT2	H

4	WPT4	H

N3D3

M

WPT3

3

PLOT

SAVE

DEL

DIST

INS

(ROUTE)	The Route name is displayed for your reference.

STOPS	This field displays the number of  Waypoints on the Route.

Also displayed on this screen is a list of all Waypoints in the selected Route file.  You may use [Ú] or [Ù] to scroll the Waypoint list up or down.  Each line of Waypoint information contains three columns:

Column 1	Waypoint number (starting from 1)
	Column 2	Waypoint name
	Column 3	This is an indicator of your estimate of the accuracy of position data for the 
			highlighted Waypoint .  Enter “H” for high accuracy, “M” for medium
			accuracy, and “L” for Low accuracy.  This is for your own reference only.

Function Key Assignments:

[F1](DIST)	Display the distance information between two Waypoints (Section 4.3.3).

[F2](DEL)	Delete the highlighted Waypoint after receiving your confirmation.  Exercise care as a deleted record cannot be recovered.

[F3](INS)	Insert a new Waypoint at the current position and move to the EDIT WAYPOINT screen.  Waypoint numbers for subsequent records will be re-assigned.

[F4](PLOT)	Plot current Route file to the screen (Section 4.3.4).

[F5](SAVE)	Save the information on this screen and return to the ROUTES screen.

Editing a Waypoint on the Route

To edit a Waypoint on the Route, use  [Ú] or [Ù] to highlight the Waypoint, then press [ENTER].  This will bring up the EDIT WAYPOINT screen to permit you to change the information for that Waypoint (Section 4.3.2).

Adding a Waypoint to the Route

You may press [F3](INS) to insert a Waypoint before the currently highlighted Waypoint.  To append a new Waypoint to the displayed list of Waypoints, use [Ú] to move the cursor to the last Waypoint in the list, then press [Ú] one more time.  This brings up the EDIT WAYPOINT screen to permit you to change the information for that Waypoint (Section 4.3.2).

### 4.3.2  Adding or Editing a Waypoint

The EDIT WAYPOINT screen is used for editing an existing Waypoint record or to append or insert a new Waypoint record.  It will also pop up when you have just created a new Route.

You may use this screen to record a Waypoint in two ways:

15:37	EDIT WAYPOINT

WPT: SAMPLE SITE #5	ACC    H

MSG:  CONTAMINATION PROBABLE

LAT:	44 45  49.4437	N

LON:	123  15  49.4987	W

MSL:  1999  M     WPT#:  12 / 33

N3D3

LOG

SAVE

HELP

NEXT

PREV

- If you already have the position data for the Waypoint, enter the identification information and the position data into the appropriate data fields.

- If the location has not yet been determined, press [F3](LOG) to collect the position data.  Before issuing this command, check to make sure that the GPS receiver is ON.

Press [F4](SAVE) to save the information entered on this screen.

Field Descriptions:

(WPT)	This is the Waypoint name.  You may enter up to 16 characters, including blanks.

ACC	Enter a relative accuracy estimate for the location information.  Available options are “H” (High), “M” (Medium),  or “L” (Low).  For example, you may want to enter 'H' for known surveyed locations,  'M' for differentially corrected Waypoint locations, and 'L' for uncorrected locations.  Accuracy levels entered are for user reference only and do not affect the operation of MC-GPS.

MSG	Enter an optional warning message.  When in navigation mode, MC-GPS may flash this message as a warning that you are approaching the Waypoint.  If WARN MSG in the DATA LOGGING setup screen (Section 3.2) is enabled and the distance from current position to next Waypoint is less than the WARN RANGE, the Warning Message will appear on the navigation screen (Section 6).

LAT	Enter the Latitude of the Waypoint in degrees, minutes, and seconds.  This field may not be edited if COORD is set to “UTM” or “SPC” in the DATUM setup screen (Section 3.3).

LON	Enter the Longitude of the Waypoint in degrees, minutes, and seconds.  This field may not be edited if COORD is set to “UTM” or “SPC” in the DATUM setup screen (Section 3.3).

MSL	Enter the MSL (Mean Sea Level) altitude of the current Waypoint.  This field may not be edited if COORD is set to “UTM” or “SPC” in the DATUM setup screen (Section 3.3).

WPT#	This field displays the current record number (Waypoint number) and the total number of Waypoint records in the Route (current/total).  This field may not be edited.

Function Key Assignments:

[F1](NEXT)	Move to the next Waypoint record.  If the current Waypoint is the last one in the file, this action appends a new blank record.

[F2](PREV)	Move to the previous Waypoint record.  If the current Waypoint is the first one, then there is no change.

[F3](LOG)	Take one position fix to log the current position of the GPS receiver as a Waypoint.  Please note that  Waypoints logged in this manner are not tagged with Attributes and Values.  In addition, unlike Point Features, you may not perform differential correction on Waypoint location data.  If you require Attribute and Value information to be saved with the position data to which you may later apply differential correction, then you should log the data by using the Data Collection function.

[F4](SAVE)	Save the information on this screen and go to the EDIT ROUTE screen.

[F5](HELP)	Display a message about the Function keys available in this screen.

### 4.3.3  Distance between Waypoints

To find the distance between two Waypoints, press [F1](DIST) from the EDIT ROUTE screen to bring up the following screen.

This screen is a utility for computing distance and direction between Waypoints.  Except for the “FR” and “TO” fields, nothing can be edited in this screen.  After you have specified any two Waypoints on the currently selected Route, press [F3](CAL).  MC-GPS will compute the distance and direction between the two Waypoints and display the information for your reference.

15:37	DISTANCE

FR  	4  :	SAMPLING SITE #5

TO  	6  :	BOREHOLE #3

UNITS      FEET    STOPS    2

DISTANCE:	793   FT

DIRECT DIST:	634   FT

BEARING:	354.000        DEG

N3D3

CAL

QUIT

NEXT

PREV

Field Descriptions:

FR	Enter the origin Waypoint record number.  The name of the Waypoint will be automatically displayed on its right side.

TO	Enter the target Waypoint record number.  The name of the Waypoint will be automatically displayed on its right side.

UNITS	Press [Ø] or [×] to set the distance unit as FEET, MILE, METER, or KM.  The default is determined by the UNITS field in the DATUM setup screen (Section 3.3).

STOPS	Display the number of Waypoints between the origin (FR) and target (TO) Waypoints.  If you are going from Waypoint #3 to Waypoint #10, then the number of stops would be “7”.

DISTANCE	Display the distance along the Route from the origin (FR) Waypoint to the target (TO) Waypoint.

DIRECT DIST	Display the direct (straight-line) distance between the origin and target Waypoints.

BEARING	Display the azimuth from the origin Waypoint to the target Waypoint in degrees.

Function Keys Assignments:

[F1](NEXT)	Move to the next Waypoint on the Route.  The Waypoint number in both the FR and TO fields will automatically advance by 1.

[F2](PREV)	Move to the previous Waypoint on the Route.  The Waypoint number in both the FR and TO fields will automatically decrease by 1.

[F3](CAL)	Request MC-GPS to re-compute the distances and bearings.  Press this key to see the new distances after you have changed the data in the FR or TO field.

[F5](QUIT)	Return to the EDIT ROUTE screen.

When you change the value in any field, you should press [F3](CAL) to instruct MC-GPS to re-compute the distances.  MC-GPS will display a message to inform you that it is busy computing.  The time it takes to compute the distance and bearing information depends on the number of STOPS between the origin and target Waypoints.  If you cannot wait and wish to abort the computation, simply press [ESC].

### 4.3.4  Plotting a Route

To see a plot of the Waypoints on the Route, press [F4](PLOT) from the EDIT ROUTES screen.  You may get something like this:

The direction of North is indicated at the top of the display.  The Waypoints are connected by a solid line that goes sequentially from the first Waypoint to the last.

NORTH

SCALE: 400 X 600 M

N3D3

OUT

IN

Press [F1](IN) or [F5](OUT) to zoom in on (enlarge) or zoom out of (reduce) the plot.  You may also manipulate the graphics in the ROUTE PLOT screen by using the following keys:

KEY		FUNCTION									
[Ù], [Ú],	Move your viewing position up or down.  (The graph moves in the opposite direction.) 
[×], [Ø]		Move your viewing position left or right.  (The graph moves in the opposite direction.)
[A]		Fit the entire Route onto the screen. 
[B]		Toggle a box mark as an indicator for the target Waypoint. 
[G]		Prompt for a Waypoint to designate as the current Waypoint. 
[H]		Toggle 4 different modes of status :

1.  Width X Length	Show the width and length of the range area of the screen.
		The units are dictated by the UNIT field in the DATUM
		setup screen.

2.  Latitude, Longitude	Display the position of the current Waypoint record.

3.  Waypoint name	Display the current Waypoint name.

4.  Waypoint record #	Display the current Waypoint record number.

[I]		Zoom in to the plot.
[O]		Zoom out from the plot.
[X]		Toggle a cross sign as a visual indicator of the current Waypoint.
[+]		Move the Waypoint indicator to the next Waypoint in the Route.
[–]		Move the Waypoint indicator to the previous Waypoint in the Route.

You may press [ESC] to exit the plot screen.

# Section 5 - GPS Data Collection

Once you have the MC-GPS properly set up, it is a simple matter to go out and collect GPS data.  The MC-GPS data collection routines let you efficiently perform the following tasks:

- Collect data for a selected Job in REMOTE mode or BASE mode.

- Collect location data for Point, Line (with or without Nested Points), and Area Features.  Feature data may be collected in the OFFSET mode.

- Add new Features and edit the characteristics of existing Features.

- Log raw data to be used for differential correction.

- Display the data collected for each Feature in text or graphic format.

- Navigate along the Route associated with the selected Job.

The contents of some data input fields may be changed by simply pressing [Ø] to select from available options.  To save the data you have entered, please remember to press the Function key labeled “SAVE” or “CNFRM” (confirm) before pressing [ESC] to leave the data entry screen.

## 5.1  Identifying a Job

To begin a data collection job, select “Collect Data” from the MAIN MENU.  If the GPS receiver is off, you will be prompted with the message: “GPS receiver is off.  Do you wish to turn it on?”  If you select  “NO”, you will be back at the MAIN MENU.  If you select “YES”, the following screen will be displayed:

This screen looks very much like the JOB INFORMATION screen discussed in Section 4.1.1.  It displays the general information about the currently selected Job file.  The “ID” is used for identifying the GPS data collected by a group of users within the same organization.  Your system administrator would be charged with the duty of assigning unique ID codes to the MC-GPS receivers.  Enter your receiver ID code into the “ID” field in this screen.

15:37	JOB DATA

JOB FILE    A061011A     ID    A

FEATURE LIST    	NONE

ROUTE      PDXSEA

CREW:	GIS GROUP #32

DEPT:	MAPPING

DESC:	WETLAND SURVEY

N3D3

GO

BASE

SAVE

HELP[

VIEW

Selecting a Job

- With the “Job File” field highlighted, you may use [Ø] or [×] to scroll through the list of Job files currently in your MC-GPS system.  One of the available selections in the list is “NEW”.  To select an existing Job, press [F1](GO) when its name is displayed in the JOB FILE field.  All data for a given Job should be collected within the same GPS week.  If you attempt to open a Job that was started during the previous GPS week, MC-GPS will issue a message to request you to start a new Job.  Each GPS week starts and ends at midnight Saturday UTC Time (Greenwich Mean Time).

- Select “NEW” and then press [F1](GO) if this is a brand new job you are starting.  MC-GPS will automatically name the new Job with a GPS receiver ID code, the current UTC month, UTC day, UTC hour and a sequence number (A to Z) as shown in the following example (ID code is “G”, month is “06”, day is “10”, hours is “15”, sequence number is “A”).

If you wish to rename the new job, please do so now.  Keep in mind that MC-GPS will not allow duplicate Job file names.  Optionally, enter information into the CREW, DEPT, and DESC fields to further identify the Job.  These fields are fully defined in Section 4.1.1.

15:37	JOB DATA

JOB FILE    NEW           ID    G

FEATURE LIST

ROUTE	PDXSEA

CREW:	GIS GROUP #32

DEPT:	MAPPING

DESC:	WETLAND SURVEY

N3D3

FILE NAME:  G061015A

GO

BASE

SAVE

HELP[

VIEW

In general, it is advisable to have the Jobs defined beforehand in the convenience of your office, so that all you have to do in the field is to select the appropriate one to work with.

Selecting a Feature List

- If you have used the database management functions discussed in Section 4.2 to define one or more Feature Lists, you can easily tag and characterize the GPS position data you are about to collect (Sections 5.2.1 and 5.2.2).  With the “Feature List” field highlighted, press [Ø] or [×] to scroll through the available Feature Lists.  You may change the Feature List for the Job at any time.

- If you do not wish to associate any default Feature List with your Job, select “NONE”.  In that case, you will need to define a temporary Feature tag for each data logging session (Section 5.2).

Selecting a Route

- If you designate a Route to be associated with a Job, you may access the navigation functions while doing the data collection Job.  With the “Route” field highlighted, press [Ø] or [×] to scroll through the Routes that you have previously defined (Section 4.3).  You may change the Route for the Job at any time.

- Select “NONE” if you do not wish to associate a Route with the Job.

The Function keys on the JOB DATA screen act on the currently displayed Job file.

Function Key Assignments:

[F1](GO)	Go to the data logging screens (Section 5.2).

[F2](VIEW)	Display and chart collected data (Section 5.4).

[F3](BASE)	Log data as a base station (Section 5.5).

[F4](SAVE)	Save the displayed information under the currently selected Job file name.

[F5](HELP)	Display a helpful message about the currently highlighted data field.

## 5.2  Collecting Feature Data (Featuring)

A data collection job involves recording location information for one or more Features.  This process is also referred to as  “Featuring”.  The general procedure is as follows:

- Select or define a Feature and optionally assign Attributes and Values to it.  You may change the definition of the selected Feature.

- Log position data in STATIC or DYNAMIC mode based on the definition of the selected Feature.

- Select or define another Feature and repeat the above process.

- If you wish, you may access a Line or Area Feature that you have worked with before and log additional coordinate points into it.

Point Features are logged in STATIC mode.  This means that you stay at the point of interest and let MC-GPS take a pre-defined number of position fixes from which it will compute the position of that location.

Line and Area Features are normally logged in DYNAMIC mode, in which each position fix is recorded as a separate coordinate point.  The coordinate points are recorded one after another at a preset interval as you walk along the line or around the area.

While logging a Line Feature, you may stop at certain points of interest and switch to STATIC mode to tag and record a Point Feature.  Such Point Features are called Nested Points.

You may also use MC-GPS to log data for a Line or Area Feature that is formed by a number of discrete points joined together in a given sequence by straight lines.  This is referred to as a Static Line/Area Feature.  When using this method, you will simply be logging Point Feature data at the connecting points and do not have to worry about walking a perfectly straight line between those points.

### 5.2.1  Selecting a Feature

With the appropriate Job file name displayed in the JOB DATA screen, press [F1](GO) to begin the Featuring process.

Defining a Temporary Feature Tag

If you have selected “NONE” for the FEATURE LIST field in the JOB DATA screen (Section 5.1), then a screen similar to the following will appear when you press [F1](GO).  Enter the pertinent information, then press [F5](CNFRM).  The Feature item you define this way is of a temporary nature, i.e. it can only be used to tag the current data logging session and will not be stored in the MC-GPS.

Field Descriptions:

15:37	FEATURE INFO

FEATURE NAME:   GAME TRAIL

FEATURE TYPE     POINT

FEATURE ID:  GAME TRA001

LOGGING MODE	 STATIC

INTVL/SESSION:	60   SEC

N3D3

CANCEL[

CNFRM

NAV

APP C

FEATURE NAME	Enter the name of the Feature tag that you are defining.  Spaces are permitted.

FEATURE TYPE	Press [Ø] to specify one of the following types of Feature: 
	P - Point Feature
	L - Line Feature
	A - Area Feature

FEATURE ID	MC-GPS will automatically assign a unique Feature ID based on the Feature name you input.  If the Feature Name has 8 or fewer characters (including spaces), MC-GPS will append a 3-digit sequence number to the Feature Name and use it as the default Feature ID.  If the Feature Name has more than 8 characters, MC-GPS will truncate the excess characters and append the 3-digit sequence number.  You may change the Feature ID; it may be up to 11 characters long.

LOGGING MODE	Press  [Ø] to specify one of the following data logging modes: 
	STATIC (for Point and Static Line/Area Features)
	DYNAMIC (for Line and Area Features)

INTVL/SESSION	For DYNAMIC mode data logging, enter the time interval (in seconds) between successive position fixes as you walk along a line or around an area.  For STATIC mode data logging, specify the session length, i.e. the number of position fixes to be used for recording each stationary Point.  The position fixes for STATIC mode are taken once every second.

Function Key Assignments:

[F1](CANCEL)	Abandon the information entered and return to the previous screen.

[F2](APP C)	Append additional coordinate points to the currently selected Line or Area Feature (Section 5.3).

[F4](NAV)	Navigate along the currently selected Route (Section 6).  This function is active only if a Route has been specified in the JOB DATA screen.

[F5](CNFRM)	Accept the data entered and then go to the FEATURES screen.  From the FEATURES screen, press [ENTER] to select the Feature you have just defined.  This will bring up the FEATURING screen.

Selecting a Feature from the Active Feature List

If you have selected a Feature List in the JOB DATA screen (Section 5.1), then a screen similar to the following will appear when you press [F1](GO):

This screen lists each of the available Feature names and the corresponding Feature type, data logging mode and session parameter.  Feature Type may be “P” for Point Feature, “L” for Line Feature or “A” for Area Feature.  The data logging mode may be “S” for STATIC or “D” for DYNAMIC.  The session parameter for STATIC mode is the session length in seconds.  The session parameter for DYNAMIC mode is the time interval (in seconds) between successive position fixes.

15:37	FEATURES

FEATURE	    TP   S/D  SESS

ROAD	   L     D        20

MARSH	   A     D         2

GAME TRAIL	   L     D         5

NESTING SITE	   P     S       20

SAMPLING SITE	   P     S   1800

N3D3

EDIT

NAV

HELP[

APP C

LISTS

- Use [Ú] or [Ù] to highlight the Feature Name that is appropriate for the data you are about to collect.

For example, if you wish to record the position of an eagle nest that you have spotted, select “NESTING SITE” from the list in the above sample screen.  This Feature is defined as a Point Feature to be logged in STATIC mode by taking 20 position fixes.  When you find another eagle nest, select “NESTING SITE” again as the Feature name.

- If you wish to define a new Feature to be used for the data you are about to collect, go to the end of the displayed list, then press [Ú] one more time.  The FEATURE INFO screen will be displayed for you to enter the information for the new Feature.  When done, press [F5](CNFRM) to go to the FEATURING screen.  The new Feature you just entered will not be saved into the Feature List.  Note that if Feature error-checking is active (Section 4.2), you may not enter any Feature name unless it is identical to one of those in the currently active Feature List.

From the FEATURES screen, press [ENTER] to select the highlighted Feature.  This will bring up the FEATURING screen for you to continue with the data collection process.  Please refer to Section 5.2.2 for instructions on assigning Attributes and Values to the selected Feature.  If you do not wish to tag the Feature with Attributes and Values, please skip to Section 5.2.3.

Following are some additional functions that you may perform from the FEATURES screen:

[F1](LISTS)	Go to the FEATURE LISTS screen displayed below to select, edit or create a Feature List for use in the current data logging session.  If you wish to use a Feature that is not in the default Feature List tied to this Job but is in another Feature List in your MC-GPS, press [F1](LISTS) from the FEATURES screen.  Then select the appropriate Feature List by highlighting its name and pressing [F1](SELECT) from the following FEATURE LISTS screen.  Then, when you highlight a Feature from that Feature List and press [ENTER], the selected Feature will be placed into the FEATURING screen, ready for you to assign Attributes and Values to it (Section 5.2.2).

From the above screen, you may also modify any existing Feature List.  To do so, highlight the Feature List, then press [ENTER] to go to the EDIT FT LIST screen (Section 4.2.1).

15:37	FEATURE LISTS

<NEW>	WETLND05

02069601	02069602

02069603	02069604

02069605	02069606

02069607

N3D3

HELP

SELECTMARK

[F2](APP C)	Append additional coordinate points to the currently selected Line or Area Feature (Section 5.3).

[F3](EDIT)	Go to the FEATURE INFO screen to modify the information for the currently selected Feature.  For example, if you wish to log certain nesting sites by taking 50 instead of 20 position fixes, you may highlight that Feature and then use [F3](EDIT) to change it.

Feature Name and Feature Type may not be edited.  Since Feature ID normally stays the same, the cursor will skip to the LOGGING MODE field.  If you wish to change the Feature ID, press [Ù] to go to the FEATURE ID field then edit the data.  When done, press [ESC] to switch back to normal mode, then press [F5](CNFRM) to save the data in this screen and proceed to the FEATURING screen.

[F4](NAV)	Navigate along the currently selected Route (Section 6).  This function is active only if a Route has been specified in the JOB DATA screen.

[F5](HELP)	Display a helpful message for the currently highlighted data field.

### 5.2.2  Assigning Attributes and Values

With a valid Feature highlighted in the FEATURES screen, pressing [ENTER] displays the following screen:

The type and name of the Feature is displayed on the second line of the screen.

15:37	FEATURING

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER

SOIL TYPE

N3D3

SURFACE

<1

>3

DEPTH

SELECT

Use the following procedure to assign Attributes and Values to the Feature.

- If a Feature List is active, the attributes associated with the selected Feature will be automatically displayed in the FEATURING screen.  A VALUE box is popped up for the first Attribute in the list.  The first line in the VALUE box is the Value input line.  Use [F3](EDIT/SELECT) to toggle the cursor between this Value input line and the list of Values below.  To assign an Attribute/Value pair to the Feature, use [Ú] or [Ù] to highlight the appropriate Value, then press [ENTER] to confirm and go to the pop-up VALUE box for the next Attribute.

If Value error-checking is not active, you may type in a Value that is not on the list.

Repeat the process until you have fully described the Feature.  If you wish to skip an Attribute, press [ESC] when its VALUE box pops up.

- If the active Feature is not from a Feature List, you may manually enter the Attributes and Values to further characterize it.  You could use a bar code reader to input the appropriate Attributes and Values.  After entering one pair of Attribute and Value pair, press [Ú] to enter additional Attributes and Values, if you wish.  These Attributes and Values will be tagged onto the data you are about to log, but will not be saved in the MC-GPS system.

- When you have completed the Attribute/Value assignment, a screen similar to the following will be displayed.  You may also enter additional Attributes in the field after pressing [Ú] to go to a blank entry line.  These additional Attributes and any Values you enter in the field will be used to tag the active Feature, but will not be saved into the Feature List.

Function Key Assignments:

15:37	FEATURING

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER	24

DEPTH	>3

SOIL TYPE	SAND

N3D3

DEPTH

DEL

OFFSET

STORE

HELP[

NEW FT

[F1](DEL)	Delete the highlighted Attribute-Value description tag for the currently active Feature.

[F2](NEW FT)	Go back to the FEATURES screen to select another Feature for which to log data.

[F3](OFFSET)	Log indirect position data in the OFFSET mode (Section 5.2.4).

[F4](STORE)	Start logging GPS data (Section 5.2.3).

[F5](HELP)	Display a helpful message for the currently highlighted data field.

### 5.2.3  Data Logging

When you have entered the appropriate Feature information, you are ready to log GPS data.

- MC-GPS will record data based on the Feature type and data logging parameters specified for the Feature.  The parameters specified in the DATA LOGGING setup screen will be used by default if you have not redefined them for the currently active Feature.

- On your part, please make sure the GPS receiver is turned on and MC-GPS is tracking at least 6 satellites or the Tracking Status Indicator shows a steady “N3DX” status.  Then follow the appropriate procedure provided below for the type of Feature you are working with.

Notice the change in the beeps and the Tracking Status Indicator in the upper-right corner when you begin and end featuring.  When you are not logging Feature data, the Tracking Status Indicator in the upper-right corner will display N2DX or N3DX.  Once you begin logging Feature data, the beeps will change noticeably and the Tracking Status Indicator will display either F2DX or F3DX.  See Section 1.5.1 for a description of the Tracking Status Indicator.

If for whatever reason, the MC-GPS is not tracking a sufficient number of satellites or the DOPs have exceeded the limits you defined, the SETTLE TIME message screen will be displayed until the MC-GPS can compute a position again.  The Tracking Status Indicator will show TRK when this occurs.  The Settle Time displayed on the screen will count down while GPS data smoothing occurs and then the program will proceed normally.

If your MC-GPS has Carrier Phase capabilities, please see Section 5.2.8 for additional information.

STATIC Point

In a collection of STATIC Points, each individual Point is a stand-alone Feature.  The large dots shown below may represent three buildings on a college campus.

Stay at the Feature location and do not move around while you are logging data for a Point Feature.

P1

P2

P3

- Press [F4](STORE) from the FEATURING screen to record the Feature information and log data for the Feature.  A screen titled “STATIC” will be displayed to show the progress of data logging.

Press [F3](POS) to look at the position data being collected.

15:37	STATIC

POINT FEATURE:  SAMPLING SITE

F3D3

STORING DATA NOW...

COLLECTED:59/60

0          25           50            75        100

PAUSE

POS

END

EDIT

- When 100% of the data has been collected for this Point Feature, MC-GPS will stop logging data and display the FEATURES screen.  If necessary, you may press [F5](END) to abort the data logging process at any time.

- Repeat the above process for any additional STATIC Points you wish to log.

- When done with the Job, turn off the GPS receiver.

Field Descriptions:

POINT FEATURE	This field identifies the Feature by type and name.

COLLECTED	This field shows the progress of data logging by displaying the number of fixes taken over the total number of fixes specified for the session.  A percent bar graph is also provided as a visual aid.

Function Key Assignments:

[F1](PAUSE)	Halt data logging and prompt you to continue or end data logging for the currently selected Feature.  Press [F1](CONT) to log additional data for the Feature.  Pressing [F5](END) will bring up another confirmation prompt screen.  From the second prompt screen, press [F5](YES) if you wish to terminate data logging for the currently active Feature and return to the FEATURING screen.  Press [F1](NO) to continue logging data for the currently active Feature.

Note:	The PAUSE function is not available on units with Carrier Phase capability.

[F2](EDIT)	This function allows you to enter or edit Attributes and Values while you are storing Feature data (Section 5.2.6).  Press [F5](EXIT) when you are done editing Attributes and Values.  MC-GPS will continue to store position data for as long as EDIT is active.

[F3](POS)	Display information about the currently computed position via the POSITION screen (Section 5.2.7).

[F5](END)	Display the message “Do you wish to end feature?”.  Press [F5](YES) if you wish to terminate data logging for the currently active Feature and return to the FEATURING screen.  Press [F1](NO) to continue logging data for the currently active Feature.

STATIC Line or Area

For the STATIC Line or Area Feature, you only log data at the individual Points that define the Feature.  These Points and the connecting lines automatically supplied by MC-GPS form the complete STATIC Line or Area Feature.

Go to the first Point of the STATIC Line or STATIC Area Feature.  Stay there and do not move around while you are logging data for this Point.

2	            3

1					           4

Press [F4](STORE) from the FEATURING screen to record the Feature information and log data for the Feature.  A screen titled STATIC will be displayed to show the progress of data logging.  This screen is identical to the STATIC screen described above for STATIC Point data logging, except that it shows in the COORD NO.  field the sequence number of the current Point in the STATIC Line/Area Feature, and, in addition, [F4] is labeled (STOP).  Pressing [F4](STOP) will abort the data logging process for the Point that is currently being logged.  This action does not end the data logging for the entire STATIC Line/Area Feature.

15:37	STATIC

AREA FEATURE:  AG BOUNDARY

COORD NO.:  12

F3D3

STORING DATA NOW...

COLLECTED:59/60

0          25           50            75        100

PAUSE

POS

STOP

END

EDIT

- When 100% of the data has been collected for this Point, MC-GPS will display a screen similar to the following:

Move to the next Point of the STATIC Line/Area Feature, then press [F4](STORE) to log data for that Point.  Or, move to a location that you wish to log as a Nested Point, then press [F2](PNT).  The NESTED POINT screen is described later in this section.  You may also use [F1](DEL) to delete an Attribute/Value tag for the STATIC Line or Area Feature.

15:37	STATIC

AREA FEATURE:  AG BOUNDARY

ATTRIBUTE	VALUE

FENCED	NO

CROP	GRASS SEED

RUNOFF	YES

N3D3

PNT

DEL

POS

STORE

END

- Repeat Steps 3 and 4 until you have finished logging data for all points of the STATIC Line/Area Feature.

- At any time, you may press [F3](POS) to look at the position data being collected.

- Press [F5](END) when done logging data for the STATIC Line/Area Feature.  You may also use this key to abort the data logging process at any other time.

- When done with the Job, turn off the GPS receiver.

DYNAMIC Line/Area

With a Line or Area Feature logged in DYNAMIC mode, MC-GPS collects data while you are moving along the Line Feature or the perimeter of the Area Feature.

Go to the beginning point of the Line or Area Feature for which you wish to collect data.

A1 - Begin and end here.

- Press [F4](STORE) from the FEATURING screen to record the Feature information and log data for the Feature.  A screen titled DYNAMIC will be displayed to show the progress of data logging.

Start moving (i.e. walking, driving) along the Line Feature or the perimeter of the Area Feature.

15:37	DYNAMIC

AREA:  SWAMP

F3D3

STORING DATA NOW...

COLLECTED:467

INTERVAL:3 SEC

PAUSE

PNT

POS

EDIT

END

- You may press [F3](POS) at any time to look at the position data being collected.  From the POSITION screen (Section 5.2.7), you may stop the current data logging session and then immediately resume data logging for a new Feature of the same type as the current Feature.

- Press [F5](END) when you have reached the ending point of the Line Feature or the beginning point of the Area Feature.  You may also use this key to abort the data logging process at any other time.

- When done with the Job, turn off the GPS receiver.

Field Descriptions:

LINE FEATURE	This field identifies the Feature by type and name.  It will display “AREA FEATURE” for an Area Feature.

COLLECTED	This field shows the progress of data logging by displaying the number of fixes taken so far.

INTERVAL	This is the time interval between successive fixes taken for the Line or Area Feature.

Function Key Assignments:

[F1](PAUSE)	Halt data logging and prompt you to continue or end data logging for the currently selected Feature.  Press [F1](CONT) to log additional data for the Feature.  Pressing [F5](END) will bring up another confirmation prompt screen.  From the second prompt screen, press [F5](YES) if you wish to terminate data logging for the currently active Feature and return to the FEATURING screen.  Press [F1](NO) to continue logging data for the currently active Feature.

Note:	The PAUSE function is not available on units with Carrier Phase capability.  If you need to halt the data logging process for a Line/Area Feature, you may press [F2](PNT) to go to the NESTED POINT screen.  MC-GPS will immediately stop logging data.  When you are ready to continue logging data for the Line/Area Feature, press [ESC] to return to the DYNAMIC screen.  MC-GPS will automatically resume data collection.

[F2](PNT)	If applicable, use this function to record a Point that is not on the Line/Area Feature.  The NESTED POINT screen is described later in this section.

[F3](POS)	Go to the POSITION screen (Section 5.2.7) to view information about the currently computed position, or to stop the current data logging session and then immediately resume data logging for a new Feature with the same characteristics as the current Feature.

[F4](EDIT)	This function allows you to edit Attributes and Values while you are storing Feature data (Section 5.2.6).  Press [F5](EXIT) when you are done editing Attributes and Values.  MC-GPS will continue to store position data for as long as EDIT is active.

[F5](END)	Display the message “Do you wish to end feature?” Press [F5](YES) if you wish to terminate data logging for the currently active Feature and return to the FEATURING screen.  Press [F1](NO) to continue logging data for the currently active Feature.

Nested Point

MC-GPS allows you to interrupt a Line or Area Feature (without terminating it) to record an associated Point Feature.  This kind of Point Feature is called a Nested Point Feature.  For example, assume you are mapping a particular river, but wanted to also inventory all of the  boat ramps along the banks.  You can put your Line Feature (the river) on hold by pressing [F2](PNT), record a Nested Point Feature and then resume logging the same river Feature.

The procedure for logging a Nested Point is similar to that for logging a regular Point Feature.  It is summarized below:

N2

L1

N1

- From the DYNAMIC screen, press [F2](PNT).  A list of available Point Features will be displayed.

Highlight the appropriate Point Feature or create a new one.  You may use [F1](LIST) to modify the Feature List that is currently associated with your data collection Job.  If you press [F3](EDIT) or press [Ú] at the end of the list to enter a new Point Feature, you may enter Feature information into the following screen.  When done, press [F5](CNFRM).

15:37	NESTED POINT

FEATURE NAME:  BURROW

FEATURE TYPE:    POINT

FEATURE ID:    00000021

LOGGING MODE:  STATIC

INTVL/SESSION:	60

15:37	NESTED POINT

FEATURE	  	TP  S/D SESS

NESTING SITE	          P     S    20

SAMPLING SITE	          P     S 1800

BOREHOLE	          P     S    15

BURROW	          P     S    60

N3D3

CNFRM

NAV

CANCEL[

N3D3

EDIT

NAV

HELP[

LIST

- With the desired Feature highlighted, pressing [ENTER] will bring up a screen similar to the following:

Select the appropriate Value from the VALUE box, then press [ENTER].  Repeat until you have assigned all the Attribute/Value pairs with which you wish to identify this Nested point Feature, then press [ESC].  A screen similar to the following will be displayed.  The Function key definitions are identical to those for the FEATURING screen.

15:37	NESTED POINT

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER

DEPTH

N3D3

SOIL TYPE

SAND

SEDIMENT

SELECTDIT

Stay at the Point Feature location and do not move around while you are logging data.  The logging mode of a Nested Point Feature always defaults to STATIC mode even though the current Line/Area Feature is being recorded in DYNAMIC mode.  You may use [F3](OFFSET) to offset a Nested Point (Section 5.2.4).  If you wish to abandon the data you have just logged and start anew with another Feature, press [F2](NEW FT).

15:37	NESTED POINT

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

SOIL TYPE	SAND

DEPTH	>3

N3D3

DEPTH

DEL

OFFSET

STORE

HELP

NEW FT

- Press [F4](STORE) to record the Feature information and log data for the Nested Point Feature.  A screen will pop up to show the progress of data logging in terms of the number of position fixes taken over the total number of fixes specified for logging the Nested Point.  A percent bar graph is also provided as a visual aid.

Press [F3](POS) to look at the position data being collected (Section 5.2.7).  If necessary, you may press [F5](END) to abort the data logging process for the Line/Area Feature.

15:37	NESTED POINT

POINT FEAT:  ANIMAL BURROW

F3D3

STORING DATA NOW...

COLLECTED:11/60

0           25           50           75        100

PAUSE

POS

END

EDIT

- When 100% of the data has been collected for this Nested Point Feature, MC-GPS will stop logging data and give you a choice to log another Nested Point or to continue logging the original Line or Area Feature.  If you choose to continue logging the Line or Area Feature, press [F1](CONT) and start moving again along that Feature.  If you need to log another Nested Point, press [F2](PNT).  This will bring up the list of available Point Features as shown in step 1 above.  Select an appropriate Point Feature and repeat the above steps to log the Nested Point.

Note:	Nested Points are recorded with Feature Type “N”.

15:37	NESTED POINT

POINT FEATURE:  ANIMAL BURROW

F3D3

CONTINUE LINE/AREA FEATURE OR ENTER ANOTHER NESTED POINT.

CONT

PNT

### 5.2.4  Data Logging in OFFSET Mode

For a Feature that is not directly accessible, you can log data for one that is accessible and record the OFFSET information along with the Feature data.  The position data for the Feature of interest can be computed by applying the OFFSET parameters to the position data actually logged.  MC-GPS provides for two types of OFFSET computations:

- Distance OFFSET - This method is useful when you can log data at a fixed slope distance, azimuth and slope from the target Feature.  For example, to determine the location of the tip of an antenna tower, you would log data for a Point Feature on the ground after entering the Distance OFFSET parameters into the MC-GPS.  And, to map an underwater area within a lake, you would log data while boating around an area of the same shape and size and let MC-GPS figure out the coordinates for the actual Feature based on its depth under the water.

- Left/Right OFFSET - This method is useful when you can easily log data to one side of the target Feature.  For example, to log Area Feature data for a marsh in the woods, you would move parallel to the perimeter of the marsh at a fixed distance.  Specify a “Right” OFFSET if the target Feature is to your right while you are moving parallel to its perimeter; specify a “Left” OFFSET if the target Feature is to your left while you are moving parallel to its perimeter.

To access the OFFSET function, press [F3](OFFSET) from the FEATURING screen or the NESTED POINT screen.  If you have selected a Point Feature, the following screen will be displayed for the Distance OFFSET mode.  You may press [F3](LR OFS) to switch to the Left/Right OFFSET mode.

If you have selected a Line or Area Feature, the following screen will be displayed for the Left/Right OFFSET mode.  You may press [F3](D OFS) to switch to the Distance OFFSET mode.

15:37	FEATURING

N3D3

OFFSET

SL. DIST:               20.00  M

AZIMUTH:           220.15  °

SLOPE:                     5.30  °

LR OFS

RESET

HELP[

STORE

OFFSET

SL. DIST	20 M

DIRECTION	LEFT

15:37	FEATURING

N3D3

D OFS

RESET

HELP[

STOREAVE

Field Descriptions:

Left/Right OFFSET:

SL. DIST	Enter the horizontal distance from where you are collecting GPS data to location of the target Feature.  For a Line or Area Feature, this distance should be on a line perpendicular to the direction you will be moving.

DIRECTION	Select “LEFT” or “RIGHT” to indicate whether the target Feature is to the left or right, respectively, of where you are collecting GPS data.

Distance OFFSET:

SL. DIST	Enter the slope distance from where you are collecting GPS data to the corresponding point on the target Feature.

AZIMUTH	Enter the azimuth of the target Feature.

SLOPE	Enter the slope to the target Feature.

Function Key Assignments:

[F2](RESET)	Set the OFFSET parameters to “0”.

[F3](LR OFS)	Toggle between “LR OFS”  and “D OFS”.  For “LR OFS” (Left/Right Offset), enter the horizontal distance and then select “LEFT” or “RIGHT” for the DIRECTION field.  For “D OFS” (Distance Offset), input data for SL. DIST, AZIMUTH and SLOPE.

[F4](STORE)	Save the information in this screen and return to the FEATURING screen or the NESTED POINT screen.  Press [F4](STORE) again to begin data logging.

[F5](HELP)	Display a helpful message about this screen.

Following is the general procedure for logging a Feature in OFFSET mode:

- From the FEATURING screen or NESTED POINT screen, press [F3](OFFSET) to bring up the OFFSET screen.

- Use [F3](LR OFS/D OFS) to select Left/Right Offset or Distance Offset, respectively.

- Enter the appropriate OFFSET parameters via the keyboard.

- Press [F4](STORE) to save the OFFSET parameters and return to the FEATURING screen or the NESTED POINT screen.

- From the FEATURING screen, press [F4](STORE) to begin data logging.  When logging a Point Feature or Nested Point Feature, stay at the appropriate location.  When logging a Line or Area Feature, move parallel to the target Feature.

- Press [F5](END) when you have finished logging the Line Feature or Area Feature.  For a Point Feature, the data logging will automatically end when the specified number of position fixes have been collected.

- When done with the Job, turn off the GPS receiver.

### 5.2.5  Data Logging in Traverse Mode (Only available in CMT Field Ver. 3.4)

While the OFFSET function is typically used when you have difficulty collecting one Feature at one location because of the lack of a good GPS signal, the TRAV function is provided as a tool to allow you to continue mapping several Features and locations when a good GPS signal is not available.

There are two modes in which you may operate the TRAV function. One mode is based on traditional traversing methods where you traverse from point-to-point.  For example, if you are mapping the boundary of a timber sale, and one section of the sale has dense canopy and is also on a severe slope, you may have trouble getting good GPS reception.  The point-to-point traverse mode lets you perform a standard “chain and compass” traverse through the difficult section and then resume your GPS mapping when conditions are better.

POINT-TO-POINT

GPS1

T1

T4

T3

T2

In this example, a GPS position is recorded at GPS1.  This position must be a Point Feature.  If you are logging a Line or Area Feature when you start to lose your GPS reception, end the Feature and store a Point Feature.  It is recommended the Point Feature at GPS1 be used to indicate the start of your traverse, so that there are no questions later.

Note:	If you are logging a Line or Area Feature when the GPS reception becomes poor, you should stop your Line or Area and then log a Point Feature before begining your traverse.

You would then traverse to T1 (first traverse point), entering the distance, slope and azimuth from GPS1 to T1.  As you move from point-to-point, you will enter the distance, slope and azimuth to each new point.  Each traverse coordinate Point is determined from the distance, slope, azimuth from the last point. Traverse measurements can be determined by using a compass and tape measure, or by using a laser rangefinder.

In the example above, lines are drawn to indicate the traverse path.  In CMT-Field 3.4, each of the traverse positions will be an independent Point Feature.  You will need to Join these points together in PC-GPS at a later time.

The other mode is to simply occupy a single GPS point and then record measurements to the multiple traverse points from that position (radial traverse).  For example, if you are mapping water meter locations in an urban environment with many tall buildings, it may be difficult to get good GPS reception between buildings.  The radial traverse mode lets you occupy one position in the clear and then enter the distance, slope and azimuth to each meter which can be seen from that GPS position.

T2

FROM ONE POINT

T3

GPS1

T1

T4

In this example, you would record a Point Feature at GPS1. Then you would map each point T1-T4 by entering the azimuth, slope and distance from GPS1 to each point.  Traverse measurements can be determined using a compass and tape measure, or by using a laser rangefinder.

In the example above, lines are drawn to indicate the traverse path.  In CMT-Field 3.4, each of the traverse positions is an independent Point Feature.

Regardless of which method you use for traversing, the traverse must be started with a GPS position. If you attempt to store a traverse point before storing a GPS position, the following error will be displayed:

Please store a reference point first!

If you have stored several GPS positions in your job already, the traverse will begin from the last GPS coordinate point that was stored.

Note:	If you are logging a Line or Area Feature when the GPS reception becomes poor, you should stop your Line or Area and then log a Point Feature before entering traverse data.

#### 5.2.5.1 Traversing using the Point-to-Point Method

After you have stored a reference Point Feature, you begin a new Feature by pressing [F2](NEW FT), just like you are going to store another GPS Feature. However, instead of selecting a Feature like you normally would (by highlighting the Feature name and pressing ENTER), you should highlight the Feature name and then press F3-EDIT to move to the FEATURE INFO screen.

Press F3-TRAV to indicate that the Feature will be a Traverse Feature and move to the TRAVERSE screen.

15:37	FEATURE INFO

FEATURE NAME: TRAIL HEAD

FEATURE TYPE     POINT

FEATURE ID:  TRAIL HE001

LOGGING MODE	 STATIC

INTVL/SESSION:	60   SEC

N3D3

CANCEL[

CNFRM

NAV

APP C

TRAV

In the TRAVERSE screen,  you will enter the Value information for the Attributes, just as if you were logging a GPS Feature.

15:37	TRAVERSE

TRAV FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER

SOIL TYPE

N3D3

SURFACE

<1

>3

DEPTH

EDIT

Once you have entered the Value information, you will see that the [F4] key is labeled “MEAS” (instead of “STORE”, like it is in GPS mode).  Press F4-MEAS to display the MEASUREMENT screen.

15:37	TRAVERSE

TRAV FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER	1

2.5

SOIL TYPE	LOAM

N3D3

DEPTH

HELP

MEAS

NEW

DEL

Function Key Assignments:

TRAVERSE screen:

[F1](DEL)	Delete an attribute/value record.

[F2](NEW)	Select a new feature name for the next traverse point.

[F4](MEAS)	Go to the MEASUREMENT screen to enter measurement data.

[F5](HELP)	Display a helpful message about this screen.

The FROM LAST field is used for you to define if the measurement will be recorded from the last GPS point or from the last TRAVerse point. Using the point-to-point method, the first traverse point is always FROM LAST GPS. Then, all of the subsequent points are FROM LAST TRAV.

15:37   MEASUREMENT

N3D3

FROM LAST

SL. DIST:               20.00  M

AZIMUTH:           220.15  °

SLOPE:                    5.30  °

GPS

EDIT

DONE

After completing the FROM LAST field, you are prompted for the SL.DIST (slope distance), AZIMUTH and SLOPE.

Field Descriptions:

MEASUREMENT screen:

FROM LAST	Use the left or right cursor key to select either GPS or TRAV.  When  GPS is selected, the new Point Feature will be created by using the last GPS point as a reference position.  When TRAV is selected, the new Point Feature  will be created using the last traverse point as a reference position.

SL. DIST	Enter the slope distance from the reference position to the location of the new Traverse Point.

AZIMUTH	Enter the direction from the reference position to the new Traverse Point (0 to 359 degrees).

SLOPE	Enter the slope from the reference position to the new Traverse Point in degrees (-90 to 90 degrees).

After entering all of the MEASUREMENT screen data, press the F5-DONE key to store the traverse point. The following message will flash briefly on the display.

Point Stored!

You will then be returned to the TRAVERSE screen and prompted to enter Value information for the next Traverse Point.

Note:	If you are traversing in the Point to Point mode, you must now change the FROM LAST field from GPS to TRAV because you are traversing from the last traverse point, not from the last GPS point.

If you wish to use a different Feature Name, press ESC from the TRAVERSE screen and then press F2-NEW to select a new Feature.  You can then follow the same sequence as described above to enter more Traverse Points.

#### 5.2.5.2 Traversing using the From One Point Method

The operation of this method is almost exactly the same as the Point-to-Point method. There are two primary differences. The first is that all traverse points are taken using the same reference position, so you will not be moving from the GPS point after recording the GPS location. The second difference is that you will always select GPS in the FROM LAST field of the MEASUREMENT screen.

Note:	The FROM LAST field always defaults to the previous entry. For example, if you last selected TRAV in the FROM LAST field, TRAV will automatically be selected the next time you enter the MEASUREMENT screen.

### 5.2.6  Editing Attributes and Values

While actively logging data for a Feature, you may edit the Attributes and Values to be associated with it.  Pressing the [F2](EDIT) from the STATIC screen or NESTED POINT screen, or pressing [F4](EDIT) from the DYNAMIC screen will display a screen similar to the following:

You may rename any Attribute by editing or typing over the existing name.

15:37	FEATURING

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER	24

DEPTH	>3

SOIL TYPE	SAND

N3D3

DEPTH

DEL

EXIT

To remove an Attribute from the selected Feature, highlight that Attribute and then press [F1](DEL).

To add an Attribute, go to the end of the list and then press [Ú].  Key in the name of the new Attribute, then press [ENTER].   You may enter an appropriate Value for the new Attribute.

To change the Value assigned to an Attribute, highlight that Attribute and then press [ENTER] to pop up the VALUE box.  An example is shown below.

Input a Value or press [Ú] or [F3](SELECT) to go to the Values list and select a pre-defined Value.  While the cursor is in the Value list, [F3] is labeled “EDIT”.  You may use this key to quickly go back to the Value edit line.

15:37	FEATURING

POINT FEATURE:  SAMPLING SITE

ATTRIBUTE	VALUE

NUMBER

DEPTH

SOIL TYPE

WATER CONTENT

N3D3

SURFACE

<1

DEPTH

SELECT

When done changing the Attribute/Value assignments, press [ESC] to return to the previous screen.  Keep in mind that MC-GPS will continue to store position data as long as EDIT is active.

### 5.2.7  Displaying Position Data

While you are actively logging data, you may press [F3](POS) from the STATIC, DYNAMIC or NESTED POINT screen to display the POSITION screen to view your position as it is updated once a second.  All of the information shown on this screen is for display only.  The coordinates displayed are those most recently computed by the GPS receiver.

Field Descriptions:

15:37	POSITION

LAT: 	44  33  47.4567	N

LON:	123  15  33.9876	W

MSL:  321.9    HAE:  156.7   FT

RAW DATA LOG:   ON

COG: 252o    VEL:   4.5 FT/S

N3D3

UTM

PLOT

REPEAT

LAT	Display the Latitude of the last computed position.

LON	Display the Longitude of the last computed position.

Note:	You may press [F4](UTM) to change the coordinate system displayed.  When the display is LAT and LON, pressing [F4](UTM) will display NORTHing and EASTing coordinates of UTM.  MC-GPS will also display the correct UTM zone for the location.  Pressing [F4](SPC) will display State Plane Coordinates (SPC), Zone, and Region.

If you get the error message “ILLEGAL DATUM”, it means that MC-GPS is unable to convert the current LLA or UTM coordinates to SPC.  Usually, this happens when the GPS receiver has not begun computing positions and MC-GPS is attempting to convert zero coordinates to SPC.  To remedy this, simply wait until the GPS receiver begins computing positions, and then press [F4](SPC).

MSL	Display the elevation of the current position in Height above Mean Sea Level.  The unit shown depends on the selection made for the UNITS field in the DATUM setup screen (Section 3.3).

HAE	Display the elevation of the current position in Height Above the Ellipsoid.  The unit shown depends on the selection made for the UNITS field in the DATUM setup screen (Section 3.3).

RAW DATA LOG	This field informs you whether raw data is being logged.  “ON” means you have set  LOG RAW DATA to “YES” in the DATA LOGGING setup screen and raw data is being recorded along with the computed positions.  “OFF” means raw data is not recorded and you will not be able to differentially correct the position data by using PC-GPS.

COG	Display your current Course Over Ground (in azimuth).  COG is updated once a second.

VEL	Display your current speed (velocity).  VEL is updated once a second.  The unit of velocity is dependent on the selection made for the UNITS field in the DATUM setup screen (Section 3.3).

Function Key Assignments:

[F2](REPEAT)	From the POSITION screen, you may end this data logging session and immediately start logging data for a new Feature that has inherited the name, type, Attributes and Values of the currently active Feature.  This is useful in the situation when you are traveling at high speed and need to change the value for an attribute without losing track of the Line along which you are moving.  You would press [F3](POS), then press [F2](REPEAT) to call up the FEATURING screen.  You may change the Attribute and Value (e.g. speed) for the Line Feature, if you wish.  Press [F4](STORE) to log the new Line Feature.

Note:	The REPEAT function will not record offset information with the Feature data.  You may manually enter the offset information later when you review the collected data (Section 5.4.3).

[F4](UTM)	Toggle among the coordinate systems available.  LLA and UTM are available for all datums.  If you have selected NAD 27 or NAD 83 in the DATUM field in the DATUM setup screen (Section 3.3), then SPC may also be displayed.  Otherwise, the message “ILLEGAL DATUM” will flash and no SPC coordinates will be displayed.

[F5](PLOT)	Display a real-time scatter chart of the computed position data.

Plotting Position Data on Screen

When you press [F5](PLOT) from the POSITION screen, you will see a screen that looks similar to the following:

The current position is displayed as a “+” on the screen.  The “North” indicator is located at the top of the display.  If the position moves off the screen, MC-GPS will adjust the display to show the current position.

NORTH

X

SCALE: 400 X 600 M

N3D3

OUT

IN

The average position is displayed with an 'X'.  Note that if you leave the POSITION PLOT screen momentarily and then return, the 'X' may not appear to be the average.  This is because the POSITION PLOT screen displays only the positions computed since the screen was accessed, whereas the average position takes into account all of the positions computed since the Job was opened.

If there is a Route associated with the Job, it will also be displayed in the POSITION PLOT screen.  If the Route you selected to be associated with the Job is at a great distance from your current location, then you may not be able to see the scatter of your position fixes because it is negligible compared to the distance between the Route and your current location.

You may zoom in or zoom out by using [F1](IN) or [F5](OUT), respectively.  You can also manipulate the plot in other ways by pressing these keys:

KEY		ACTION
[A]		Adjust the display to show all graphic data.
[I]		Zoom in to the plot.
[O]	 	Zoom out from the plot.
[H]		Toggle between two kinds of status:

1.  Width X Length	Show the width and length of the range area of the screen.
		The units are dictated by the UNITS field in the SETUP
		DISPLAY screen.

2.  Present Coordinates	Show the present location in degrees of Latitude and
		Longitude to 5 decimal places.

[C]		Clear the screen and re-display the plot screen to show only those position fixes
		recorded after the screen has been refreshed.

Press [ESC] to return to the POSITION screen.

### 5.2.8  Carrier Phase GPS Data Logging

This section relates to the -CM Carrier Phase option.  Please skip this section if you are only interested in logging C/A code GPS data.

The Carrier Phase option of MC-GPS consists of two parts: the software on the hand-held called -CM option, and a differential correction module for PC-GPS called CMTCM.  PC-GPS treats CMTCM corrected Jobs differently with respect to the quality indicator.

To collect Carrier Phase data, the GPS receiver needs to continually track the same 4 satellites.  If this condition is not met, the field software will automatically stop logging data.  (It is recommended that you use the PC-GPS Mission Planning function to find a time-block where 5 or 6 satellites are always visible.)  In addition, the PDOP should be less than 4..

MC-GPS can be used to yield data in three different accuracy ranges:  The first is the standard 1-5 meter accuracy range, the second is the sub-meter accuracy range, and the third is the 10-30 cm accuracy range.  There are several requirements for achieving each accuracy range, the key factor being the time spent at each data logging location.  (Please see the chart presented below.)

There are two methods for logging Carrier Phase GPS data: STATIC method and KINEMATIC method.  Please use the appropriate method for each data collection Job to ensure quality of the collected data and accuracy of the computed coordinates.

- To accurately determine the coordinates of an unknown point, use the STATIC method, i.e. log data in STATIC mode for the Point Feature with a session time of 15 to 90 minutes (1800 seconds recommended), depending on certain conditions and the desired degree of accuracy.  MC-GPS must stay still and unobstructed during this entire period of time.  Wait at least 2 minutes after MC-GPS is in N3D mode before storing any data.

- To log a collection of Point, Line or Area Features, use the KINEMATIC method, i.e. first initialize your position by logging that Point in STATIC mode for at least 900 seconds (1800 seconds recommended).  Once the initial Point has been established, you should only need to log any additional Point Features for 5 to 15 minutes each.  You may also move around and collect potentially very accurate GPS locations rapidly.  While you are moving around it may not be possible for your MC-GPS to maintain continuous satellite lock.  When you lose satellite lock for even an instant, that Job is over.  Therefore, it is important to make sure that the signals from the satellites are not blocked by your body or any other objects.  In case you lose the satellites, go back to the last known good point and continue the Job from there.

The primary consideration in determining the coordinates of an unknown point are baseline and occupation time.  Baseline is the distance between you and the base station that is collecting data that you will later use for differential correction.  The following chart is based on PDOP = 4 and number of tracked SVs = 6.

Baseline	Occupation time	  Probable Accuracy

<10 km	5-15 minutes	  Sub-meter
<10 km	15-30 minutes	  Some cm-accuracy FIXED points; mostly sub-meter FLOATING
<10 km	30-45 minutes	  Mostly FIXED cm-accuracy; some sub-meter FLOATING
<10 km	45-90 minutes	  Nearly all points FIXED with cm-accuracy
10-20 km	<15 minutes	  Unpredictable
10-20 km	15-30 minutes	  Some sub-meter FLOATING
10-20 km	30-45 minutes	  Mostly sub-meter FLOATING; some FIXED cm-accuracy
10-20 km	45-90 minutes	  Mostly FIXED with cm-accuracy

To initialize on a known point for a kinematic survey you should occupy the initial point for the appropriate time duration shown in the following chart:

Baseline	Occupation time	  Probable Accuracy

<10 km	10 minutes	  Many cm-accuracy FIXED; some sub-meter FLOATING
<10 km	15 minutes	  Mostly FIXED cm-accuracy; occasional sub-meter FLOATING
10-20 km	5-15 minutes	  Some cm-accuracy FIXED; mostly sub-meter FLOATING
10-20 km	15-30 minutes	  Many cm-accuracy FIXED; some sub-meter FLOATING
10-20 km	30-45 minutes	  Mostly FIXED with cm-accuracy

Carrier Phase techniques with the MC-GPS are most effective for a baseline of less than 20 km.

The Job must be processed with CMTCM.  CMTCM is very similar to CMT-DIFFERENTIAL, so only the differences will be explained here.

Differences between MC-GPS and Carrier Phase Data Collection

For Carrier Phase data collection, in the DATA LOGGING setup screen (Section 3.2), you will want to set a large value for STATIC SESSION (300 seconds is the minimum useful time for a Carrier Phase session).  In the RECEIVER setup screen (Section 3.1), set EL. MASK to 15 degrees.  The FEATURING screen is significantly different when you store a coordinate point.  When you press [F4](STORE), MC-GPS will ask for ANTENNA HEIGHT for the first Feature of each Job.  When prompted, enter the appropriate antenna height.

Kinematic GPS requires continuous satellite lock on at least 4 satellites after initialization on a STATIC point.  MC-GPS will show a message "WARNING! LESS THAN 4 SVS TRACKED! CONTINUE?" the first second in which it tracks fewer than 4 satellites.  This message tells you that any kinematic GPS survey you may be doing is over.

Differences with PC-GPS

The Carrier Phase differential program is called CMTCM Differential.  It is very similar to CMT Differential.  The initial GPS DIFFERENTIAL screen is identical.

The SETUP button will take you to the RELATIVE PROCESSING SETUP screen.  This screen is similar in form and function to the DIFFERENTIAL SETUP screen, with some important differences.  This screen allows for the ANTENNA HEIGHT of the BASE STATION, in case you use a mobile base station (which will often be the case with Carrier Phase work).  There is no Altitude Fixing in Carrier Phase differential correction.

You have the option to start your first STATIC Point at a known location.  This greatly decreases the required occupation time to start a kinematic survey.  You must enter the coordinates of the known point (in WGS-84 LLA, with altitude in meters HAE) below the field "Check here and enter coordinates below if first static point is known".   If you check the box then your first Static Point feature will be at exactly those coordinates after correction.

CMTCM Differential tags corrected points with a QUALITY indicator between 1 and 15.  To see the quality of a point, go to JOB/OPEN and open the CMTCM-corrected Job.  Double-click on the feature.  Click on COORDINATES.  The lower the Quality number, the greater probability that the coordinates are accurate.  If the QUALITY is from 1 to 3 then the point is fixed.  This means there is a high probability it is correct to within 10-30 cm.  If QUALITY is from 4 to 10, the point may be fixed or floating.  This means that it is likely accurate to within a meter, but probably not within 30 cm.  If QUALITY is worse than 10, it is likely not within a meter.

Sample Job using the -CM Option

Make sure your base station is within 20 kilometers and is logging data.  Carrier Phase GPS requires that the receiver track the same 4 satellites for the entire time.  Use Mission Planning to find a time-block where the same 5 or 6 satellites are always visible.  Start a new Job and set SESSION FOR STATIC POINT to 2400 seconds (for a centimeter-accuracy fix).  Set MC-GPS where it will be unobstructed the entire 2400 seconds.  Wait at least 2 minutes after MC-GPS is in N3D mode before storing any data.  Mark the location.  You may find a tripod and external antenna very useful for Carrier Phase GPS since you must keep the antenna motionless for 30 minutes or more.  Store a Point Feature in STATIC mode.  Wait while MC-GPS stores.  Start a new Feature (make it a Line Feature in DYNAMIC mode) and store it.  Keep the GPS antenna raised above your body, and slowly walk around.  If you see the message "WARNING! LESS THAN 4 SVS TRACKED! CONTINUE?", then you have tracked less than 4 SVs and this survey is over.

Differentially correct this Job with your base station data.  Process it with CMTCM Differential to get sub-meter accuracy.  You still have the option to correct the Job with CMT Differential for 1 to 5 meter accuracy.

Do another Job.  This time, set SESSION FOR STATIC POINT to 600.  Set the MC-GPS exactly where it was last time.  Store the first Line Feature in STATIC mode.  Store a Line Feature in DYNAMIC mode and include several Nested Points.

This time, when you correct with CMTCM Differential, check the box marked  "Check here and enter coordinates below if first static point is known", and enter the coordinates for that point.  You should find that this Job is about as accurate as the first one.  This is the advantage of starting from a known point.

Carrier Phase GPS will often require extensive mission planning.  Use the AVAILABILITY chart in MISSION PLANNING to find a long-enough time period for your entire Job.  Use the DOPS chart to make certain the DOPS are optimal for the entire Job.

## 5.3  Appending Coordinate Points

If you wish to add GPS data to an existing Feature, press [F2](APP C) to display the APPEND COORDS screen.  You will find this function very useful when you need to log additional data for a road you mapped yesterday, or when you need to continue working with the same Feature after replacing the MC-GPS battery pack.

You will see a list of the names of all the Line or Area Features collected so far for the currently selected Job.  Also displayed for your reference are the Feature type, data collection mode and Feature ID.

15:37	APPEND COORDS

FEATURE	  T   M	FEATURE ID

SHORELINE	L   S   SHORELIN001

MARSH	A   D   MARSH001

GAME TRAIL	L   D   GAME TRA001

SHORELINE	L   S   SHORELIN002

POND	A   D   POND001

N3D3

SELECT

CANCEL

AN

Function Key Assignments:

[F1](CANCEL)	Return to the FEATURES screen.

[F5](SELECT)	Select a Line or Area Feature for which to log additional data.

After you have selected the appropriate Feature, the procedure for logging additional data is identical to that described in Section 5.2.3.  If you choose to also change the Attribute/Value assignments to the selected Feature by using the procedure described in Section 5.2.2, the entire Feature will be tagged with the new descriptions.

Note:	If you forgot to append data to an existing Feature and, by mistake, logged the additional data to a new Feature, you can join the two Features later by using PC-GPS.  The two Features to be joined must be of the same type.

## 5.4  Viewing and Editing Collected Feature Data

After you have collected Feature data for a Job, you may view the results and edit the Features, Attributes and Values, if you wish.  To do so, press [F2](VIEW) from the JOB DATA screen.  A screen similar to the following will pop up:

Select “Collected data” to view the data and edit the Features in the currently selected Job (Section 5.4.1 - 5.4.3).

15:37	        VIEW

JOB FILE

FEA

ROU

CRE

DEPT:

DESC:

N3D3

MARSH3

Collected Data

Plot

HELP[

- Select “Plot” to view a screen plot of the collected data (Section 5.4.4).

### 5.4.1  Viewing and Editing Feature/Attribute/Value

From the VIEW (JOB) screen, select “Collected data”.  A screen similar to the following will be displayed:

All of the Feature records collected so far for the Job are listed on this screen.  If they don't all fit on the screen, use [Ú] and [Ù] to scroll the list.

15:37	COLLECTED DATA

FEATURE	 TP	  S/D	SESS

MARSH	A	D	1

GAME TRAIL	L	D	1

NESTING SITE	P	S	20

SAMPLING SITE	P	S	20

SHORELINE	L	D	1

BOREHOLE	P	S	60

N3D3

ATTR

OFS

LOC

SAVE[

DEL

Features are listed by name, type, data logging mode and the session length (for STATIC mode) or interval between position fixes (for DYNAMIC mode).  To edit a Feature name, simply type over it.  The other parameters may not be changed.

Function Key Assignments:

[F1](ATTR)	List all Attributes and Values of the highlighted Feature for review and editing.

[F2](DEL)	Delete the highlighted Feature.

[F3](OFS)	View/edit the OFFSET parameters for the selected Feature (Section 5.4.3).

[F4](LOC)	Display the coordinate location data for the highlighted Feature (Section 5.4.2).

[F5](SAVE)	Save any changes made on this screen.

To edit Attributes and Values for a Feature, first highlight that Feature in the above screen, then press [F1](ATTR).  All the Attributes and Values assigned to the selected Feature will be displayed in a screen similar to the following:

The Feature type and Feature name are shown near the top of the display.  You may view, change or delete the Attribute and Value data in this screen.  No new Attributes or Values may be added.

15:37	COLLECTED DATA

POINT FEATURE:  BOREHOLE

ATTRIBUTE	VALUE

NUMBER	23

DEPTH	<1

SOIL TYPE	SAND

WATER CONTENT	15

N3D3

NEXT

DEL

LOC

SAVE[

PREV

Field Name Descriptions:

ATTRIBUTE	A list of the Attributes tagged to this Feature

VALUE	The Value assigned to the Attribute

Function Key Assignments:

[F1](NEXT)	Display the Attributes and Values of the next Feature.  If the current Feature is the last one in the Feature List, MC-GPS will beep and flash the warning message “LAST RECORD” at the bottom line of the display.

[F2](PREV)	Display the Attributes and Values of the previous Feature.  If the current Feature is the first one in the Feature List file, MC-GPS will beep and flash the warning message “FIRST RECORD”.

[F3](DEL)	Deletes the highlighted Attribute and Value after warning and confirmation.

[F4](LOC)	Show the data for the first coordinate location in the Feature record (Section 5.4.2).

[F5](SAVE)	Save any changes made and return to the previous screen.

### 5.4.2  Viewing Location Data for a Feature

To view the location of the coordinate points you have logged for a Feature, highlight the desired Feature displayed in the COLLECTED DATA screen, then press [F4](LOC).  A screen similar to the following will be displayed for the first coordinate location in the Feature.  Press [F2](NEXT) or [F1](PREV) to display other coordinate locations in the Feature.  The Feature name and Feature type are displayed on the first FEATURE LOC screen.  “P” stands for Point Feature, “L” for Line Feature, “A” for Area Feature, and “N” for Nested Point Feature.  To see the second screen, press[Ú].  Data in these screens may not be edited.

15:37	FEATURE LOC

FT_NAME:  BOREHOLE        P:   I

UTC TIME:	13  :  42  :  57

UTC DATE:	01  /  24  /  96

LAT:	24   46   31.47	N

LON:	123   15   25.43	W

MSL:    112.34	FEET

N3D3

NEXT

UTM

QUIT

PREV

Field Descriptions:

15:37	FEATURE LOC

POINT STATUS 3D  DIFF CORR  N

HDOP:1.6  VDOP: 1.3  PDOP: 2.1

N3D3

NEXT

UTM

QUIT

PREV

UTC TIME/DATE	These fields show the UTC time and date tagged to the collected data.  If the TIME MODE in the DATUM setup screen (Section 3.3) is set to “LOCAL”, these fields will be titled “LOC TIME/DATE”.

LAT, LON	The coordinate location is displayed as Latitude and Longitude by default.  You may press [F4](UTM) to switch the display to UTM NORTHing and EASTing.  MC-GPS will also display the correct UTM zone for the location.  Pressing [F4](SPC) will display State Plane Coordinates, Zone, and Region.

MSL	The elevation is displayed as height above Mean Sea Level.  The unit shown depends on the selection made for the UNITS field in the DATUM setup screen (Section 3.3).

POINT STATUS	This field indicates whether the location data was logged in 2-dimensional or 3-dimensional mode.

DIFF CORR	This field indicates whether the displayed point has been differentially corrected.

HDOP, VDOP, PDOP	These fields show the values for Horizontal Dilution of Precision, Vertical Dilution of Precision and Position Dilution of Precision as an indication of the quality of the data collected for the displayed location.

Function Key Assignments:

[F1](PREV)	Display the previous coordinate location of the selected Feature.

[F2](NEXT)	Display the next coordinate location of the selected Feature.

[F4](UTM)	Switch among different coordinate systems for displaying position data.

[F5](QUIT)	Return to the COLLECTED DATA screen.

### 5.4.3  Viewing and Editing Offset Parameters

To view the OFFSET parameters for a Feature record, highlight the desired Feature displayed in the COLLECTED DATA screen, then press [F3](OFS).  A screen similar to one of the following will be displayed if offset parameters were previously saved with the Feature.

15:37	COLLECTED DATA

OFFSET

SLOPE DIST:	20 M

SLOPE:	-15 o

AZIMUTH:	322 o

OFFSET

SLOPE DIST:	20 M

DIRECTION	LEFT

N3D3

SAVE

RESET

HELP[

You may edit the OFFSET parameters displayed or set them all to zero by pressing [F2](RESET).

15:37	COLLECTED DATA

N3D3

SAVE

RESET

HELP[

Function Key Assignments:

[F2](RESET)	Set all OFFSET parameters for the selected Feature to zero.

[F4](SAVE)	Save the changes made in this screen.  Data for the Feature will be re-computed based on the new OFFSET information.

[F5](HELP)	Display a message relevant to this screen.

### 5.4.4  Viewing a Plot of the Collected Data

To see an on-screen plot of all logged locations for the Job file, select “Plot” from the VIEW JOB screen.  The plot screen may look like this:

All of the coordinates for the position data collected in the currently selected Job file are plotted on this screen.  A larger Job file will take longer to plot.  If the Job contains offset Features, MC-GPS will spend some time computing the actual positions before displaying the plot.  You may zoom in or zoom out by using [F1](IN) or [F5](OUT), respectively.  You may also manipulate the plot in other ways by pressing these keys:

NORTH

SCALE: 6999 X 9999 M

OUT

IN

N3D3

KEY		ACTION									
[Ù], [Ú],	Move your viewing position up or down.  (The graph moves in the opposite direction.) 
[×], [Ø]		Move your viewing position left or right.  (The graph moves in the opposite direction.)
[A]		Adjust the display to show all graphic data.
[B]		Toggle a circle and a box for the first and last coordinate locations in the Job., 
		respectively.
[G]		Prompt for a Feature to designate as the Feature of interest. 
[H]		Toggle among four kinds of status:

1.  Width X Length	Show the width and length of the range area of the screen.
		The units are dictated by the UNITS field in the DATUM
		setup screen.

2.  Present Coordinates	Show the location highlighted by the cross mark, in degrees
		of Latitude and Longitude to 5 decimal places.

3.  Feature Name	Display the Name of the Feature of interest.

4.  Coordinate Record#	Display the record number of the highlighted
		coordinate  location in the Feature of interest.

[I]		Zoom in to the plot.
[O]	 	Zoom out from the plot.
[X]		Toggle a cross mark to highlight the beginning location of the Feature of interest.
[+]		Move the cross mark to the next coordinate location in the Feature being viewed. 
[–]	 	Move the cross mark to the previous coordinate location in the Feature being viewed.

Press [ESC] to return to the VIEW (JOB) screen.

## 5.5  Logging Base Station Data

MC-GPS allows you to choose if you want to log data as a remote (rover) unit or as a base station.  The data it collects as a base station may be used in the PC-GPS post-processing software to differentially correct Feature data collected within the same time period.

If you need to produce very accurate differentially corrected data, you should first have the precise location of the base station determined by using a land surveying method.  This preliminary step is not necessary for jobs that do not require great accuracy.

To log data as a base station,

- Make sure that “LOG RAW DATA” field in the DATA LOGGING setup screen (Section 3.2) is set to “YES”.

- Make sure that MC-GPS is tracking at least 4 satellites.

- Leave “NEW” in the “Job File” field of the JOB DATA screen and press [F3](BASE).  A “FILE NAME” box containing a default file name will pop up.  The format for the default file name is “MMDDYBnn” which is made up of the UTC date, the letter “B” and a sequence number.  Note that the year is truncated to one digit and the “B” at the 6th character position in the default file name provides a hint that this Job file is a Base Job file.  You may change the file name at this time.  To confirm the displayed file name, press [ENTER].

- Make sure that you will not be logging base data at 12:00 AM Sunday morning, UTC time.  This is the time when the new “GPS Wee starts.  Your field data and base station jobs cannot collect data on both sides of this time.  You can collect data in one base file before 12:00 AM Sunday, then start a new base file after 12:00 AM Sunday.

- On the BASE LOGGING screen described below, select BASE mode or BASEPC mode, then follow the corresponding instructions in Section 5.5.1 or 5.5.2.

The seventh line of the display (SV EPH) shows the PRN numbers of the satellites for which ephemeris data is being collected by MC-GPS.

15:37	BASE LOGGING

MODEBASE    FILE:  02076B01

MEM: 1755 KB  UTC START 13:22

LAT:	44  33  47.4234	N

LON:	123  15  47.4256	W

ALTITUDE:	63.51   FEET   MSL

SV EPH:  02  06  19  21  24  30

N3D3

HELP

START

Field Description:

MODE	Use [Ø] or [×] to select BASE mode if the base station data will be stored into MC-GPS as a base reference file.  Select BASE-PC mode if the base station data is to be saved onto the hard disk of a PC connected to MC-GPS.

File	This is the name assigned to the reference data file collected by MC-GPS acting as a base station.

MEM	This field shows the amount of memory space (in KB) remaining on the MC-GPS RAM disk.

LATITUDE	These fields display the location of the base station in terms of Latitude,

LONGITUDE	Longitude and Altitude (as height above Mean Sea Level).

ALTITUDE	The coordinates shown are the AVERAGE of all uncorrected position data since you began logging in BASE mode.  Averaging of uncorrected positions will typically (after several hours) get your base position accuracy within about 15 meters.

UTC START	This is the UTC time at which the base file was started.  If the TIME MODE in the DATUM setup screen (Section 3.3) is set to “LOCAL”, this field will be titled “LOC START”.

Function Key Assignments:

[F1](START)	Begin logging base data.  The softkey label for [F1] will change to “STOP”.  Press [F1](STOP) to stop logging base data after confirmation.

[F5](HELP)	Display a helpful message about the currently highlighted data field.

### 5.5.1  BASE Mode

In BASE mode, MC-GPS operates as a reference station for another MC-GPS.  The reference file (*.DIF) will be stored on the MC-GPS.  It is important to know that when MC-GPS is in the BASE mode, its 3MB RAM disk will fill up after about 9 hours of continuous data logging.  The 5MB RAM disk will hold about 15 hours of data and the 8 MB RAM disk will hold about 24 hours of data.

Press [F1](START) to start logging base data.  Provided that you have adequate RAM Disk space, you should log sufficient data in the base file to cover the time period of the entire Feature data file to which the differential correction will be applied. When done, press [F1](STOP) to stop the base data logging.

You can transfer files collected in the BASE mode to your PC using the same method as transferring other MC-GPS files (Section 7).

### 5.5.2  BASEPC Mode

In BASEPC mode, MC-GPS operates in a similar way as it does in BASE mode, except that the collected data is routed to a PC hard disk instead of being stored on the MC-GPS RAM disk.

Following is the procedure for logging base data in the BASEPC mode and saving the base data file to a PC running the PC-GPS software:

- Make sure that COM1: of the MC-GPS is connected to the correct communications port of your PC using the ACC-CBL-2 (equivalent to two ACC-ADAPTER5s and one ACC-CBL-1).

- Make sure the communications port of the PC is configured to the same baud as COM1: of your MC-GPS.  If you have transferred files before from the MC-GPS to the PC, most likely these settings will already be correct.  If you need to change them, you may use the CONFIGURE function in the FILE TRANSFER menu of PC-GPS to do this. The MC-GPS will use the settings of the last file that was transferred.

- In PC-GPS, select RECEIVE in the FILE TRANSFER menu.

- On the MC-GPS, press [F1](START) from the BASE LOGGING screen.

- Once data logging begins, the PC will display the size of the file and show it increasing as time goes by.

- When done logging base data, press [F1](STOP).

## 5.6  Viewing a Summary of the Collected Data

There are two ways you can review the information about a Job data file you have collected in remote mode or BASE mode:

- From the MAIN MENU, press [F3](LISTS).  Then select “Jobs” to bring up the  JOBS screen.  Select the appropriate Job file name, then press [F4](SUM) to display a summary about this Job.

- From the MAIN MENU, press [F4](XFER).  On the FILE TRANSFER screen (Section 7), select the appropriate Job file name, then press [F5](SUM).

The JOB SUMMARY and BASE SUMMARY screens are described in Section 4.1.2.

# Section 6 - Navigation

Navigating is the process of following a Route to reach a target.  You have already learned how to create a Route in Section 4.  Additionally, you may use the PC-GPS utility to convert data and coordinates from Job files into Waypoints and Routes.

The navigating function allows you to do the following:

- Select a Route to help direct you to a site or area of interest.

- Calculate distance and direction to any of a series of Waypoints on the Route.

- Display a real-time, scaled scatter plot of current position as well as its location relative to the Route and the destination Waypoint.

## 6.1  Navigating along a Route

The general procedure for Navigation is as follows:

- Select or construct a Route (made up of Waypoints) that leads to your destination.

- Specify the Waypoint number of your Destination Waypoint.

- Move toward your destination, using the displayed information as your guide.

- Press [F4](INFO) to look at the cross-track error or the direct distance between any two Waypoints on the Route.

- Press [F3](PLOT) to view a graph of your current position with respect to the Route.

- If set up to do so, your MC-GPS will warn you when you get close to the destination.

You may access the NAVIGATION screen by one of the following methods:

- Select “Navigate” from the MAIN MENU.

- Press [F4](NAV) from the FEATURE INFO screen or the FEATURES screen (Section 5.2.1) after specifying a Job and selecting a Route.

The NAVIGATION screen will not show up until at least 3 satellites are being tracked.  MC-GPS will display the message “Waiting for initial position fix” while it is trying to achieve this condition.  If a Route has not yet been selected, only the data for COG (Course Over Ground), SPEED and ALT (Altitude) will show up in this screen.

15:37	NAVIGATION

ROUTEPARK

DEST:  CENTRAL PARK	             WPT4

COG:  0o               ALT:  1999 M MSL

SPEED:  4.5 M/S

BRNG: 175o

RNG:  3999 M

N3D3

HELP

INFO

PLOT

CMPASS

ROUTE

Information on this screen is used to guide you along the chosen Route.  If you have set up MC-GPS to warn you when you are in the vicinity of the Destination Waypoint, then the Warning Message you specified for the Destination Waypoint will pop up when you are within the Warning Range specified in the DATA LOGGING setup screen (Section 3.2).  The warning function is active only if you have set WARN MSG to “YES” in the DATA LOGGING setup screen.  If WARN BEEP is set to “YES”, you will also hear a beep when the Warning Message is issued.

Also displayed is a Steering Wheel in the form of a circle and a larger arrow as well as a Steering Cue in the shape of a smaller upward-pointing arrow.  The Steering Cue always points towards the Destination Waypoint.  The larger arrow inside the Steering Wheel represents the direction you are moving relative to the Steering Cue.  If you orient yourself or your vehicle such that this larger arrow aligns with the Steering Cue, you will be moving in the direction toward the Destination Waypoint.  At this time, the COG reading would be very close to that of BRNG (Bearing).  Note, however, the direction indicated by the arrow inside the Steering Wheel is meaningful only when you are moving at a sufficiently high speed, such as while you are in a moving vehicle.

Field Descriptions:

15:37	NAVIGATION

ROUTE  PARK                      WPT  4

DEST:  CENTRAL PARK

COG:  0 o	 ALT:  1999 M MSL

SPEED:  4.5 M/S

BRNG: 275o

RNG:  89 M

N3D3

NEARING CENTRAL PARK

HELP

INFO

PLOT

CMPASS

ROUTE

ROUTE	This field displays the name of the currently selected Route .  If no Route has been selected yet, you may press [Ø] to scroll the available Routes and then select the Route you would like to navigate.  If you would like to enter the Waypoints for a new Route, select “NEW”.  This will pop up a box containing a default name given to the new Route.  You may edit it at this time, if you wish.  Once you have confirmed the name for the new Route by pressing [ENTER], you will be taken to the EDIT WAYPOINT screen to enter information for the Waypoints you would like to add (Section 4.3.2).

DEST	This field displays the name of the Destination Waypoint specified in the WPT field.

WPT	Enter the sequence number of the Destination Waypoint on the selected Route.  The name of the Destination Waypoint will automatically be displayed in the DEST field.

COG	This is your current direction of travel (in degrees of azimuth).  This “Course Over Ground” value is updated dynamically with each new position fix.

ALT	This is the altitude of your current location expressed as Height above Mean Sea Level (MSL) or Height Above Ellipsoid (HAE). This field can display 4 digits, i.e. up to 9999. If that number is exceeded, it will display in the next larger distance unit.  For example, 10,000 ft will display as 1.9 mi.

SPEED	Your current speed (velocity) is updated dynamically.  The unit of speed used is dependent on what you specified for the UNITS field in the DATUM setup screen.  The relationships are as follows:

Units specified in DATUM setup screen		Units for SPEED		
METER       					Meters per Second (M/S)
FEET						Feet per Second (FT/S)
MILE     					Miles per Hour (MPH) 
KM						Kilometers per Hour   (KPH)

This field can display a number up to 99.9.  If this were exceeded, the speed would display in the next larger speed unit.  For example, 100 ft/sec would be displayed as 68.2 MPH.

BRNG	This is the bearing from your current location to the specified Destination Waypoint, in degrees azimuth.

RNG	This is the direct distance from the current location to the Destination Point.  This field can display 4 digits, i.e. up to 9999.  If that number is exceeded, it will display in the next larger distance unit.  For example, 10,000 ft will display as 1.9 mi.

Function Key Assignments:

[F1](CMPASS)	Display an on-screen compass.  Your direction of motion is indicated by the arrow and the direction labels.  Your current speed and course (azimuth) are also displayed.  Because of the presence of Selective Availability, the compass reading is meaningful only if you walk at a very fast pace or are riding a moving vehicle.  Once the restrictions imposed by Selective Availability are removed, you should be able to get a good reading even when walking at a normal pace.

[F2](ROUTE)	Access the Routes database to add or edit Routes and/or Waypoints (Section 4.3).

15:37    COMPASS

COG: 359 o

SPEED: 4.9 M/S

N3D3

N

W

S

E

HELP

[F3](PLOT)	Display a plot of the Route and the current location.

[F4](INFO)	Display information that helps guide you from one Waypoint to another.

[F5](HELP)	Display a helpful message about the navigation functions.

## 6.2  Moving from one Waypoint to another

To obtain additional information relating to your current navigation task, press [F4](INFO) from the NAVIGATION screen.  A screen similar to the following will be displayed.

This screen looks similar to the NAVIGATION screen except that it lets you specify two Waypoints to find out more about traveling from one to the other.  Any of the Waypoints along the Route may be entered as the ORIGIN Waypoint or the DEST Waypoint.  When the Origin and Destination Waypoints are changed, MC-GPS automatically re-calculates the distance and bearing information.

15:37	NAV INFO

ORIGIN:  MADISON ST	   WPT  1

DEST:  CENTRAL PARK	WPT  4

BEARING:  242o    DIST:      899 M

COG:        275o    SPEED:   4.5 M/S

ALT:   8999  M  MSL       XTE:  99  M  L

N3D3

NEARING CENTRAL PARK

HELP

NAV

PLOT

ROUTE

In addition, this screen displays a cross-tracking error in the XTE field.  The cross-tracking error is displayed in the distance unit specified in the DATUM setup screen.  It is an indication of how far off you are from the course of the Route.  If you see an “L” displayed by the cross-track error, it means you should turn left to get closer to the Route.  Similarly, “R” means you should turn right to get closer to the Route.

The Function key assignments are the same as for the NAVIGATION screen except that [F4](NAV) takes you back to the NAVIGATION screen.

## 6.3  Navigation Plot

From the NAVIGATION screen, you may press [F3](PLOT) to display a plot of your Route and current position as a visual aid to your navigation task.  The plot will be similar to the following:

The solid line in the graph is the actual Route you should be following whereas the dots indicate the real-time position trends of MC-GPS.  With each new position you occupy, a new dot is added to the screen.  The beginning point of the navigation is noted by a small open circle.  The location of the currently selected Destination Waypoint is indicated by a small square.  A tick mark indicates your current position.

NORTH

SCALE: 6999 X 9999 M

N3D3

OUT

IN

You may press [F1](IN) or [F2](OUT) to zoom in on (enlarge) or zoom out of (reduce) the plot.  You may also control the plot by using the following keys:

KEY		FUNCTION								
[Ù], [Ú]	Move your viewing position up or down.  (The graph moves in the opposite direction.) 
[×], [Ø]		Move your viewing position left or right.  (The graph moves in the opposite direction.)
[A]		Fit the entire Route onto the screen. 
[B]		Toggle a circle and a box mark as an indicator for the beginning and target Waypoint,
		respectively.
[G]		Prompt for a Waypoint to designate as the current Waypoint record.
[H]		Toggle 4 different modes of status :

1.  Width X Length	Show the width and length of the range area of the screen.
		The units are dictated by the UNITS field in the SETUP
		DATUM setup screen.

2.  Latitude, Longitude	Display the position of the current Waypoint record.

3.  Waypoint Name	Display the name of the current Waypoint.

4.  Waypoint Record#	Display the record number of the current Waypoint.

[I]		Zoom in to the plot.
[O]		Zoom out from the plot.
[X]		Toggle a cross sign as a visual indicator to highlight the current Waypoint.
[+]		Flash the Waypoint indicator at the next Waypoint in the Route.
[–]		Flash the Waypoint indicator at the previous Waypoint in the Route.

You may exit the plot screen by pressing [ESC].

# Section 7 - Data File Transfer

There are three basic file types that MC-GPS stores: Routes (*.PAT), Feature Lists (*.FBR), and Jobs (*.DEF, *.RAW, *.FTR).  In addition, when used to log data as a base station, it will also contain Base reference files (*.DIF).  You may transfer these files between your MC-GPS and a PC or another MC-GPS.  Following are some reasons for doing data file transfers:

- You would normally transfer Job files to PC-GPS to differentially correct positions and to edit the Map data before exporting your data to a GIS.

- When MC-GPS transfers a Job file to the PC, it also sends an almanac file (ALM.MVX) from the GPS receiver to the PC and saves a copy into the MC-GPS RAM Disk.

- It is very important that you save a backup copy of the data files by transferring them to another mass storage device such as the hard disk of your desk-top PC.

- If you used MC-GPS to log data as a base station, you would transfer the base files to PC-GPS and use them to differentially correct the position data collected during the same time period by another MC-GPS.

- It is more efficient to set up Job information, Feature List and Route databases on PC-GPS and then transfer them to MC-GPS.

- While out in the field, you can quickly set up Job information, Feature List and Route databases on a MC-GPS by transferring the data files from another MC-GPS.

Transferring data files between MC-GPS and the PC or another MC-GPS involves making a hardware connection between the two computers and executing certain commands on each machine.  On MC-GPS, file transfer is always performed via its COM1 port.

To get ready for data transfer to a PC, connect the COM1 port of MC-GPS to the COM1 (or in certain cases COM2) port of the PC.  You will need an ACC-ADAPTER5 (9 pin) on the MC-GPS end of the cable (ACC-CBL-1), and either an ACC-ADAPTER5 (9-pin), or an ACC-ADAPTER3 (25 pin) on the PC end of the cable, depending on the type of serial interface on the PC.  If your PC has a 9-pin serial interface, use the ACC-CBL-2 that comes with your MC-GPS Data Recorder to make the connection.

To access the file transfer functions, press [F4](XFER) from the MAIN MENU.  A screen similar to the following will be displayed:

5:37	FILE TRANSFER

FILE TYPE   	JOB

BAUD RATE   	19200

MARSH3

G030714A	G030714B

G030914A	G031210A

G043011A

F3D3

A030612A

SEND

TYPE

BAUD

RECV

SUM

Field Descriptions:

FILE TYPE	This field shows the type of files selected for sending out of the MC-GPS.  It does not affect what type of files are to be received by MC-GPS.  Press [F1](TYPE) to scroll through the available options: Job, Route, Feature List, Base, and Almanac.  You will notice that the file listing changes to display all the files that correspond to the selected file type.

BAUD RATE	Press [F2](BAUD) to scroll through the available options: 300, 600, 1200, 2400, 4800, 9600, 19200 or 38400.  The default is 19200 baud.

Note:	The other communications parameters are set to the following fixed values:

Parity:  None
Data bits:  8
Stop bits:  1
Handshake:  DTR/CTS

Function Key Assignments:

[F1](TYPE)	Scroll through the available options of types of files to send out of MC-GPS: Job, Route, Feature List, Base, and Almanac.

[F2](BAUD)	Scroll through the available options: 300, 600, 1200, 2400, 4800, 9600, 19200 or 38400.

[F3](SEND)	Initiate the transfer of files out of MC-GPS.

[F4](RECV)	Initiate the transfer of files into MC-GPS.  The machine sending the files to your MC-GPS controls which files to transfer.  The RECV command simply puts MC-GPS in KERMIT server mode.

[F5](SUM)	If the selected file is a Job file or a Base file, pressing [F5] will display a summary screen (Section 4.1.2) for you to review.

[F5](EDIT)	If the selected file is a Feature List or a Route file, pressing [F5] will give you an opportunity to edit the information in that file.

With a valid file selected, pressing [F3](SEND) or [F4](RECV) will display the following screen:

The status bars showing the progress of file transfer will be displayed only for sending files out of MC-GPS.  The “SENDING” or “RECEIVING” field shows the name of the file being transferred.  If you decide not to transfer certain files out of the set of marked files, you may press [ESC] to halt the process.

15:37	TRANSFERRING

SENDING :

CURRENT FILE (KB):   660

0	25	50	75	100

TOTAL TRANSFER (KB):   1345

0	25	50	75	100

N3D3

MARSH3.RAW

If you pressed [ESC] while sending files out of MC-GPS, you will be prompted with: “Do you wish to continue file transfer?” and the following Function keys will be displayed:

[F1](CONT)	Continue to send this file.

[F3](SKIP)	Skip this file but send the remaining marked files.

[F5](QUIT)	Return to the previous screen.  If you have just sent files to the PC, MC-GPS will display: “Delete transferred files from MC-GPS?”  If you wish to have the transferred files deleted from the MC-GPS file system, press [F5](YES).  If not, press [F1](NO).

If you pressed [ESC] while receiving files into MC-GPS, you will be prompted with: “Do you wish to quit file transfer?”, and the following Function keys will be displayed:

[F1](NO)	Continue to receive the next file.

[F5](YES)	Return to the previous screen.

## 7.1  Sending Data from MC-GPS to PC-GPS

Following is the procedure for sending MC-GPS data files to a PC that is running the PC-GPS utility programs:

- Make the proper cable connections according to the instructions provided at the beginning of Section 7.

- In the PC-GPS software utility, select the FILE TRANSFER menu and the CONFIGURE function to verify the PC communication settings.  The default communications port is COM1, and the default baud rate is 19200 baud.

- From the PC-GPS FILE TRANSFER menu, select the RECEIVE function.  This will place the PC into KERMIT server mode, ready to receive files from MC-GPS.

- On MC-GPS, press [F4](XFER) from the MAIN MENU.  Then specify the file type and transfer baud rate.

- In the MC-GPS FILE TRANSFER screen, highlight each of the files that you want to transfer and press [ENTER] to mark it with a chevron mark (»).  If you mark a file by accident, you may highlight it and press [ENTER] again to un-mark it.

- When you have selected all of the files that you wish to transfer, press [F3](SEND) to start the transfer process.

- When it is finished transferring files, MC-GPS will break the PC out of server mode.  It will then display: “Delete transferred files from MC-GPS?”  If you wish to have the transferred files deleted from the MC-GPS file system, press [F5](YES).  If not, press [F1](NO).

- On MC-GPS, press [ESC] to return to the MAIN MENU.

## 7.2  Receiving Files from the PC

Following is the procedure for receiving data files into MC-GPS from a PC that is running the PC-GPS utility programs:

- Make the proper cable connections according to the instructions provided at the beginning of Section 7.

- On MC-GPS, press [F4](RECV) from the FILE TRANSFER screen to put it into KERMIT server mode, ready to receive files from the PC-GPS utility.

- In the PC-GPS software utility, select the FILE TRANSFER menu and the CONFIGURE function to verify the PC communication settings.  The default communications port is COM1, and the default baud rate is 19200 baud.

- In the PC-GPS FILE TRANSFER menu, select the SEND function.  Click on the file you would like to send and then click on “OK”.

- When it is finished transferring files, the PC-GPS program will automatically break MC-GPS out of server mode.  On MC-GPS, press [ESC] to return to the MAIN MENU.

## 7.3  Sending Data from MC-GPS to another MC-GPS

Following is the procedure for sending MC-GPS data files to another MC-GPS:

- Connect the two units via their COM1 ports by using the ACC-CBL-2 that comes with each unit.

- On both MC-GPS units, press [F4](XFER) from the MAIN MENU.

- On the MC-GPS acting as the receiver, specify the desired transfer baud rate on the FILE TRANSFER screen, then press [F4](RECV) to go into KERMIT server mode.

- On the MC-GPS acting as the sender, specify the file type and the same transfer baud rate on the FILE TRANSFER screen.  Highlight each of the files that you want to transfer and press [ENTER] to mark it with a chevron mark (»).  If you mark a file by mistake, you may highlight it and press [ENTER] again to un-mark it.

- On the MC-GPS acting as the sender, press [F3](SEND) to start the file transfer process.

When done transferring files, the MC-GPS acting the receiver will automatically break out of server mode.  On both units, press [ESC] to return to the MAIN MENU.

# Appendix A - User License Agreement

This is a legal agreement between you, the End User, and Corvallis Microtechnology, Inc. (Company).  By opening the sealed package containing the MC-GPS and turning the machine on, you are agreeing to be bound by the terms of this Agreement.

1.  The MC-GPS contains intellectual property, i.e., software programs, that is licensed for the end user customer's use (End User).  The licensed software programs are protected by United States copyright laws and international treaty provisions.

2.  A sale of the MC-GPS machine is not a sale of the intellectual property that it contains.  The End User is granted the right to use such intellectual property, but not its ownership.

3.  The End User shall not copy, disassemble or reverse compile the software programs.

4.  THE SOFTWARE PROGRAMS ARE PROVIDED TO THE END USER "AS IS" WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.  THE END USER SHALL BEAR THE ENTIRE RISK OF THE QUALITY AND PERFORMANCE OF THE SOFTWARE PROGRAMS.

5.  THE COMPANY AND THE COMPANY'S SUPPLIERS SHALL NOT BE HELD TO ANY LIABILITY FOR ANY DAMAGES SUFFERED OR INCURRED BY THE END USER (INCLUDING, BUT NOT LIMITED TO, GENERAL, SPECIAL, CONSEQUENTIAL OR INCIDENTAL DAMAGES INCLUDING DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF BUSINESS INFORMATION AND THE LIKE), ARISING FROM OR IN CONNECTION WITH THE DELIVERY, USE OR PERFORMANCE OF THE SOFTWARE PROGRAMS.  INFORMATION IN THE MC-GPS DATA RECORDER USER'S MANUAL IS SUBJECT TO CHANGE WITHOUT NOTICE AND DOES NOT REPRESENT A COMMITMENT ON THE PART OF CORVALLIS MICROTECHNOLOGY, INCORPORATED.

This Agreement is governed by the laws of the State of Oregon.

If you have any questions concerning this Agreement, please write to:  Customer Service, Corvallis Microtechnology, Inc., 413 SW Jefferson Avenue, Corvallis, Oregon 97333, U.S.A..

Trademark Acknowledgment

- AT and IBM are registered trademarks and PC/XT is a trademarkâ of the International Business Machines Corporation.

- HP and HP SmartWand are registered trademarks of the Hewlett-Packard Company.

- Intel is a registered trademark of the Intel Corporation.

- KERMIT is a file transfer protocol developed and supported by Columbia University.

- Lotus is a registered trademark of Lotus Development Corporation.

- Microsoft, MS and MS-DOS are registered trademarks of the Microsoft Corporation.

# Appendix B - Specifications and Maintenance

System Processor

80C88 CMOS system

Operating Systems

CMT ROSII

Memory

512 Kbyte RAM

3 Mbyte (MC5GPSDRD3-IC), 5 Mbyte (MC5GPSDRD5-IC) or 8 Mbyte (MC5GPSDRD8-IC) on-board RAM Disk

Two 512Kbyte Flash EEPROMs for Operating System and CMT FIELD application software

Following is a typical memory configuration for the MC-GPS.

Drive A: MEMORY	A minimum of 256K should be formatted to memory (A: and B:) 
Drive B: MEMORY
Drive C: FILES		For other application programs and drivers
Drive D:	Joined with C:
Drive E: FILES		MC-GPS programs
Drive F: ROS (OS)	Operating system
Drive I: RAM disk	3MB, 5MB or 8MB 
Drive J: Application	MC-GPS programs

Clocks and Timers

Real-time clock and calendar for date/time-stamping.

Physical Dimensions

Size:	12.35” x 4.06” x 2.3” with antenna attached

Weight:	Approx. 3 lbs. (48 oz.), including batteries and antenna

Integrated GPS Receiver

6 channels continuous tracking

Default elevation mask angle:   	5 degrees

Default HDOP limit:	 	3

Default VDOP limit:  		6

Default PDOP limit:  		7

Position update rate:  		1 second

Constellation:			Canyon, Blockage or Clear (default is Canyon)

Integrated Antenna

Built-in antenna.

Optional external antenna available.

Display

High-contrast, supertwist Liquid Crystal Display

Back-light standard

Text -  8 lines by 21 characters

Graphics -  64 x 128 pixels

Enhancements -  Inverse video, softkey labels

Keyboard

55-key alphanumeric, multi-function keyboard
	Tactile, raised rubber-dome keys

Communications

Two RS-232 ports:	COM1 and COM2

Built-in KERMIT file transfer protocols:

Baud Rate:  	Configurable from 110 to 38400 for transfer protocol

Handshake:  	XON/XOFF, DSR/DTR, CTS/RTS, Combination
	Data Bits:  	7 or 8
	Stop Bits:  	1 or 2
	Parity:     	Even, odd or none

RS-232 Port Pin-out Diagrams:

Your MC-GPS comes with power supplied to both COM ports (DB-9 male pins).

Pin	Description

1	DCD		Carrier Detect
2	RXD		Receive
3	TXD		Transmit
4	DTR		Data Terminal Ready
5	GND		Ground
6	DSR		Data Set Ready
7	+5 V controlled	(5V +/- 5% 80 mA max current, with backlight OFF
			 5V +/- 5% 40 mA max current, with backlight ON)
8	CTS		Clear To Send
9	VBAT 		(7.5V - 5.0V, 450 mA max current)

WARNING:

Do not connect pin 7 or 9 to pin 5 (GND).  Doing so may damage your MC-GPS unit.

Power Supply

Built-in low-power alarms.

Standard rechargeable NiCad battery pack with DC input jack.  AC Adapter included.

Usage 		Estimated NiCad Battery Life
	Fully running		1 day (with GPS receiver ON)
	Shelf life		1 month (depending upon RAM Disk size)

DC Jack input voltage not to exceed 13V.

Environmental

Waterproof, dust-proof, shock-resistant.

Operating temperature:

Lower limits	-40°C
	Upper limit	 54°C

The MC-GPS and its display will work well on a hot day and under direct sunlight.  CMT does not recommend placing the MC-GPS in an oven or chamber for long-term exposure to temperatures as high as 40°C.  This can cause problems to the internal batteries.

Storage temperature:

-20°C to 40°C

To ensure optimum performance of its internal batteries, the MC-GPS should be stored at temperatures within the -20°C to 40°C range.  Exposure to higher temperatures up to 60°C should be limited to avoid unnecessary harm to the internal batteries.

General Care of the Unit

Clean the unit by gently wiping it with a piece of soft lint-free cloth.

Clean the display cover of the MC-GPS unit only with a soft, lint-free cloth and a non-abrasive cleaning agent.  This will prevent the cover from getting unwanted scratches.  Do not use any solvent such as alcohol.

Should the unit become excessively dirty, you may rinse it with water and soap then use a piece of soft lint-free cloth to wipe it dry.

Do not permit any metal object to create a short circuit between the pins in the COM ports.

Do not open the MC-GPS back panel.

Do not apply a sharp object to the RESET switch or any other part of the rubber-dome keyboard.

DO NOT plug any CMT cable with the RJ-11 connector (similar to the telephone connector in appearance) into a telephone outlet.  This can cause damage to both the MC-GPS and the phone system.

Care of the Battery Pack

MC-GPS uses a 6V 1400 mAH or 1600 mAH NiCad battery pack to maintain its memory contents.  Only use the recharger/adapter marked ACC-RE3B (for 110V wall outlet) or ACC-RE4B (for 220V wall outlet) to recharge it when its power level goes down to about 20% of full capacity.  Recharge 16 hours.  Do not overcharge the batteries by leaving the charger connected for longer than 16 hours.  If overcharged, the batteries may be damaged and their life and performance may be reduced.

As an alternative, you may use the ACC-SMC or ACC-SMC-G2-M “Smart Charger” to recharge your batteries.  The advantages to using one of the Smart Charger options include:  1)  a complete discharge of your system batteries prior to recharge, 2)  charging time of 10 hour or less, including discharge, and 3)  the smart charger will apply a trickle charge to your batteries after charging is complete to maintain the maximum charge.  The ACC-SMC-G2-L is also available to Smart Charge your GPS receiver batteries (GPS-ENG-BAT-G3 only).

To replace the MC-GPS GPS receiver battery pack, turn off the machine, back out the four stainless steel screws that secure the battery pack and lift out the pack.  Note that the screws stay with the battery pack.  Do not apply more force to the screws when they stop turning, as the screws may lose the retaining rings that hold them in place.  Replace the GPS receiver battery with another charged battery pack and secure it.

Once you have replaced your GPS receiver battery pack, you will want to adjust the charge level of the receiver battery that is displayed in the STATUS screen.  To adjust this level, press [SH1] and then [ENTER] to move to the System Status screen and then press [F2](SECND) to move to the Second Battery screen.  In the Second Battery screen, you can use the [+] and [-] keys to adjust the charge level to reflect the current status of the receiver battery.  Press [ESC] twice to return to the CMT-Field software.

After changing the receiver battery, if your MC-GPS fails to begin tracking within a few minutes, use a ball-point pen to gently press the RST button near the communications ports and then turn the unit on again.

CAUTION:

Before replacing the battery pack or sending your MC-GPS Data Recorder to CMT for service, make sure you have saved the data to another mass storage medium.

Warranty Statement

This CMT product is warranted against defects in material and workmanship for a period of 90 days from the date of purchase.  During this period, CMT will, at its option, either repair or replace this product if it proves to be defective.  When sending in an item for warranty repair, please enclose a copy of the invoice.

The buyer shall prepay shipping charges to return the product to CMT for warranty service, repair or replacement.  The buyer shall pay all shipping charges, duties and taxes for products returned to CMT from any country outside the United States.  Products repaired under warranty will be returned via ground delivery service to buyers in the United States and Canada, and via air parcel post to buyers in other countries.  Buyer shall be responsible for any applicable customs duties and taxes.  Buyer shall be responsible for all shipping charges for a delivery service requested by the buyer.

This warranty shall not apply to defects or malfunctions resulting from improper or inadequate maintenance by the user of the MC-GPS Field Data Recorder, failure to observe precautions against electrostatic discharges, unauthorized modification, misuse or operation outside the environmental specifications listed earlier in this Appendix.  In particular, this warranty shall be declared void in case the user disconnects the keyboard, display or I/O port from the main printed circuit board of the MC-GPS or disassembles the MC-GPS in any way.

This warranty shall not apply to the batteries and any damages caused by the batteries.

To register your MC-GPS ownership, please complete the enclosed Warranty Registration card and return it with a copy of the invoice.

Service Information

Should your MC-GPS Field Data Recorder require service, notify the Service Department of CMT.  Do not return any product without first contacting CMT about your problem.

Contact CMT to obtain a Return Material Authorization Number (RMA#) before returning any parts for repair.  Make sure that the parts are properly packaged and the RMA# is clearly marked outside, as well as inside, the package.  When returning a unit or an accessory, please include a letter in the package stating the serial number of your MC-GPS Field Data Recorder, the RMA# you were given, your return address and a description of the problem you are having.

Liability Statement

CMT makes no expressed or implied warranty with regard to MC-GPS Field Data Recorder hardware and software, or the merchantability or fitness of the hardware and software for any particular purpose.  The hardware and software are made available solely on an 'as is' basis and the entire risk as to their quality and performance is with the user.  Should the MC-GPS Field Data Recorder hardware and/or software prove defective, the user shall bear the entire cost of all necessary correction and all incidental or consequential damages in connection with or arising out of the furnishing or use or performance of the MC-GPS Field Data Recorder hardware and/or software.

FCC Statement

Radio/Television Interference Potential

The MC-GPS unit generates and uses radio frequency energy and, if not installed and used properly - that is, in strict accordance with the instructions in this manual - may cause interference with radio and television reception.  It has been tested and found to comply with the limits for a Class A computing device in accordance with the specifications in Subpart J of Part 15 of the FCC rules, which are designed to provide reasonable protection against such interference in a residential installation.  However, there is no guarantee that interference will not occur in a particular installation.  In the unlikely event that the unit does cause interference to radio or television reception (which can be determined by turning the MC-GPS on and off) you are encouraged to try to correct the interference using one or more of the following measures:

Re-orient the receiving antenna.
	Relocate the MC-GPS with respect to the receiver.
	Move the MC-GPS away from the receiver.
	Plug the AC adapter into a different AC outlet so the MC-GPS and the receiver are on
	different branch circuits.

If necessary, contact an experienced radio/television technician for additional suggestions.  You may find the following booklet, prepared by the Federal Communications Commission, helpful:  How to Identify and Resolve Radio - TV Interference Problems.  This booklet is available from the U.S. Government Printing Office, Washington, DC 20402, Stock number 004-000-00345-4.

# Appendix C - Peripherals

Following are 																														some accessories and peripherals that may be used with your MC-GPS Field Data Recorder.

Peripheral Software

-CM	Carrier Phase option for 10-30 cm accuracy

PC-GPS	Post-processing and differential correction software for PC.

Peripheral Hardware

GPS-HARDCASE	Hard carrying case for MC-GPS and accessories

GPS-ENG-BAT-G2	Field replaceable NiCad battery pack for GPS receiver

GPS-ENG-BAT-G3	Field replaceable NiMH battery pack with thumbscrews.  Operates with ACC-SMC-G2-L

GPS-BATPAK-BK-1600	Replacement 1600 mAh NiCad battery pack for the MC-GPS system battery.  Includes back case.

GPS-BATPAK-BK-2100	Replacement 2100 mAh NiCad battery pack for the MC-GPS system battery.  Includes back case.  (This optional battery pack is rechargeable by the ACC-SMC or ACC-SMC-G2-M only.)

ACC-SMC-G2-L	Smart Charger for GPS-ENG-BAT-G3

GPS-CASE-G2	Nylon case for MC-GPS

ACC-SMC-G2-M	Smart charger for MC-GPS system batteries

GPS-XANT-2M	External antenna with 2 meter cable

GPS-XANT-6M	External antenna with 6 meter cable

GPS-XANT-2M PKG	External antenna with rangepole and 2 meter cable

GPS-XANT-6M PKG	External antenna with rangepole and 6 meter cable

ACC-HBCR8300	Bar code wand (contact-type) â

Cables, Connectors, Rechargers

ACC-RE3B	110V adapter/recharger for MC-GPS

ACC-RE4B	220V adapter/recharger for MC-GPS

ACC-RE6	110V adapter/recharger for GPS-ENG-BAT

ACC-RE11	220V adapter/reharger for GPS-ENG-BAT

ACC-RE5	Automobile lighter adapter/recharger for MC-GPS

ACC-RE5-G	Automobile lighter adapter/recharger for GPS-ENG-BAT

ACC-COMKIT	Communications kit with cables, adapters and software for interfacing to PC's, printers, modems and other MC-GPS-Series devices

ACC-CBL-1	2 meter, 6-wire coiled cable with RJ-11 terminations

ACC-CBL-2	2 meter, 6-wire coiled cable with DB-9 terminations

ACC-ADAPTER1	Cable adapter for Hayes compatible modems

ACC-ADAPTER2	Cable adapter for serial printers

ACC-ADAPTER3	25-pin Cable adapter for PC/XT

ACC-ADAPTER5	9-pin cable adapter for PC/AT computer

ACC-ADAPTER-DG	Adapter for interfacing with the Forest Service Data General

# Appendix D - Antennas

MC-GPS comes with an integrated antenna for receiving GPS signals.  MC-GPS can also support an external antenna supplied by CMT.

The GPS-XANT-2M PKG and GPS-XANT-6M PKG external antenna packages include an antenna that attaches to a rangepole.  The 2M or 6M package ID refers to the 2 meter or 6 meter length of cable.  The package also includes a 2.6 meter adjustable rangepole (tapered at the bottom end) and pole adapter.

INSTALLING AN EXTERNAL ANTENNA

When using the external antenna, it is important that you install the cable properly using the strain relief system provided.  If it is not installed properly, the antenna connector will bear the bulk of the strain and may break.

To attach the external antenna cable, please do the following:

- Unscrew the strain relief attachment above the Input/Output ports of your MC-GPS.

- Remove the connector of the integrated antenna from the antenna port.  If the antenna connector is caught in the strain relief attachment, you may insert a small rod (such as a small screw driver) into the smaller hole in the strain relief to push it out.

- Push the external antenna connector straight into the antenna port.  (It may "click".)  This connector is quite delicate and needs to be handled carefully.

- Fit the channel on the inside of the strain relief attachment to the external antenna connector and the antenna cable. 

	CAUTION:	Failure to route the external cable through the channel may place excess stress on the connector 				and result in damage.

- Snap the two tiny pegs on the strain relief attachment into the two tiny holes in your MC-GPS.

- Tighten the thumb screws to firmly connect the strain relief attachment with the MC-GPS unit.

- Snap the connector of the integrated antenna into the larger hole in the strain relief attachment.

# Appendix E - System Operations for Advanced Users

## E.1  Exiting to the MC-GPS Operating System

To exit to the MC-GPS operating system, press [ESC] at the MAIN MENU.  MC-GPS will display a warning message.

Normal operations do not require exiting to the MC-GPS operating system.  You should exit to the MC-GPS operating system only if you wish to do one of the following:

- Run other programs stored in a MC-GPS.

- Issue an MC-GPS operating system command (such as KERMIT).

As a general rule, do not exit to the operating system if you are not familiar with its operations.  If you entered the MC-GPS operating system by mistake, press [ENTER] until you see [F5] (OFF).  Then press [F5](OFF) to turn off the unit.  Now, when you press the [ON] key, the unit will restart the MC-GPS field application programs.

## E.2  Issuing Commands from the Operating System

The MC-GPS operating system offers many commands that are identical or similar to the familiar DOS commands on a personal computer.  These commands are useful to the system administrators and programmers but are not essential for collecting GPS data.

To issue an MC-GPS operating system command, first press [ESC] at the MC-GPS MAIN MENU to exit the application program.  Then enter the command into the command line.  Please refer to the MC5 Operator’s Manual for additional information about the MC5 Operating System.

## E.3  Communicating with a PC

You may use either of the COM ports to transfer data into or out of your MC-GPS Field Data Recorder.  The necessary connecting cables and adapters (ACC-COMKIT) are available from CMT and its dealers.  Within the CMT Field 2.5 and 3.4 software, you will be restricted to using COM1: for your file transfers.

The GPS application programs installed in your MC-GPS Field Data Recorder provide functions that facilitate transfer of relevant files to or from an IBM compatible PC.  These functions are detailed in Section 7 of this manual.

For more information about transferring files between your MC-GPS and your PC, please refer to the MC5 Operator’s Manual provided with your unit.

# Index

—A—

Aborting an operation, 7

Accuracy estimate, 49

ACQ, 8

Almanac, 9, 11, 30, 85

Altitude, 10, 82

Altitude control, 30

Antenna, 94

Antenna Height, 72

APPEND COORDS screen, 73

Area Feature, 1, 55

Attribute, 1, 42, 45, 46, 57, 75

Adding a new one, 45, 69

—B—

Bar code reader, 7, 35, 36, 57

BASE LOGGING screen, 78

BASE mode, 78, 79

Base reference file, 78, 85

Base station, 78

BASE SUMMARY screen, 40, 41

BASEPC mode, 78, 79

Battery pack, 3, 91

Battery power, 9

Baud rate, 86

Bearing, 82

Beep

Error message, 7

Featuring, 32

Navigation, 32

—C—

Carrier phase, 30, 32, 71

COG, 70, 82

Cold start, 9

COLLECTED DATA screen, 74

Combining Job files, 24

Communication parameters, 29

Compass, 6, 83

Constellation, 29

Coordinate positions, 77

Coordinate system, 33, 70, 76

Course, 82

Course over ground, 70

Creating

Feature List, 26, 43

Job, 40, 53

Route, 47

—D—

Data collection, 53

Data collection modes, 32, 53, 55

Data entry, 7

DATA LOGGING setup screen, 31

Data transfer, 85

Database lists, 39

Date, 34

Datum, 33

DATUM setup screen, 33

Declination, 33

Defining

Attribute, 45

Job, 40

Value, 46

Deleting

Attribute, 69

Feature List, 44

Job, 40

Routes, 48

Description of Job, 41

Differential correction, 21, 31, 37, 78, 85

Dilution of Precision (DOP), 30

Direction keys [Ù], [Ú], [×], [Ø], 4, 7

Distance between two Waypoints, 50

DISTANCE screen, 50

Distance units, 33

DOPS, 71

DOPS screen, 11

DYNAMIC mode, 32, 44, 55, 61

—E—

EAST, 10

Easting, 10, 76

EDIT ATTR screen, 45

EDIT FT LIST screen, 43

EDIT ROUTE screen, 48

EDIT VALUE screen, 46

EDIT WAYPOINT screen, 49

Editing

Attribute, 44

Feature List, 26, 43, 86

Job definition, 40

Route, 48, 86

Waypoint, 49

Elevation mask angle, 30

ENTER, 4, 7

Entering data, 7

Ephemeris data, 9, 78

Error checking, 7

ESC, 5

Exiting

MAIN MENU, 95

Submenu, 7

—F—

F2D, 8

F3D, 8

Feature, 1, 42, 55, 74

Adding a new one, 45

Error-checking, 44, 56

ID, 55

Name, 44

Repeating the same, 70

Type, 44

FEATURE INFO screen, 55

Feature List, 1, 40, 42, 54, 85

Creation, 26, 43

Deletion, 44

Managing database, 39

FEATURE LIST screen, 43

FEATURE LOC screen, 75

FEATURES LISTS screeen, 56

FEATURES screen, 56

Featuring, 1, 54

Featuring Beep, 32

FEATURING screen, 69

File transfer, 19, 27, 85

FILE TRANSFER screen, 85

FIX indicator, 9

Function keys [F1] - [F5], 4, 6

—G—

GPS receiver, 9, 89

Channel number, 10

Initialization, 31

Setup, 29

Tracking mode, 11

Turning off and on, 3, 4, 10, 16, 30

GPS receiver ID code, 53

GPS status, 9

GPS Week, 53, 78

Greenwich Standard Time, 34

—H—

HAE, 33

HDOP, 30, 76

Height

Above Mean Sea Level, 70

Above the Ellipsoid, 70

HELP, 6

—I—

IDL, 8

Input/output, 35

INPUT/OUTPUT screen, 35

INS, 4

Inserting

Characters, 7

Interval for DYNAMIC mode, 32

—J—

Job, 85

Creation, 40

Deletion, 40

Description, 41

File name, 40

Managing database, 39

Selection, 53

Summary, 40

JOB DATA screen, 53

JOB INFO screen, 40

JOB SUMMARY screen, 40, 41

JOBS screen, 39

—K—

Keyboard, 3

KINEMATIC method of Carrier Phase data logging, 71

—L—

LAT, 10, 30, 49, 70, 76

Latitude, 10

Line Feature, 1, 18, 55

Listing databases, 39

LLA, 10

LOC time/date, 76

Location data, 75

LOCATION screen (satellites), 11

LON, 10, 30, 49, 70, 76

Longitude, 10

—M—

MAIN MENU, 5

Marking

Job files, 39

Marking files, 6, 20, 87

Memory, 9, 78, 89

Menu, 5

MSL, 10, 30, 33, 49, 70, 76

—N—

N2D, 8

N3D, 8

NAD 27...., 33, 70

NAD 83...., 33, 70

NAV INFO screen, 83

Navigation, 56, 57, 81

Cross tracking error, 83

Plot, 84

Route, 40, 54

Setup, 32

Warning beep, 81

Warning message, 81

Navigation Beep, 32

NAVIGATION screen, 81

Nested Point, 18, 55, 62

NESTED POINT screen, 62

NMEA 0183, 35, 36

NORTH, 10

North reference, 33

Northing, 10, 76

—O—

OFF, 4, 15, 95

OFFSET mode, 58, 64, 76

ON Key, 4, 9

On-screen help, 6

Operating system, 95

—P—

PAUSE (in data logging), 59, 61

PC-GPS, 20

PDOP, 11, 30, 76

Plot

Feature data, 77

Navigation, 84

Position scatter, 70

Route, 51

Point Feature, 1, 54

Position fix, 32

POSITION PLOT screen, 71

POSITION screen, 69, 70

Power meter, 12

PseudoRandom Number, 10

—R—

Range, 82

Raw data, 32, 41, 42, 53, 78

Rebooting the MC-GPS system, 5

RECEIVER screen, 29

Receiving data files, 87

Recharging the battery pack, 3, 91

Remote unit, 78

RESET, 5

Restarting the MC-GPS, 5

Route, 31, 40, 47, 54, 82, 85

Creation, 47

Deletion, 48

Managing database, 39

Plot, 51

ROUTE PLOTscreen, 51

ROUTES screen, 47

Rover unit, 78

RTCM_104, 8, 35

—S—

Satellite

Azimuth, 10

Elevation, 10

Location, 11

Selection, 12

Signal level, 11

Status, 6, 10

SATS, 10

SELECT (database) screen, 39

Selecting an option, 5

Sending out data files, 86, 87

Session length for STATIC mode, 32

Session parameter, 44

Settle time, 32, 58

Setup

Coordinate system, 29

Data logging, 29, 31

Datum, 29

GPS receiver, 29

Input/Output, 29

Navigation, 32

RS-232 Input/Output, 35

Time/Date, 29

SETUP menu screen, 29

SH (SHIFT), 4, 6

Shifted Function keys, 6

Soft RESET, 5

Softkey commands, 4, 6

SPC, 10, 70, 76

Speed, 82

Start-up, 9

Static Line/Area Feature, 55

STATIC method of Carrier Phase data logging, 71

STATIC mode, 32, 44, 54, 60

STATIC screen, 58

STATUS, 4

STATUS screen, 9

Steering cue, 81

Steering wheel, 81

STS, 8

Summary information, 79, 86

SV HEALTH screen, 11

SV STATUS screen, 10

System date, 34

SYSTEM STATUS screen, 12

System time, 34

—T—

Time, 34

Time conversion, 34

Time mode, 33

Time to first fix, 9

TIME/DATE setup screen, 34

Tool Bar, 20

Tracking mode, 11

Tracking Status Indicator, 8, 16, 58

Transferring data files, 85

TRANSFERRING screen, 86

Traverse mode, 65

TRK, 8

Turning ON/OFF

GPS receiver, 10

MC-GPS GPS, 4

—U—

Units, 33

UTC, 34

UTC time offset, 34

UTC time/date, 76

UTM, 10, 70, 76

—V—

Value, 1, 42, 46, 75

Adding a new one, 47

Error-checking, 46, 57

VALUE CHECKING screen, 46

VDOP, 11, 30, 76

Velocity, 70

Version number, 15

VERSION screen, 6

View

Attributes (collected data), 74

Collected data, 74

Dilution of Precision, 11

Features (collected data), 74

Location of Feature, 75

MC-GPS system status, 12

OFFSET parameters, 76

Plot of collected data, 74

Position data, 69

Position plot, 70

Power meter, 12

Satellite health status, 11

Satellite locations, 11

VIEW (JOB) screen, 74

—W—

Warm start, 9

Warning

Beep, 32

Message, 32

Range, 32

Waypoint Message, 49

Waypoint, 31, 47, 48, 49, 82

Adding a new one, 48

Editing, 48

User’s estimate of accuracy, 49

WGS-84, 33

—X—

XFER (Transfer), 85

XTE, 83

—Z—

Zoom in/out, 24, 51, 71, 77, 84